3GPP TSG-RAN WG4 Meeting #113	R4-2419252

# Orlando, US, November 18th – 22nd 2024

Source:	Ericsson

Title:	Discussion on methodology of NR SCM for demodulation

Agenda item:	7.14.2

Document for:	Discussion

# Introduction

In RAN4#112bis meeting, companies agree to focus on SU-MIMO PDSCH test case and CDL approach for comparison. The detailed configurations and some uncleared parameters were discussed. Many candidate options are captured in WF [1] for interested companies to simulate.

In this contribution, we delivered our observations and analysis based on the comparison simulation results and give proposals on furtherly optimization and alignment on CDL extension approaches.

# Discussion

## 2.1	CDL based methodology

In previous meeting, some detailed configurations for comparison simulation were discussed. In 38.901 and 38.827, some different implementations are used for different purposes. For demodulation, more modifications could be applied to generalize model and simplify the configurations.

### 2.1.1	Frequency bands impact

Frequency Bands

Proposals:

- Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range.

Assumptions for CDL based modelling:

Proposals:

- Option 1:

  - For companies submitting 38.901 CDL based results, it is encouraged to base themselves on 38.901 CDL-C (i.e., on denormalized 38.901 table 7.7.1-3) and bring final CDLC table (delays, AoDs, ASDs, etc.) in the next meeting for alignment.

  - It is further encouraged to also consider the 38.827 "randomness reduction framework" on top of 901 modelling.

- Other options not precluded.

All existing demodulation requirements are defined for a certain frequency range not specific band. Of course, some features are only used in a certain deployment which is related to special bands, such as NR-U, NTN, and the corresponding simulations would use a typical center frequency. But there is no explicit restriction for a requirement applied for frequency bands within a frequency range. One of the reasons behind is the receiver algorithm might not be different within a frequency range, so one set of requirements could be enough.

During the link level simulations, frequency band would impact the center frequency, SCS and channel bandwidth which furtherly impact Doppler shift and spectrum. Generally, a typical center frequency would be chosen (i.e., 2GHz with 15kHz SCS and 4GHz with 30kHz SCS for FR1) for demodulation requirement simulations. Furthermore, existing demodulation channel model, such as TDLB100-400, are defined with fixed Doppler shift no matter which center frequency is set in simulations. In that case, the coherent time is fixed, and the performance will be impacted by DM-RS density and SCS rather than center frequency.

The existing TDL models used for demodulation requirement use fixed Doppler shift which leads the no performance difference per center frequency.

As we all known, the CDL models and corresponding TDL models in 38.901 are general models which are feasible for 0.5GHz ~100GHz with maximum 2GHz channel bandwidth. But they have a lot of randomized variables then would be hard for result alignment. Thus, companies agree to derive more specific TDL models based on TDL models in 38.901 for demodulation. TDL models in 38.901 use relative delay and zero Doppler in the profile. For demodulation requirement, an absolutely delay spread and fixed Doppler shift is applied on these TDL models. Same Doppler spectrum (Jakes receiver model) is applied for all taps once center frequency is decided, such as TDLB100-400.

With CDL model approach, different Doppler spectrum would be expected per cluster due to different departure and arrival angles. However, it is also possible to align simulation results if channel model and UE velocity vector definition are aligned. CDL models in 38.827 include typical BS antenna configurations based on frequency, such as <=2.5GHz FR1, >2.5GHz FR1 and FR2, and also fixed delay spread and UE velocity vector (speed and angle directions). The Doppler shift would be variant due to different center frequency. It makes CDL models in 38.827 not general.

CDL models in 38.901 are general but hard for results alignment. CDL models in 38.827 are dependent to center frequency and not general.

If we follow similar approach used in existing TDL model approach, RAN4 could try to derive new general CDL models based on models in 38.901 for demodulation which only fix angle relevant variables and some initial randomness. As for specific delay spread, Doppler and antenna impact, RAN4 could derive specific CDL model on top of new general CDL model according to specific deployment, see figure 2.1-1 and figure 2.1-2. Once the new general models are agreed, it would simplify the specific CDL model derivation and give more flexibility on delay, Doppler and antenna variant. Furthermore, the new general CDL models could be applied for UL either.

- CDL model in 38.827 and expected “Black box” SCM model for demodulation

- Relationship between possible general SCM model and black box SCM model

Figure 2.1-1 General SCM model and specific SCM model

Figure 2.1-2 Derive procedures for new general CDL model and specific CDL model

Following procedures could be considered and furtherly studied:

- Derive scalable deterministic CDL models based on 38.901 model for MIMO demodulation:

  - Reuse relative delay and power per cluster defined in CDL models in 38.901.

  - Set UE is static as 38.901.

  - Assume one isotropic antenna element for both BS and UE side.

  - Set desired angles, desired angel spread, coupling patterns and initial phase of polarization matrix etc., as 38.827 have done.

  - Derive channel model profiles (for example, call them “CDLM-A/B/C”) which can be considered as a common model for a frequency range.

- Derive specific CDL models for demodulation requirements:

  - Set desired delay spread, desired UE velocity vector (Doppler shift and directions) and proper BS and UE antenna configurations according to a typical feature scenario.

  - Derive specific model profiles on top of scalable channel model profiles from step 1 above.

It could be possible to derive a common CDL model with partial fixed variables (angle relevant and initial random parameters) which can be applied for different frequency bands, scenarios and both DL/UL directions.

Proposal 1	RAN4 to discuss if a new scalable deterministic CDL model is necessary from demodulation perspective. Following procedures could be considered.

- Derive scalable CDL models based on 38.901 model and 38.827 method for MIMO simulation:

  - Reuse relative delay and power per cluster defined in CDL models in 38.901.

  - Set UE is static as 38.901.

  - Assume one isotropic antenna element for both BS and UE side.

  - Set desired angles, desired angel spread, coupling patterns and initial phase of polarization matrix etc., as 38.827 have done.

  - Derive channel model profiles (for example, call them “CDLM-A/B/C”) which can be considered as general model for a frequency range.

- Derive specific CDL models for demodulation requirements:

  - Set desired delay spread, desired UE velocity vector (Doppler shift and directions) and proper BS and UE antenna configurations according to typical feature scenario.

  - Derive specific model profiles on top of scalable channel model profiles from step 1 above.

It should be noted that, CDL models in 38.827 have several subsets for Umi, Uma and Indoor, but there is no such kind of differentiation among existing TDL models. For DL side, the UE could move in and out among different deployment scenarios, but the receiver algorithm might be the same. It could be furtherly discussed the necessary of have multiple subsets for CDL models used for demodulation.

Proposal 2	It could be furtherly discussed that the necessary of having different subset of models per deployment (i.e., Uma, Umi, Indoor etc.) for demodulation requirements.

### 2.1.2	Antenna coordination

Antenna Coordinates, Angles, and other parameters for gNB (for UMa-CDL-C)

Proposals:

- Option 1: For the gNB, use the following parameters for alignment with UMa CDL-C model:

  - GCS Location Coordinates for the gNB:

  - Height = 25m (based on UMa);

  - Azimuth = 0; (placement on the x axis);

  - GCS to LCS Conversion angles  (α,β,γ = bearing, downtilt, slant) for the gNB;

  - α,β,γ=(0,0,0);

- Option 2:

  - Frequency Fc = 3.5 GHz, height BS = 25m, and D2D = 100m.

  - BS orientation: (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees). Intention is that BS points towards horizon along x-axis.

- Other options not precluded

Antenna Coordinates, Angles, and other parameters for UE (for UMa-CDL-C)

Proposals:

- Option 1: For the UE, use the following parameters for alignment with UMa CDL-C model:

  - Height = 25m (same as gNB)

  - Azimuth = 0; (placement on the x axis);

  - α,β,γ=(90,0,45);

- Option 2:

  - Frequency Fc = 3.5 GHz, height UE = 1.5m and D2D = 100m.

  - UE orientation: (alpha = -90 degrees, beta = -13.2 degrees, gamma = 45 degrees). Intention is that UE points towards BS and uses "[+ + ... +]" crosspol elements.

- Other options not precluded.

It is important to align the BS and UE antenna coordination during system and link level simulations. In that case, the specific coordination configurations should be clear. Different antenna locations would cause different propagation gain at the receiver side. In WF, there are two options are captured for BS antenna and UE antenna GCS to LCS conversion, see figure 2.1-3 below.

Figure 2.1-3 Antenna coordination between BS and UE

From PDSCH demodulation perspective, two options could achieve same performance if configure it properly. For example, use θ=90o (boresight of antenna element or subarray is pointing along x axis) for option 1 and use θ=103.2o (boresight of antenna element or subarray is pointing to UE antenna) for option 2. In 38.827, the boresight of the strongest beam is pointing to  θ=100o for FR1 Uma deployment and the corresponding channel model profile have included the tilt impact, see figure 2.1-3. In that case, Option 2 sees necessary for simulations when using CDL FR1 Uma model in 38.827.

(a)	original beam direction 		 (b)  beam direction with 13.2o down tilt compensation

Figure 2.1-3 Down tilt impact in CDL-C Uma FR1 in 38.827

From PMI demodulation perspective, different down tilt would impact the PMI index report due to the 2D precoding weight is used. In that case, a proper tilt should be considered when a channel model is derived for PMI test, especially a time variant channel angles are used.

CDL FR1 Uma model profiles in 38.827 include 10o BS down tilt.

Once the boresight of radiation between BS and UE is aligned, it is not critical for PDSCH link level simulation that CDL channel model capture down tilt or not, but down tilt will impact PMI index report.

### 2.1.3	Antenna radiation pattern and array virtualization

Radiation Pattern

Proposals:

- Option 1: For the purposes of CDL alignment, disregard radiation pattern parameters included in TR 38.827 Section 7.2 and consider Omnidirectional antenna pattern

- Option 2: The antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1

TDL and CDL Methodologies

Agreement:

Interested companies to use the following candidate SCM options:

- (Option A) CDL (as defined in R4-2415283) with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- (Option B) CDL (as defined in R4-2415283), i.e., with AAV without subarray (fully connected)

  - [(M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports]

- (Option C, for interested companies) CDL (as defined in R4-2415283) without AAV with the following configuration:

  - [(M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports]

  - [(M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports]

- (Option D, for interested companies) CDL from 38.901 with AAV using subarray

  - (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

  - (M,N,P,Ms,Ns)  = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y)

- (Option E, for interested companies) CDL from 38.901 with AAV without subarray (fully connected)

  - [(M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports]

- (Option F, for interested companies) TDL extensions as defined in R4-2412762)

- (Option G for interested companies) TDL extensions as defined in R4-2415382)

In the context of channel model derivation such as 38.901 or 38.827, antenna element, subarray and array are relevant and different concepts. Generally, they would impact the final beam forming gain distribution in spatial domain. In 38.901 CDL models, there is no clear antenna element radiation assumption and no antenna subarray/array configuration, but 3 types of antenna element radiation patterns are proposed (isotropic, rectangular mask and simplified Gaussian pattern). In 38.827 CDL models, the BS antenna element radiation pattern uses simplified Gaussian model with (HorHPBW, VerHPBW, MaxGain) = (65o, 65o, 8dBi), and the BS antenna parameters are defined in Table 7.2-7 depending on frequency. But there is no UE antenna element radiation pattern is defined in 38.827 due to its purpose is for UE OTA test and UE antenna radiation pattern will be measured at the first stage.

As proposed in previous meeting, “black box” SCM model would be a better choice for demodulation to simplify the test setup which will capture antenna radiation pattern, antenna subarray/array configuration impact into the model profile. But one direct issue comes out that how to do CSI port mapping and beamforming in such SCM model.

In 38.827 3.5G CDL model, there are 128 elements for BS antenna and a “full connection” method is used for beamforming (one CSI-port is connected to all antenna elements). But it is a bit different from typical mid-band BS antenna which uses subarray and array virtualization construction. In that case, companies propose to compare the difference between “full connection” method and subarray virtualization method. Furthermore, companies also propose to compare the performance if only one antenna element per CSI-port is used, see Figure 2.1-4.

As indicated by 38.827 and companies’ proposal, the 60 beam grids with 10o step in vertical plane and 15o step in horizontal plan should be created, and the 4 strongest beam directions for 8 layers transmission are formed:

- 1st strongest beam: AoD = -7.27o, ZoD = 100o.

- 2nd strongest beam: AoD = -21.82o, ZoD = 100o.

- 3rd strongest beam: AoD = -35.98o, ZoD = 100o.

- 4th strongest beam: AoD = -50.97o, ZoD = 100o.

To support 8 layers, the four strongest beams should have enough spatial separation, otherwise the interlayer interference would be high and corresponding performance will be highly degraded. In following parts, we give analysis for CSI port mapping and beamforming impact with 38.827 CDLC Uma FR1 model.

Figure 2.1-4 BS antenna configuration options for 8 layers transmission

- Full connection

In 38.827, it is indicated that one CSI-RS port is connected to all 64 co-polarized antenna elements from 128 total elements. By this method, four beams with better shape could be generated than other options. This method could also be applied for option (b), (c) and (d) in principle, but the formed beam shape would be worse than (a) due to all elements per subarray can only set same weight. But from the product implementation perspective, the full connection would request very high digital BF complexity, so it is not typical for large antenna array.

On the other hand, if we reduce the total number of antenna element, for example use 64 AE (M, N, P) = (8, 4, 2) or even only use 8 AE (M, N, P) = (1, 4, 2), the pointing beam direction will be less precise than 128AE and the beam width get much larger. The corresponding performance will be degraded.

The beam shapes comparison can be found in Figure 2.1-5 and the performance comparison can be found in Figure 2.1-6.

- Full connection with 128AE (M, N, P) = (8, 8, 2) without AAV, HorHPBW =13o

    - Full connection with 64 AE (M, N, P) = (8, 4, 2) without AAV, HorHPBW =28o

    - Full connection with 8 AE (M, N, P) = (1, 4, 2), HorHPBW = 22o, VerHPBW = 65o

Figure 2.1-5	Beam shape comparison for full connection method

Figure 2.1-6 	Performance comparison for full connection method, PDSCH 8 layers

Full connection with 128AE has best performance compared to 64AE and 8AE full connection mostly because the much narrower beam width (HorHPBW = 13o) which could have less interlayer interference, and the beam gain of 128AE is higher. The performance gap between two CW partially caused by the beam pair gain difference between (1st + 2nd vs. 3rd + 4th), especially the 4th beam.

Full connection without subarray virtualization method is not typical for large antenna implementation in real network.

Larger antenna array is helpful to have better beam shape and performance for multiple layer transmission.

- Subarray virtualization

Subarray virtualization is more typical implementation for large antenna to reduce digital domain complexity. All co-polarized elements in same subarray use same excitation weight, and then the BF weights are variant per subarray to form final beams.

If one subarray has multiple rows and columns, it can also steer subarray beam to a certain direction. In that case, it is possible to have 4 beams with different directions at the same time by option (b) (M, N, P, Ms, Ns) = (8, 8, 2, 8, 2) in Figure 2.1-4. But the beam shape is much worse than full connection. Furthermore, if we use option (c) (M, N, P, Ms, Ns) = (8, 4, 2, 8, 1) in Figure 2.1-4, then subarray beam has no horizontal freedom to steer beam direction in horizontal plane and only array can steer final beam to a single direction (i.e., strongest beam direction (AoD = -7.27o, ZoD = 100o) or boresight direction(AoD = 0o, ZoD = 90o)). See Figure 2.1-7 for beam shape comparison.

- 128AE AAV with 4 beam directions and with 1 beam direction (strongest beam direction)

(ii)   64AE AAV with 1 beam direction, strongest beam direction and boresight beam direction

Figure 2.1-7 	Beam shape comparison for subarray virtualization method, one to one mapping between CSI port to RF port.

With CDLC Uma FR1 model, we simulate the performance by subarray virtualization with 128AE and also 64AE configuration, see Figure 2.1-8. You can see the obvious performance degradation compared to full connection method.

- 128AE performance comparison (full connection, AAV beam direction pointing to 1st strongest beam and pointing to boresight direction

- 64AE performance comparison (full connection, AAV beam direction pointing to 1st strongest beam and pointing to boresight direction

Figure 2.1-8 	Performance comparison for subarray virtualization method, PDSCH 8 layers

Similar as full connection method, the large antenna array also has obvious performance gain. With subarray virtualization, 128 AE AAV with 1 direction (strongest beam direction) could achieve quite similar total performance compared with full connection method. It is an interesting result might be caused by the receiver algorithm on how to estimate channel and cancel interference for different layer. It indicates that the CDL model would help to check the receiver performance on angle differentiation. By checking the per CW performance, the performance of CW0 is even better than full connection, but meanwhile the CW1 gets much worse performance. So the performance gap between CWs gets more obvious. The performance of boresight direction is much worse since all layer directions have large offset from strong beam directions in the channel model. But the degradation looks constant compared to strongest direction case.

Note that it is not typical that subarray generate different beam directions like the left figure in (i) of Figure 2.1-8. In 38.827, only horizontal spatial plan is used to support high rank transmission. But in real deployment, 128AE could be split into 32 subarrays to have more spatial freedom to support even higher layer transmission, see figure 2.1-9.

Figure 2.1-9 128AE AAV with 64 RF ports (M, N, P, Ms, Ns) = (8, 8, 2, 2, 1)

All layers are transmitted by strongest beams could have relative better performance than transmitted by different beam directions.

More subarray in a certain plane (horizontal or vertical) would be needed to support more beam direction freedom on this plane.

Subarray virtualization could be feasible for 8 layers transmission with CDLC Uma FR1 if there are enough subarrays to steer the beam.

- One by one CSI port to AE mapping

Different from the full connection by 8AE method, here it just can use same one direction at the same time. The performance would be even worse than full connection method. Changing element radiation pattern to narrower beam width, such as (HorHPBW, VerHPBW, maxGain) = (65o, 12.5o, 17dBi), then the performance would be similar as 64AE with AAV (M, N, P, Ms, Ns) = (8, 4, 2, 8, 1). The Figure 2.1-10 show beam shape of 8AE array pointing to the boresight, and Figure 2.1-11 show corresponding performance.

Figure 2.1-10 Beam pattern for single element per CSI port mapping

Figure 2.1-11 Performance comparison for one by one mapping method, PDSCH 8 layers

The total performance gets better than full connection method. The reason could be the boresight transmission beam HorHPBW is 28o which can cover the strongest beam direction (AoA= -7o), so all layers are transmitted by strongest beam directions. While the full connection method only can let two CSI ports of CW0 are transmitted by the strongest beam, and the layers on CSI ports of CW1 are transmitted by 3rd and 4th beam which is much worse. It seems the propagation condition is more dominant for performance than inter layer interference here.

Similar as subarray virtualization method, all layers transmitted by the strongest beam direction could have better performance. The propagation condition of selected beam is more dominant for performance than inter-layer interference.

One by one CSI port to AE mapping is not feasible for 8 layers performance verification with CDL Uma FR1 model in 38.827.

For comparison with CDL model performance, the performance with TDLC300-100 can be found in Figure 2.1-12. It can be seen that no performance different no matter large antenna array is used or not and the performance difference is very small. If isotropic antenna element is used, no performance gap between two CWs. Comparing to CDL results with 128AE, the performance at 70% NThp is close.

Figure 2.1-12 Performance comparison with TDLC300-100 low

To summarize the simulation results and observations, we can see that the feasibility of using CDL model for checking demodulation performance. From the real deployment perspective, it is true that the propagation condition could be quite different from layers, but the bottom line is that the worse layer can still secure the CW can be correctly decoded at a certain level, otherwise the layers should be reduced. In that case, the performance gap between layers under a channel model should not be too much from demodulation requirement perspective.

In CDLC Uma FR1 model, the propagation condition of 3rd and 4th strongest beam in horizontal plane are quite worse than 1st and 2nd strongest beam. It would require very large antenna array to create good beam gain to compensate the propagation loss on CW1. In that case, it is necessary to consider large antenna array for high layer transmission, especially on the plane with more beams. For example, to transmit 4 beams at the same time along horizontal plane, it would need at least 16AE in horizontal plane to have more freedom.

To support higher layer transmission under CDL model, the large antenna array is necessary to have large spatial freedom.

If set all desired transmission directions along a certain plane (i.e., all strongest beams are along horizontal plane as in CDLC Uma FR1 model), more antenna element is needed (i.e., at least 16 AE for 4 beam directions) to steer the beam to expected direction.

The beamforming seems necessary for higher layer simulation with CDL model. How to capture a proper beamforming weight in the channel model becomes critical. The method of full connection used in 38.827 seems not a typical implementation for BS antenna but could have better performance in most of situations, while subarray virtualization is more typical for implementation but might degrade the performance at a certain level due to fixed excitation weight per subarray. From the demodulation requirement perspective, maybe subarray virtualization method is better since it is close to the real network. The questions then come out that how to define subarray and how to define array of subarray?

- Companies could firstly decide the array of subarray pattern based on the expected beam numbers along 2D direction, such as (Hor, Ver) = (1, 8).

- Then decide typical subarray pattern, such as only one column per subarray.

- As for how many elements in a subarray, companies could discuss based on typical implementation, such as 2 or 3 for 3.5GHz.

- Subarray excitation weight could refer to (7.3-1) in 38.901 and have subarray radiation pattern on top of element radiation pattern.

- The whole beamforming weight could be calculated based on subarray radiation pattern.

Proposal 3	Consider subarray virtualization method for beamforming weight generation in CDL model derivation.

Proposal 4	Choose typical BS antenna configurations and subarray virtualization (1 column per subarray) to have proper beamforming weight in CDL channel model.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The existing TDL models used for demodulation requirement use fixed Doppler shift which leads the no performance difference per center frequency.

Observation 2	CDL models in 38.901 are general but hard for results alignment. CDL models in 38.827 are dependent to center frequency and not general.

Observation 3	It could be possible to derive a common CDL model with partial fixed variables (angle relevant and initial random parameters) which can be applied for different frequency bands, scenarios and both DL/UL directions.

Observation 4	CDL FR1 Uma model profiles in 38.827 include 10o BS down tilt.

Observation 5	Once the boresight of radiation between BS and UE is aligned, it is not critical for PDSCH link level simulation that CDL channel model capture down tilt or not, but down tilt will impact PMI index report.

Observation 6	Full connection without subarray virtualization method is not typical for large antenna implementation in real network.

Observation 7	Larger antenna array is helpful to have better beam shape and performance for multiple layer transmission.

Observation 8	All layers are transmitted by strongest beams could have relative better performance than transmitted by different beam directions.

Observation 9	More subarray in a certain plane (horizontal or vertical) would be needed to support more beam direction freedom on this plane.

Observation 10	Subarray virtualization could be feasible for 8 layers transmission with CDLC Uma FR1 if there are enough subarrays to steer the beam.

Observation 11	Similar as subarray virtualization method, all layers transmitted by the strongest beam direction could have better performance. The propagation condition of selected beam is more dominant for performance than inter-layer interference.

Observation 12	One by one CSI port to AE mapping is not feasible for 8 layers performance verification with CDL Uma FR1 model in 38.827.

Observation 13	To support higher layer transmission under CDL model, the large antenna array is necessary to have large spatial freedom.

Observation 14	If set all desired transmission directions along a certain plane (i.e., all strongest beams are along horizontal plane as in CDLC Uma FR1 model), more antenna element is needed (i.e., at least 16 AE for 4 beam directions) to steer the beam to expected direction.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 to discuss if a new scalable deterministic CDL model is necessary from demodulation perspective. Following procedures could be considered.

1.	Derive scalable CDL models based on 38.901 model and 38.827 method for MIMO simulation:

a.	Reuse relative delay and power per cluster defined in CDL models in 38.901.

b.	Set UE is static as 38.901.

c.	Assume one isotropic antenna element for both BS and UE side.

d.	Set desired angles, desired angel spread, coupling patterns and initial phase of polarization matrix etc., as 38.827 have done.

e.	Derive channel model profiles (for example, call them “CDLM-A/B/C”) which can be considered as general model for a frequency range.

2.	Derive specific CDL models for demodulation requirements:

a.	Set desired delay spread, desired UE velocity vector (Doppler shift and directions) and proper BS and UE antenna configurations according to typical feature scenario.

b.	Derive specific model profiles on top of scalable channel model profiles from step 1 above.

Proposal 2	It could be furtherly discussed that the necessary of having different subset of models per deployment (i.e., Uma, Umi, Indoor etc.) for demodulation requirements.

Proposal 3	Consider subarray virtualization method for beamforming weight generation in CDL model derivation.

Proposal 4	Choose typical BS antenna configurations and subarray virtualization (1 column per subarray) to have proper beamforming weight in CDL channel model.

# References

[1] 	R4-2416591, Way Forward for [112bis][318s] FS_NR_demod_SCM, Nokia

[2]	R4-2419253, Simulation results for SCM for demodulation, Ericsson