3GPP TSG-RAN WG4 Meeting #113	R4-2419256

# Orlando, US, November 18th – 22th 2024

Source:	Ericsson

Title:	Simulation results for Rel-18 NR FR2-NTN PUCCH format 3 and 4

Agenda item:	5.21.4.1

Document for:	Information

# Introduction

In RAN4#112 meeting, NR FR2-NTN demodulation requirement values are agreed to be introduced into specification, but there are TBD for PUCCH part. In this contribution, we delivered new simulation results for format 3 and 4.

# Simulations

## FR2-NTN PUCCH format 3

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| The number of UCI information bits | 16 |
| First symbol | 0 |

Table 2-1: Required SNR for PUCCH format 3 with 120kHz SCS in FR2-NTN

| Number of TX | Number of demodulation | Cyclic Prefix | Propagation conditions and | Additional DM-RS | Channel Bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| antennas | branches |  | correlation matrix (Annex D) | configuration | 50 MHz |  |
|  |  |  |  |  | Ideal | impairment |
|  | 1 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | 0.1 | 2.8 |
|  |  |  |  | Additional DM-RS | -0.1 | 2.6 |
| 1 | 2 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | -4.3 | -1.6 |
|  |  |  |  | Additional DM-RS | -5.1 | -2.4 |

## FR2-NTN PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping starting PRB | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Table 2-2: Required SNR for PUCCH format 4 with 120 kHz SCS in FR2-NTN

| Number of TX | Number of demodulation | Cyclic Prefix | Propagation conditions and | Additional DM-RS | Channel Bandwidth / SNR (dB) |  |
| --- | --- | --- | --- | --- | --- | --- |
| antennas | branches |  | correlation matrix (Annex D) | configuration | 50 MHz |  |
|  |  |  |  |  | Ideal | impairment |
|  | 1 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | 1.9 | 4.6 |
|  |  |  |  | Additional DM-RS | 1.9 | 4.6 |
| 1 | 2 | Normal | NTN-TDLC5-1200 Low | No additional DM-RS | -2.7 | 0.0 |
|  |  |  |  | Additional DM-RS | -3.3 | -0.6 |

# Conclusions

New simulation results for NR FR2-NTN PUCCH format 3 and 4 are delivered. The corresponding SNR values changes.

Original values

New Values impact

# References

[1]