3GPP TSG-RAN WG4 Meeting #113	R4-2419261

# Orlando, US, 18th – 22nd November, 2024

Source:	ZTE Corporation

Title:	Discussion on the core maintenance of SSB-less SCell operation of Network energy saving for NR

Agenda Item:	5.27.1

Document for:	Approval

- Introduction

In RAN4#112 meeting, the core part maintenance and perf part maintenance were discussed. Around SSB-less operation, some progress on core part were achieved in [1]. They are summarized as below:

Issue 1-1-1: Power difference conditions Agreement: - For EPRE difference within X dB, keep the existing requirement. - X equals to or larger than 12 dB - For EPRE difference beyond X dB and no larger than Y dB, add one more P-TRS occurrence. - Further discuss whether the upper bound of Y is needed. If needed, Y is larger than 25dB. - From RAN4 perspective, AGC compensation for carrier frequency difference can be optionally implemented by UE. Remove “[after pre-compensation]” from the spec, i.e., not specify “AGC compensation” for carrier frequency difference in the spec. - Further discuss the X and Y values in this meeting. Try to conclude in the 2nd round on Thursday afternoon. Additional Agreement: X = 12; Y = [30] Issue 1-1-2: Multiple SSB-less SCells activation Agreement: - For TRS-based multiple SSB-less SCell activation, for each to-be-activated SCell, single SCell activation requirements apply respectively. - For A-TRS-based and mixed scenario (TRS-based and A-TRS based) for multiple SSB-less SCell activation, for each to-be-activated SCell, single SCell activation requirements apply respectively. Issue 1-1-3: Relation to R15 intra-band SSB-less Agreement: - No requirements when the SSB-less SCell is configured with QCL source to both intra-band contiguous and inter-band Cells. And no spec impacts expected.

In this document, we provide our views on the remaining issues around SSB-less SCell operation.

- Discussion

Feasibility conditions

In previous meeting, the side condition of power difference has been warmly discussed with the following agreements achieved:

Agreement: - For EPRE difference within X dB, keep the existing requirement. - X equals to or larger than 12 dB - For EPRE difference beyond X dB and no larger than Y dB, add one more P-TRS occurrence. - Further discuss whether the upper bound of Y is needed. If needed, Y is larger than 25dB. - From RAN4 perspective, AGC compensation for carrier frequency difference can be optionally implemented by UE. Remove “[after pre-compensation]” from the spec, i.e., not specify “AGC compensation” for carrier frequency difference in the spec. - Further discuss the X and Y values in this meeting. Try to conclude in the 2nd round on Thursday afternoon. Additional Agreement: X = 12; Y = [30]

Originally the pre-compensation is used by UE to overcome the predicted power difference between SSB-less SCell and reference cell since of the BW difference and carrier frequency difference, unfortunately some UE vendors raised concern on the UE implementation of pre-compensation. So the solution falls back to two set of requirements defined. Follow this logic, one threshold to distinguish the application of two requirements is needed, i.e. the threshold of X. When the EPRE difference is within X dB, the requirement of two P-TRS/A-TRS samples is assumed. Otherwise, the requirement of three P-TRS samples is assumed. For the former case, one TRS sample is used to perform AGC and the other TRS sample is used to perform fine time tracking. For the latter case, two TRS samples are used to perform AGC and the other TRS sample is used to perform fine time tracking. But for the latter case, even two TRS samples are kept to perform AGC, some company still shows concern on the risk if much larger EPRE difference is possible. To prevent such extreme case of much larger EPRE difference, the threshold of Y was introduced. The threshold Y implies the UE is still capable of performing AGC within 2 TRS samples under a EPRE difference as much as Y dB. Regarding the value of Y, we prefer the value not smaller than 30 dBm.

Proposal 1: Specify the threshold of Y as the value not smaller than 30dBm.

It should be noted that for A-TRS case, due to the special A-TRS bursts framework, so only two samples of measurement is allowed. So only one set of requirement is defined for the A-TRS case, which accommodates 2 samples to perform both AGC and fine time tracking.

Proposal 2: A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y.

Neighbor cells on carrier of SSB-less SCell

For the SSB-less SCell, since no SSB transmission, so how to specify the neighbour cell as intra-frequency measurement or inter-frequency measurement, which should be discussed. In legacy RRM measurement, intra-frequency measurement is defined as below:

9.2 NR intra-frequency measurements 9.2.1 Introduction A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell indicated for measurement and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same.

Since the absence of SSB of the SSB-less SCell, we can not directly reuse this principle to decide intra-f or inter-f measurement of a neighbour cell. Regarding how to understand the “centre frequency of the SSB of the serving cell” in legacy, some company proposed that it implies the SSB configured in MO and declared as ServingCellMo in ServingCellConfig, instead of the absoluteFrequencySSB defined through ServingCellConfigCommon -> DownlinkConfigCommon ->FrequencyInfoDL -> absoluteFrequencySSB. We believe this understanding makes sense, since the configuration of ServingCellConfigCommon -> DownlinkConfigCommon ->FrequencyInfoDL -> absoluteFrequencySSB is only applicable to PSCell/SCell, not any IE in ServingCellConfigCommonSIB/ServingCellConfigCommon/ServingCellConfigCommonSIB is used to configure the frequency of PCell since actually UE determines the SSB frequency of PCell by blind search of sync raster.

Based on the clarification of “centre frequency of the SSB of the serving cell”, i.e. it is the frequency configured in MO instead of in ServingCellConfigCommonSIB/ServingCellConfigCommon/ServingCellConfigCommonSIB, it is possible due to the demand of neighbour cell measurement, still even for the SSB-less SCell, a MO with ssbFrequency configuration is allowed. But if the SMTC is configured in this MO for the sake of neighbour cell measurement, this MO should not be declared as ServingCellMo in ServingCellConfig. For the MO declared as ServingCellMo of the SSB-less SCell, no SMTC configured in this MO, but if there is ServingCellMo configured for this MO, we can not see any restriction, so it is up to NW decision.

Proposal 3: To meet the definition of SSB-less SCell, if SMTC is configured in the MO of SSB-less SCell, this MO should not be declared as ServingCellMo in ServingCellConfig. Then the SSB of this MO is only transmitted from the neighbour cell.

- For the case of no SSB measurement demand at any neighbour cell of the same frequency, the MO of the SSB-less SCell can be declared as ServingCellMo in ServingCellConfig.

Follow the above analysis, actually to guarantee UE and NW have consistent understanding in term of which SCell is the SSB-less SCell, if there is neighbour cell SSB measurement demand at the same frequency, the relevant MO should not be declared as ServingCellMo. Consequently, the decision of determine the neighbour cell measurement as intra-f or inter-f is clear, i.e. it is always defined as inter-f measurement as no ServingCellMo in the same frequency.

Proposal 4: Define the neighbour cell measurement as inter-frequency measurement since no ServingCellMo configured at the SSB-less SCell frequency.

Another point, whether the neighbour cell measurement is performed with gap or without gap, to be simplify, follow the legacy rule specified Clause 9.3.1[2] for inter-frequency.

Proposal 5: Whether the neighbour cell measurement is performed with gap or without gap, follow the legacy rule specified Clause 9.3.1 for inter-frequency.

Intra-band non-contiguous CA

In last meeting, this issue was discussed. In legacy R16/17, the SSB-less SCell activation operation is allowed in FR1 intra-band contiguous CA given the side considtions met. And in this R18 WID, the SSB-less SCell activation operation is further extended to be applicable in FR1 inter-band CA. So it seems that the scenario of intra-band non-contiguous CA is vacent. The following options were proposed by the proponent of this scenario in last meeting:

- Option 6: - Specify only assuming separate chains and reuse largely the requirements specified for inter-band cases; - Specify UE optional capability signalling for intra-band NCCA SSB-less SCell operations in a similar way as for inter-band cases; - UE with single chain implementation does not indicate support for intra-band NCCA SSB-less SCell operations and does not need to meet the requirements. - Option 7: - Specify two sets of requirements respectively assuming single and separate chains, where one of the sets reuse largely the requirements specified for inter-band cases; - Specify UE optional capability signalling for two different implementations separately; - Different UE implementations are required to meet different sets of requirements correspondingly.

Companies were diverse on the assumption of RF chain, whether single or separate chains is assumed. To move forward, we prefer to specify two sets of requirements respectively assuming single and separate chains, where the requirements of inter-band case are largely reused for the separate chains assumption, and the requriements of intra-band contiguous case are largely reused for the single chain assumtpion. Accordingly, two optional UE capabilities refers to the two cases. Once UE supports any UE capability, then UE should meet the relevant activation delay requirements. If neither of them supported by the UE, then UE does not support the SSB-less SCell activation for intra-band non-contiguous CA. We believe this is the compromised solution considering different UE implementation architecture.

Proposal 6: To move forward, we prefer to specify two sets of requirements respectively assuming single and separate chains, where the requirements of inter-band case are largely reused for the separate chains assumption, and the requirements of intra-band contiguous case are largely reused for the single chain assumption. Two optional UE capabilities refers to the two cases. If neither of them supported by the UE, then UE does not support the SSB-less SCell activation for intra-band non-contiguous CA.

- Conclusion

In this contribution, we have the following proposals for the Network power saving:

Proposal 1: Specify the threshold of Y as the value not smaller than 30dBm.

Proposal 2: A-TRS based requirement is based on two TRS samples, not impacted by the threshold of X, Y.

Proposal 3: To meet the definition of SSB-less SCell, if SMTC is configured in the MO of SSB-less SCell, this MO should not be declared as ServingCellMo in ServingCellConfig. Then the SSB of this MO is only transmitted from the neighbour cell.

- For the case of no SSB measurement demand at any neighbour cell of the same frequency, the MO of the SSB-less SCell can be declared as ServingCellMo in ServingCellConfig.

Proposal 4: Define the neighbour cell measurement as inter-frequency measurement since no ServingCellMo configured at the SSB-less SCell frequency.

Proposal 5: Whether the neighbour cell measurement is performed with gap or without gap, follow the legacy rule specified Clause 9.3.1 for inter-frequency.

Proposal 6: To move forward, we prefer to specify two sets of requirements respectively assuming single and separate chains, where the requirements of inter-band case are largely reused for the separate chains assumption, and the requirements of intra-band contiguous case are largely reused for the single chain assumption. Two optional UE capabilities refers to the two cases. If neither of them supported by the UE, then UE does not support the SSB-less SCell activation for intra-band non-contiguous CA.

- Reference

- R4-2414102, “WF on RRM requirements for NR network energy saving”, Huawei

- TS 38.133 v18.7.0