3GPP TSG RAN WG4 Meeting #113	R4-2419293

Orlando, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	5.8.3

Source: 	Ericsson

Title:	Discussion on performance part for NR in less than 5 MHz bandwidth

Document for:	Discussion

# Introduction

In this contribution, we provide values for the BFD and RLM test cases for NR in less than 5 MHz bandwidth.

# Discussion

## RLM test cases

The principle for setting the SNR levels can be followed from the legacy. Same principles are discussed in R4-092110. We reproduce the discussion below. For out-of-sync and in-sync tests, the signal level is changed at different time instants as per the required target SNR as shown in Fig. 1 and Fig. 2.

Figure 1: SNR variation for OOS testing

Figure 2: SNR variation for IS testing

In legacy the following methodology is proposed for deriving the test SNR levels and we suggest to use the same method for less than 5 MHz test cases.

SNR2 = Qout + margin1 dB

SNR3 = Qout – margin1 dB

SNR4 = Qin – margin2  dB

SNR5 = Qin + margin2  dB

SNR1 = SNR5.

Qout and Qin correspond to the average of SNR points from simulation results of different companies for out-of-sync and in-sync PDCCH formats respectively.

In the above,

for AWGN, margin1 = 2 dB and margin2 = 2 dB and

for TDL-C 100 Hz channel model, we suggest margin1 = 3 dB and margin2 = 2 dB.

- For RLM and BFD test cases, for TDL-C 100 Hz channel model, RAN4 to agree margin1 = 3 dB and margin2 = 2 dB

RAN4 agreed to introduce 3 test cases for RLM and 1 test case for BFD. In the subsequent section we discuss the SNR levels for the agreed test cases.

### Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS for 15 PRB Coreset

Simulation Assumption:

PDCCH BW: 15 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 9 PRBs are punctured and non-inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -7,23 |  | 2,86715 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -4,36285 |  |
| Adjusted values |  |  | -9.23 | -1.23 | 8 |

Based on the simulation results shown above, the gap between OOS and IS thresholds are not 8 dB, hence we suggest adjusting to the above levels shown in the table.

From the above table after adjusting OOS and IS levels, we get Qout is: -9; Qin is -1 and we consider margin1 = 3 dB and margin2 = 2 dB for TDL-C 100 Hz.

SNR levels

SNR2 = Qout + margin1 dB= -6dB

SNR3 = Qout – margin1 dB = -12 dB

SNR4= Qin – margin2 dB =-3 dB

SNR1=SNR5= Qin + margin2 dB =1 dB

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SNR on RLM RS | Config 1 | dB | 1 | -6 | -12 |

- SNR levels for Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS for 15 PRB Coreset are given by following table

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SNR on RLM RS | Config 1 | dB | 1 | -6 | -12 |

### Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB and -based RLM RS for 12 PRB coreset

Simulation Assumption:

PDCCH BW: 12 PRBs, Number of CORESET symbols: 2; Carrier freq.: 900 MHz; CORESET bandwidth 12 PRBs and non-punctured and inter-leaved. Antenna config: 2*2, low.

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -6.94 |  | 10.04 |
| IS | 4 | TDL-C 300ns 100 Hz |  | 3.1 |  |

From the above table we get OOS and IS threshold as Qout is: -7; Qin is 3 and we consider margin1 = 3 dB and margin2 = 2 dB for TDL-C 100 Hz.

SNR levels

SNR2 = Qout + margin1 dB= -4

SNR3 = Qout – margin1 dB= -10

SNR4 = Qin – margin2  dB= 1

SNR1=SNR5 = Qin + margin2  dB = 5

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM RS | Config 1 | dB | 5 | -4 | -10 | 1 | 5 |

- SNR levels for Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS for 12 PRB Coreset are given by following table

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM RS | Config 1 | dB | 5 | -4 | -10 | 1 | 5 |

### Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode

In this test in sync parameters are agreed to be distributed and OOS to be non-distributed. Not sure if this is the intended configuration or an error in the test case description. We suggested 15 PRB to be non-interleaved and 12 to be inter-leaved. Based on proposal 2 and proposal 3 we suggest following table for this test case.

- SNR levels for Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM RS | Config1 | dB | 1 | -6 | -12 | -3 | 1 |
|  | Config 2 | dB | 5 | -4 | -10 | 1 | 5 |

## BFD test case

SNR level of SSB for q0

Figure 3: SNR variation for BFD resource

For the BFD test we need to determine Qout_LR_SSB and based on Qout_LR_SSB, we need to set SNR levels at T1 to T5 as shown in figure. We suggest considering same Qout value for Qout_LR_SSB. That means SNR1= 1dB SNR2=-6dB and SNR3=-12dB.

SNR level of SSB for q1:

SNR levels for q1 resources are set based on the rsrp-ThresholdSSB. Since L1-RSRP requirements are not impacted with less than 5 MHZ, we suggest considering the same SNR levels as legacy test case.

Figure 4: SNR variation for CBD

- SNR level of SSB for q0 and q1 are given by following

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 1 | -6 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |

# Summary and conclusions

- For RLM and BFD test cases, for TDL-C 100 Hz channel model, RAN4 to agree margin1 = 3 dB and margin2 = 2 dB

- SNR levels for Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS for 15 PRB Coreset are given by following table

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| SNR on RLM RS | Config 1 | dB | 1 | -6 | -12 |

- SNR levels for Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS for 12 PRB Coreset are given by following table

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM RS | Config 1 | dB | 5 | -4 | -10 | 1 | 5 |

- SNR levels for Radio Link Monitoring In-sync Test for FR1 PCell with 3MHz Channel Bandwidth configured with SSB-based RLM RS in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR on RLM RS | Config1 | dB | 1 | -6 | -12 | -3 | 1 |
|  | Config 2 | dB | 5 | -4 | -10 | 1 | 5 |

- SNR level of SSB for q0 and q1 are given by following

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_SSB of set q0 | Config 1 | dB | 1 | -6 | -12 | -12 | -12 |
| SNR_SSB of set q1 | Config 1 | dB | -10 | -10 | 10 | 10 | 10 |
| SSB_RP of set q1 | Config 1 | dBm/SCS kHz | -108 | -108 | -88 | -88 | -88 |

# References

- R4-2410431

# Annexe

In previous meeting, following PDCCH parameters are agreed for RLM or BFD OOS and IS.

Table 8.1.2.1-3: PDCCH transmission parameters for out-of-sync evaluation for [less than 5MHz UE]

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) | 5MHz (20 PRBs) |  |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |  |
| Aggregation level (CCE) | [4] | [8] | [8] |  |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 4dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 4dB |  |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |  |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE | Distributed | Non-Distributed and Distributed |  | Distributed |

Table 8.1.2.1-4: PDCCH transmission parameters for in-sync evaluation for [less than 5MHz UE]

| Attribute | Value for BLER Configuration #0 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3MHz (12 PRBs) | 3MHz (15 PRBs) | 5MHz (20 PRBs) |  |
| DCI format | 1-0 |  |  |  |
| Number of control OFDM symbols | [2] | [3] | [3] |  |
| Aggregation level (CCE) | [2] | [4] | [4] |  |
| Ratio of hypothetical PDCCH RE energy to average SSS RE energy | 0dB |  |  |  |
| Ratio of hypothetical PDCCH DMRS energy to average SSS RE energy | 0dB |  |  |  |
| Bandwidth (PRBs) | 12 | 15 | 20 |  |
| Sub-carrier spacing (kHz) | SCS of the active DL BWP |  |  |  |
| DMRS precoder granularity | REG bundle size |  |  |  |
| REG bundle size | 6 |  |  |  |
| CP length | Normal |  |  |  |
| Mapping from REG to CCE | Distributed | Non-Distributed and Distributed |  | Distributed |

We have performed following set of simulations based on the above assumptions and the simulation parameters agreed during the core part of the discussion.

Out of sync simulations:

- 12 PRB interleaved with TDL-C and above OOS parameters

- 15 PRB interleaved with TDL-C and above OOS parameters

- 15 PRB non-interleaved with TDL-C and above OOS parameters

- 20 PRB interleaved with TDL-C and above OOS parameters

- 12 PRB interleaved with HST and above OOS parameters

In sync simulations:

- 12 PRB interleaved with TDL-C and above IS parameters

- 15 PRB interleaved with TDL-C and above IS parameters

- 15 PRB non-interleaved with TDL-C and above IS parameters

- 20 PRB interleaved with TDL-C and above IS parameters

- 12 PRB interleaved with HST and above IS parameters

We present results in the following table for relevant simulation assumption.

Simulation Assumption:

PDCCH BW: 15 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 9 PRBs are punctured and inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -8.66 |  | 7.8 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -0.855 |  |

Simulation Assumption:

PDCCH BW: 20 PRBs, Number of CORESET symbols: 3; Carrier freq.: 900 MHz; the CORESET is generated from 24-PRB CORESET 0 of which top 4 PRBs are punctured and inter-leaved, Antenna config: 2*2, low

Simulation Result:

| Scenario | AL | Channel | OOS (with -4 dB adjustment to 10% BLER) | SNR dB for 2% BLER In-Sync | Difference between IS-OOS |
| --- | --- | --- | --- | --- | --- |
| OOS | 8 | TDL-C 300ns 100 Hz | -8,04762 |  | 3,27125 |
| IS | 4 | TDL-C 300ns 100 Hz |  | -4,77637 |  |