3GPP TSG RAN WG4 Meeting #113	R4-2419298

Orlando, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	7.24.2.1

Source: 	Ericsson

Title:	Discussion on measurements enhancements for purpose of supporting LTM

Document for:	Discussion

# Introduction

In this contribution we discuss Event triggered measurement requirements based L1 measurement requirements.

# Discussion

## Event triggered measurement requirements

### Impact of event triggered to reporting to LTM procedures

In Rel-19, RAN2 is introducing event triggered L1 measurement reporting to reduce the signalling overhead in the UL due to the periodic LTM CSI reports in the UL. RAN2 agreed on introducing the following events for the purpose of triggering measurement reporting and the reporting content for the event triggered reporting is under further study in RAN2.

| Event | Description |
| --- | --- |
| Event LTM2 | Beam of serving cell becomes worse than absolute threshold; |
| Event LTM3 | Beam of candidate cell becomes amount of offset better than beam of serving cell; |
| Event LTM4 | Beam of candidate cell becomes better than absolute threshold; |
| Event LTM5 | Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. |

Before discussing the RAN4 RRM requirements scope, we look at the intention or use case of the RAN2 introduced events.

- Event LTM2: As the name suggests this event is similar to event A2 for the event triggered L3 measurements. When the beam of the serving cell becomes worse than absolute threshold, NW may request an aperiodic report to schedule early TCI state activation or PDCCH order based RACH.

- Event LTM3: As the name suggests this event is similar to event A3 for the event triggered L3 measurements. When beam of candidate cell becomes amount of offset better than beam of serving cell, NW can trigger the cell switch based on the event triggered report from the UE.

- Event LTM4: As the name suggests this event is similar to event A4 for the event triggered L3 measurements. When beam of candidate cell becomes better than absolute threshold, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list.

- Event LTM5: As the name suggests this event is similar to event A5 for the event triggered L3 measurements. When Beam of serving cell becomes worse than absolute threshold1 AND beam of candidate cell becomes better than another absolute threshold2, based on the UE report, NW may activate the TCI states of the candidate cell or update the active TCI state list or perform LTM cell switch.

Based on the above analysis we can observe that RAN2 agreed events are designed to request a candidate cell L1 measurement report to perform early DL/UL synchronization on candidate cells or trigger an LTM cell switch.

How to use these events and how many events to configure is up to NW implementation. The above events may be configured with other events or a single event (e.g., NW may configure only event LTM3). Assuming NW may trigger LTM cell switch based on the LTM3, to utilize the potential of LTM, prior to the LTM3 event entering condition is met, NW may have to schedule early TCI state activation and PDCCH order based RACH. Early TCI state activation can be based on L3 measurement reports or based on other L1 event triggered reporting. If the early TCI state activation is based on L3 measurement report and cell switch is based on the event triggered L1 report, current TCI state known conditions may not be valid as the existing known conditions (at least for TCI state activation) are defined based on the periodic L1-RSRP report.

Rel-18 candidate cell TCI state activation delay requirements are applicable for known TCI states and for unknown TCI states under certain scenarios. Rel-18 TCI state known conditions and conditions (clause 8.25 of TS 38.133) under which TCI state activation requirements are applicable (clause 6.3.1 of TS 38.133) are mentioned in below table.

Known conditions of the TCI state: The candidate TCI state in the TCI state activation list is known if the following conditions are met: - During the period from the last transmission of the RS resource used for the L1-RSRP measurement reporting for the target downlink TCI state to the completion of active downlink TCI state activation, where the RS resource for L1-RSRP measurement is the RS in target downlink TCI state or QCLed to the target downlink TCI state - The MAC CE command indicating the activation of downlink TCI state is received within 1280 ms upon the last transmission of the RS resource for beam reporting or measurement - The UE has sent at least 1 valid L1-RSRP report for the target downlink TCI state before the MAC CE command is received. - The target downlink TCI state remains detectable during the downlink TCI state activation period - The SSB associated with the downlink TCI state remain detectable during the downlink TCI activation period - SNR of the downlink TCI state ≥ -3dB Otherwise, the candidate TCI state is unknown. TCI state activation requirements are applicable for: Known TCI state in FR1 and FR2 specifed in clause 8.25.2, and - Unknown TCI state in FR1 specifed in clause 8.25.2, provided the following conditions are met: - UE has reported L3 measurement result with the associated SSB index of the TCI state within 5 seconds before the LTM TCI state activation command. - SNR of the associated SSB is above -3dB.

We may need to consider a scenario where event triggered reporting is configured but the event is not triggered. In this scenario, though UE was measuring the RS, since the report was not sent, as per the existing known condition defintion, TCI state is considered unknown. If the TCI state is unknown, cell switch delay requirements are not applicable as per Rel-18 agreements. Due to this, we think TCI state known condition defintion needs to be modified. Moreover, TCI state activation requirements are still under discussion for Rel-18 LTM. We may need to look at the impact of TCI state activation requirements with the introduction event triggered reporting.

- RAN4 to revisit following requirements to accommodate event triggered LTM reporting

  - Defintion of TCI state known condition

  - TCI state activation requirements

### L1-RSRP requirements for event triggered reporting

In previous meeting, many companies suggested taking event triggered L3 measurement requirements as baseline for L1 event triggered measurement requirements. Existing L3 event triggered reporting delay requirements are defined considering RRC reconfiguration as a starting point keeping the testability aspect in mind. That means, when the test is started, the carrier is not detectable (e.g., during T0, ) and after a certain point (e.g., during T1), the signal level of the carrier is increased to an extent that carrier is detectable and event entering condition is met.

The event triggered measurement requirement for L3 measurement is shown in below table.

The measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE starts to transmit the measurement report over the air interface. This requirement assumes that the measurement report is not delayed by other RRC signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH. This measurement reporting delay excludes a delay which caused by no UL resources being available for UE to send the measurement report on. The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap. If a cell which has been detectable at least for the time period Tidentify intra without index or Tidentify intra with index defined in clause 9.2.5.1 or clause 9.2.6.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again with the same spatial reception parameter and triggers an event, the event triggered measurement reporting delay shall be less than TSSB_measurement_period_intra provided the timing to that cell has not changed more than 3200/2µ Tc while the measurement gap has not been available and L3 filtering has not been used, where µ is the SCS configuration as defined in clause 4.2 of TS 38.211 [3]. When L3 filtering is used, an additional delay can be expected. In EN-DC and NE-DC operation, when the UE is configured to perform E-UTRA SRS carrier-based switching an additional delay can be expected in FR1 if the UE is capable of per-FR gap, or an additional delay can be expected in both FR1 and FR2 if the UE is not capable of per-FR gap.

Figure 1: L3 event triggered measurement period

The requirement highlight in yellow represents the scenario where carrier is configured/detected for the first time or re-detected after 5 secs from last measurement. In this scenario, UE needs to perform cell search with SSB index detection or without SSB index detection based on the report configuration and perform SSB based measurement. The point at which UE realises the event is met is Point C.

The requirement shown in cyan represents the scenario where UE detected the cell within last 5 seconds. In this scenario, the maximum amount of time UE takes to detect event met condition is less than SSB measurement period (in some cases, if UE implemented sliding window, UE may detect the event with one additional sample after the event is met in channel or 5 additonal samples depending on the UE measurement window). The point at which UE realise the event is met is Point B or A based on the UE implementation.

The requirement highlighted in green shows that maximum uncertaininty to insert the report into the DCCH. This can be considered as the report preparation time after the UE realises that the event is met.

- L3 event triggered reporting requirements are specified for unknown cell and known cell.

In Rel-18, LTM L1-RRSP measurements are defined for the known cell scenario. That means UE has measured or sent a L3 measurement report within last 5 seconds. It is reasonable to keep the same assumption for the event triggered measurement reporting too.

- RAN4 to define the event triggered measurement report for known cell scenario.

Event triggered L1-RSRP reporting delay:

In last meeting RAN4 agreed to reuse the start point similar to L3 event triggered measurement reporting delay. Agreement is shown below.

  - Use the following as starting point. Wording can be discussed in CR.

    - Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting.

Since start point followed the L3 event triggered reporting requirements, we think other components can also be adapted. Following observation can be made from the above-mentioned analysis of L3 event triggered reporting delay requirements.

- Event triggered measurement delay for L3 measurement consists of, start point, measurement period, maximum uncertainty to insert the report into the DCCH

Based on the above observation we need to define following components for the L1 event triggered reporting delay.

- Start point

- Measurement period

- Maximum uncertainty to insert report to MAC CE/ Max uncertainty to insert the dedicated SR into the PUCCH

Measurement period:

In Rel-18, LTM L1-RSRP measurement period is based on the single shot measurement (i.e., M is 1). However, event triggered measurement report main purpose is to reduce the signalling overhead in the UL. To avoid false event triggering reports due to fast fading, RAN1 is discussing filtering for the event triggered measurement reporting. If the filtering is agreed, we need to define new measurement delay with higher number of samples than single shot measurement. That means measurement period may contain single shot or multiple samples measurement.

- For the number of samples in measurement period of event triggered measurement delay for L1-RSRP, RAN4 to wait for RAN1 conclusion on filtering.

Maximum uncertainty to insert report to MAC CE/ Max uncertainty to insert the dedicated SR into the PUCCH:

Since the LTM L1 measurement report is used for mobility decisions, UE should be able to send the report as soon as event condition is met. Event triggering conditions may be changed as fast as the measurement occasions. UE is expected to evaluate the event after every measurement occasion to check if any of the configured event is met. When the event entering condition is met, UE sends either SR or special SR based on the following RAN2 agreement.

The legacy SR procedure for resource allocation is the baseline to send the event triggered L1 measurements MAC CE. NW can configure a dedicated SR configuration for MR MAC CE transmission.

Once the event is met, UE needs to send the indication on the SR or special SR to the NW to indicate to the NW that event is met. It can be denoted as TTime to first UL channel. RAN4 to further discuss the components and criteria for defining TTime to first UL channel.

- RAN4 to discuss components and criteria for defining TTime to first UL channel.

End point of the delay: If NW configures the dedicated SR, then dedicated SR transmission can be used as ending point for the event triggered reporting delay. When UE do not support special SR or NW do not configure, RAN4 can further discuss the ending point.

- For event triggered reporting delay, end point is defined as time to first special SR transmission. RAN4 to discuss if we need to consider scenario when UE supports normal SR only.

### Requirements for reporting criteria per measurement category

In last meeting there was a proposal to share the capability of L3 events with the LTM events.  of the proposal discussed is mentioned below.

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |

We think L3 measurement is normally configured by CU while L1 measurement may be configured by DU. Hence, it would be good to avoid any coordination need between CU and DU on the Ecat capability for LTM L1 events. Moreover, based on our internal check with the product implementation team, we think the existing Ecat capability is fully utilized as different applications may need to be configured with different events for the frequency carrier.

- RAN4 to agree that Ecat capability is not shared between  L3 events and LTM L1 events

RAN4 first needs to discuss and decide on the granularity of the LTM L1 events capability. The Ecat is defined for frequency layer and different Ecat is defined for intra-frequency, inter-frequency and so on. Currently LTM CSI config contains multiple cells consisting of both intra-freq and inter-frequency measurements. We think the number of cells and number of SSB UE can measure with LTM L1 measurements will not change with event triggered measurement reporting. We think this capability as more of the evaluation capability than the measurement capability. Some of the aspects which comes under event evaluation is number of events to evaluate and when the event is met, number of TTT timers UE can maintain. Currently the TTT handling and TTT granularity is being discussed in RAN2. Some of the options which are in discussion in RAN2 are mentioned below.

- TTT is per beam

- TTT is per cell

- TTT is per event

Based on the TTT handling agreed in RAN2, UE capability on the number of events will be impacted. We suggest RAN4 to wait for further RAN2 progress on the TTT handling.

- RAN4 to wait for RAN2 progress on the TTT handling before discussing the UE capability of number of events to support.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- RAN4 to revisit following requirements to accommodate event triggered LTM reporting

  - Defintion of TCI state known condition

  - TCI state activation requirements

- RAN4 to define the event triggered measurement report for known cell scenario.

- For the number of samples in measurement period of event triggered measurement delay for L1-RSRP, RAN4 to wait for RAN1 conclusion on filtering.

- RAN4 to discuss components and criteria for defining TTime to first UL channel.

- For event triggered reporting delay, end point is defined as time to first special SR transmission. RAN4 to discuss if we need to consider scenario when UE supports normal SR only.

- RAN4 to agree that Ecat capability is not shared between L3 events and LTM L1 events

- RAN4 to wait for RAN2 progress on the TTT handling before discussing the UE capability of number of events to support.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2414060, WF on NR mobility enhancements Phase 4