3GPP TSG-RAN WG4 Meeting #113									R4-2419300

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.12.3.1

Source:	CAICT, SAICT

Title:	Discussion on TRP TRS requirements

Document for:	Approval

1	Introduction

In this contribution, we provide our views on Rel-19 TRP TRS requirements.

2	Discussion

2.1 2Tx requirements for TRP and TRS

According to the previous WF [1], the measurement campaign for non-coherent UL MIMO at n41/n78 should take place in Rel-19, with a need to verify the feasibility of obtaining a sufficient number of commercial devices.

Issue 4-1-5: 2Tx requirements related performance measurement campaign Agreements: - RAN4 need to check the feasibility of obtaining a sufficient number of commercial devices that support non-coherent UL MIMO. - Encourage vendors to provide input and support, facilitating the gathering of relevant data on available devices.

After an initial survey of several top vendor brands in the regional market, most vendors have only 1-2 series supporting non-coherent UL MIMO, and some brands do not yet support this feature. Given the difficulty in obtaining a sufficient number of 2Tx phones to meet the minimum sample size requirement, it is recommended to relax the minimum sample size for the 2Tx measurement campaign.

Proposal 1: It is recommended to relax the minimum sample size for the 2Tx measurement campaign.

2.2 Size 2 TRP TRS requirements (PDA hand)

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (assume no large scale measurement campaign, current requirement is a basis) Agreements: - RAN4 can initiate a preliminary (not large scale) measurement campaign for size 2 UEs, FFS how to down select between measurement-based approach and the offset-based approach. - Encourage volunteer test labs to perform the above measurement campaign, Rel-17 and/or Rel-18 aligned test lab is preferred. - Encourage companies to provide seize 2 UEs (small phone with PDA hand) for preliminary measurements. - Note: Size 1 (wide, width >72mm and ≤92mm), Size 2 (narrow, width ≥56mm and ≤72mm);

In the last meeting, RAN4 agreed to initiate a preliminary measurement campaign for size 2 UEs and FFS how to specify PDA hand related TRP/TRS requirements. At present, there are relatively few commercial smartphones that meet size 2, and RAN4 should clarify the required number of test samples for the preliminary measurement campaign for Size 2.

Proposal 2: RAN4 should clarify the required number of test samples for the preliminary measurement campaign for Size 2.

In addition, some landscape fold phones have a width of less than 72mm when closed, and RAN4 should clarify whether these phones should be included in the preliminary testing of Size 2 UE.

Proposal 3: RAN4 should clarify whether landscape fold phones with width less than 72mm when closed should be included in the preliminary testing of Size 2 UE.

3	Conclusion

This contribution makes the following observations and proposals.

Proposal 1: It is recommended to relax the minimum sample size for the 2Tx measurement campaign.

Proposal 2: RAN4 should clarify the required number of test samples for the preliminary measurement campaign for Size 2.

Proposal 3: RAN4 should clarify whether landscape fold phones with width less than 72mm when closed should be included in the preliminary testing of Size 2 UE.

References

- R4-2416590, WF for [112][323] TRP_TRS_ph3, vivo