3GPP TSG-RAN WG4 Meeting #113	R4-2419325

Orlando, Florida, USA, November 18 – 22, 2024

Source:	Nokia

Title:	Testability and interoperability issues for positioning accuracy enhancement

Agenda item:	7.17.5.2

Document for:	Discussion

Introduction

In RAN#102, the Release-18 study on artificial intelligence (AI)/machine learning (ML) for NR air interface (FS_NR_AIML_Air) [1] was completed [2] and the findings (with agreements and open issues) were documented in the outcome technical report 3GPP TR 38.843 V18.0.0 [3]. Furthermore, a new Release-19 work item on artificial intelligence (AI)/machine learning (ML) for NR air interface (NR_AIML_Air) [4] was also approved in RAN#102 to start the normative work for the general AI/ML framework for air interface and to enable the recommended use cases in the preceding study.

During the SI, as mentioned in the extract from 3GPP TR 38.843 V18.0.0 [3] below, following sub use cases for positioning accuracy enhancement are considered:

The following are selected as representative sub-use cases: - Direct AI/ML positioning: - AI/ML model output: UE location - e.g., fingerprinting based on channel observation as the input of AI/ML model - AI/ML assisted positioning: - AI/ML model output: new measurement and/or enhancement of existing measurement - e.g., LOS/NLOS identification, timing and/or angle of measurement, likelihood of measurement More specifically, the following Cases are considered for the study: - Case 1: UE-based positioning with UE-side model, direct AI/ML or AI/ML assisted positioning - Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning One-sided model whose inference is performed entirely at the UE or at the network is prioritized in Rel-18 SI.

During SI, RAN4 studied different test metrics for performance requirements for AI/ML enabled positioning use case.  Here is an extract from 3GPP TR 38.843 V18.0.0 [3] covering these aspects:

Both direct AI/ML positioning and AI/ML assisted positioning are considered. For metrics for positioning requirements/tests, the candidate options include - Option 1: positioning accuracy: Ground truth vs. reported - only option available for direct positioning - Option 2: CIR/PDP, channel estimation accuracy - Option 3: ToA, RSTD and RSRP, and RSRPP - Option 4: others (e.g., intermediate KPIs, LoS/NLoS)/combinations of the above The feasibility and testability of different options should be further justified in WI.

In RAN4#110, the following has been agreed on the testability and interoperability issue for AI/ML positioning [5].

2.3 Testability and interoperability issues for positioning accuracy enhancement 2.3.1 Agreements in ad-hoc session (R4-240xxxx) Issue 3-2: Requirements for case 3a/3b RAN4 will not define positioning accuracy requirements for case 3a/3b Issue 3-6: Requirements for case 2a/2b RAN4 to come back to case 2a/2b based on progress in the other working groups

In RAN4#110bis, the following has been agreed on the testability and interoperability issue for AI/ML positioning [6].

Issue 3-1: Requirements for case 1 Agreement: postpone discussion until reporting scheme(if defined) is clear. if reporting scheme is introduced, RAN4 will further discuss whether to define requirements or not. RAN4 will not define any accuracy requirements if no reporting scheme is introduced

In RAN4#111, the following has been agreed on the testability and interoperability issue for AI/ML positioning [7].

Issue 3-2: Requirements for case 2a Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups Issue 3-3: Requirements for case 2b Agreement: No requirements on LMF for positioning accuracy FFS on RAN4 requirements for any UE reported measurements defined by other groups

In RAN4#112, the following has been agreed on the testability and interoperability issue for AI/ML positioning [8].

Issue 3-2: Requirements for case 2a Agreement: RAN4 should not define any positioning accuracy requirements because positioning is LMF based Issue 3-3: Reported metrics for case 2a Agreement: - RAN4 will take existing requirements for existing metrics as baseline - FFS whether to revisit these requirements - RAN4 will study how to define requirement for any new reported metrics - FFS how to define requirements for LOS/NLOS Issue 3-5: Reported metrics for case 2b Agreement: postpone discussion on what and how to define requirements until RAN1 agrees to what metrics are reported. Issue 3-6: Reported metrics for case 3a/3b Agreement: postpone discussion until RAN1 agrees on any metric to be reported by gNBs.

In RAN4#112bis, the following has been agreed on the testability and interoperability issue for AI/ML positioning [9].

Issue 3-1: Requirements for case 1 Agreement: Companies are invited to bring further analysis on feasibility of defining requirements for case 1 and how to define requirements, if feasible Issue 3-3: Report applicability for existing reported metrics Agreement: Extend applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

The issues on AI/ML based positioning require further discussion on selected sub use cases. In this paper, the performance requirements for prioritized Case 1 and Cases 3a/3b are discussed in regard to aspects of performance test metric, measurement accuracy requirements, testability, and LCM-related latency requirements.

Discussion

UE-based and UE-assisted positioning

This section discusses testability aspects and performance test metric in the use cases with a UE-side model. Considering that RAN#105 decided that RAN1 will not focus on 2nd priority cases anymore since RAN1#118bis, we exclude Cases 2a/2b here.

Testability aspects in Case 1

In RAN4#110, we discussed the testability of Case 1 where the test metric in Case 1 can be the positioning accuracy.  Therefore, in this paper, we further discuss the testability aspects of positioning accuracy test metric. To define the feasible requirements for Case 1, it is useful to consider a possible test framework with the four essential steps of identification, setting, running, and evaluation as shown in Figure 1.

Figure 1. Test framework in Case 1

When AI/ML inference results consist of positioning coordinates, the AI/ML functionality can be verified based on the ground truth. The ground truth may consist of the location point(s) with known positioning co-ordinates, for example, in form of a test area/grid as shown in Figure 2. To know the co-ordinates of each location point, a PRU (i.e., Positioning Reference Unit location is known to gNB/LMF) is placed, or the position co-ordinates of each location point form a known reference location derived by, e.g., using GNSS based positioning. In that case, while UE moves from one location point to another location point, the AI/ML inference results at each location corresponding to the location point is reported to the test equipment and compared to the ground-truth information for verification of positioning accuracy.

Figure 2. Grid map providing a ground-truth information for inference evaluation

The test setup can be implemented for a field test. If testing is performed in a limited indoor space, the same test setup can be scaled down to be fitted into the laboratory space. For instance, the test map granularity may be further reduced to a certain level depending on the achievable precision of an AI/ML-based positioning technique, and configured bandwidth size, and channel conditions.

Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based).

RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1.

Regarding test mechanisms for positioning accuracy, a grid map in a limited indoor space should be defined to derive the ground truth. The UE is placed on one of these location points and receives PRS resources from a set of TRPs with known location, the set size may be e.g. four. The PRS resources are on the same PFL. The measurement period is derived from the set size of TRPs for single PFL according to TS 38.133. The UE reports its position co-ordinates to the test system (emulating LMF), which compares the reported co-ordinates to ground-truth.

RAN4 to specify a test grid map with known location points in indoor space and an appropriate set of TRPs for the positioning measurement in Case 1.

Performance test metric in Case 1

The metric for positioning accuracy for Case 1 identifies the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth, i.e. Δx, Δy and Δz, using a Local Coordinate System (LCS) or a Global Coordinate System (GCS) reference and can be derived according to Eq.1:

e= Δx2+Δy2+Δz2	 (Eq.1)

RAN4 to select the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth as performance metric for the positioning accuracy in Case 1.

gNB assisted positioning

This section discusses testability aspects and performance metric in the use cases with a gNB-side model (use cases 3a/3b).

Report mapping aspects in Case 3a / Case 3b

- RAN4#112bis agreed to extend the applicability of existing report mappings for UL-RToA and UL SRS-RSRPP to report UL measurements for AI/ML based positioning use cases 3a/3b.

- Clause 13 of TS 38.133 on Measurement Performance Requirements for NR gNB also specifies report mapping for following UL measurement types: UL SRS RSRP and AoA/ZoA. Whilst angular measurements were not discussed in RAN1 for AI/ML based positioning, it is noted that the UL SRS-RSRPP measurement type was introduced in TS 38.215 in Rel-17 to complement the UL SRS-RSRP measurement. Thus, it is proposed to extend also the applicability of existing report mappings for the UL SRS-RSRP measurement type to AI/ML based positioning.

RAN4 to extend the applicability of existing report mappings also for UL SRS-RSRP to AI/ML based positioning in Cases 3a/3b.

Testability aspects and Performance test metric in Case 3a / Case 3b

- Considering RAN#105 decided that RAN1 will focus on 1st priority cases, such as Case 1, Case 3a and Case 3b, testability for Case 3a and Case 3b is subject to be investigated. Based on the earlier RAN4 #110 agreement not to specify positioning accuracy requirements for Case 3a and Case 3b, there are no testability aspects for Case 3a and Case 3b, except if RAN1 specifies new measurement types for AI/ML-based positioning or enhances existing measurement types. This discussion is currently deferred in RAN1. In the latter case, testability for Case 3a (gNB-side model) and Case 3b (direct positioning, related to gNB measurement output) should be investigated in greater detail.

Based on the earlier RAN4 #110 agreement not to specify positioning accuracy requirements for Case 3a and Case 3b, there are no testability aspects for Case 3a and Case 3b, except if RAN1 specifies new measurement types for AI/ML-based positioning or enhances existing measurement types for AI/ML-based positioning.

RAN4 to consider testability aspects for Case 3a and Case 3b only, if RAN1 specifies new measurement types for AI/ML-based positioning or enhances existing measurement types for AI/ML-based positioning.

Conclusion

In this paper we share our views on potential RAN4 impacts from issues related to AI/ML based positioning for Case 1 and Cases 3a/3b. In the paper, the following observations and proposals were made:

- Positioning accuracy can be verified based on the ground truth which may consist of the location points with known positioning co-ordinates (e.g., PRU or GNSS based).

- RAN4 should specify (i) the test mechanisms for positioning accuracy and (ii) the metric for positioning accuracy performance in Case 1.

RAN4 to specify a test grid map with known location points in indoor space and an appropriate set of TRPs for the positioning measurement in Case 1.

RAN4 to select the three-dimensional error e of the absolute reported positioning co-ordinates relative to the evaluated ground truth as performance metric for the positioning accuracy in Case 1.

RAN4 to extend the applicability of existing report mappings also for UL SRS-RSRP to AI/ML based positioning in Cases 3a/3b.

- Based on the earlier RAN4 #110 agreement not to specify positioning accuracy requirements for Case 3a and Case 3b, there are no testability aspects for Case 3a and Case 3b, except if RAN1 specifies new measurement types for AI/ML-based positioning or enhances existing measurement types for AI/ML-based positioning.

RAN4 to consider testability aspects for Case 3a and Case 3b only, if RAN1 specifies new measurement types for AI/ML-based positioning or enhances existing measurement types for AI/ML-based positioning.

- References

- RP-221348, Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#96, Budapest, Hungary, June 06 – June 09, 2022.

- RP-233132, Status report for SI on AI/ML for NR air interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- 3GPP TR 38.843 V18.0.0, Technical Report (TR): Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface.

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#102, Edinburgh, Scotland, December 11 – December 15, 2023.

- R4-2406617, WF on AI/ML, Qualcomm, RAN4#110bis, Changsha, China, April 15 - 19, 2024.

- R4-2403712, WF on AI/ML, Qualcomm, RAN4#110, Athens, Greece, February 26 – March 1, 2024.

- R4-2410570, WF on NR ALML air interface, Qualcomm, RAN4#111, Fukuoka, Japan, May 20 – 24, 2024.

- R4-2404323, WF on NR ALML air interface, Qualcomm, RAN4#112, Maastricht, Netherlands, Aug. 19 – 23, 2024.

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm, RAN4#112bis, Hefei, China, Oct. 14 – 18, 2024.