3GPP TSG-RAN WG4 Meeting #113	R4-2419326

Orlando, Florida, USA, November 18 – 22, 2024

Source:	Nokia

Title:	General aspects for XR enhancements

Agenda item:	7.25.1

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on general aspects for XR enhancements [1] with regard to the previously received RAN1 LS [2] and the TU allocation for RAN4 work related to NR_XR_Ph3. As part of this discussion, we provide corresponding observations and proposals.

Reply LS to RAN1

In the RAN4 #112bis meeting, the following issue was left for discussion in the agreed WF [3]:

## Topic#7: Reply LS to RAN1 ## Issue 7-1: Response on RAN1 working assumption and time offset <Way forward>: - Companies are encouraged to bring analysis on the feasibility of the working assumption from the RAN1 LS in R4-2414910.

In RAN1#118 it was decided that DCI-based indication of measurement skipping would be considered as working assumption by RAN1. As part of the conclusion, RAN1 sent an LS to RAN4 and tasked RAN4 to inform RAN1 if there are issues related to that decision [2].

From RAN4 point of view, we can cite at least few cases where similar requirements were made, where UEs had to adapt measurements in a dynamic manner.

In Rel-16, RAN4 has defined requirements for NR_unlicensed. In unlicensed bands all transmissions are subject to CCA failures, which means that some SMTC occasions were not available to be measured by the UEs. The availability of the SMTC occasions was considered as part of the measurement delay requirements.

In Rel-17, RAN4 has defined requirements for Pre-configured measurement gaps. In one example, PreMGs can be activated/deactivated for example by DCI containing BWP switch command. In that case, one measurement gap patterns may be activated or deactivated dynamically, and the UE has to adapt measurement procedure accordingly.

Therefore, the important aspects of Alt 1-1 from RAN4 perspective are

- the definition of the measurement delay requirements, and

- the timeline for DCI-based measurement skipping

RAN4 already defined requirements for features where dynamic adaptation of measurements were necessary, e.g. NR-U and Pre-configured gaps.

RAN4 to reply to RAN1 that Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on timeline.

We have included a proposal for the draft LS in the Annex A of this paper.

TU allocation for RAN4 RRM work

At RAN#105, the proposal [4] was submitted to extend the RAN4 RRM TU allocation from 0.5 to 1.0 TU, which was not agreed.

It is noted aside the increase of discussion time at RAN4 #112, a total of 2.5 h was also spent at RAN4 #112bis, including 1 online session (1.5 h), 1 coffee break discussion (0.5 h) and 1 come-back discussion (0.5 h), which significantly exceeds the amount of 0.5 TU (1 h) in the WID [1].

There was a document load of 19 contributions to XR enhancements at RAN4 #112bis meeting, which resulted in 35 different issues in the moderator summary [6], and this number is expected to further increase.

Considering on one hand the many aspects for RRM requirements to be discussed in RAN4 related to XR enhancements as observed in our contribution [5] and other contributions, and on the other hand the tight time frame for Rel-19, we believe the current TU allocation of 0.5 (= 1h) is clearly insufficient.

Considering on one hand the many aspects for RRM requirements to be discussed in RAN4 related to XR enhancements and the current duration of online discussion estimated to be 1.25 TU, thus well exceeding 0.5 TU, and the tight time frame for Rel-19, the current TU allocation of 0.5 TU for RRM is considered clearly insufficient.

Thus, we plan to submit a renewed request to RAN #106 in order to increase the RAN4 TU allocation for RRM requirements from 0.5 to 1.0 TU.

RAN4 to request a higher TU allocation for the NR_XR_Ph3 work at RAN#106, in particular to increase TU allocation from 0.5 to 1.0 TU starting from RAN4 #114.

We invite feedback from other companies on this proceeding.

Conclusion

In this paper we have presented Nokia’s views with regard to the previously received RAN1 LS and the TU allocation for RAN4 related to XR ph3. As part of this discussion, the following Observations and Proposals were made:

Reply LS to RAN1

- RAN4 already defined requirements for features where dynamic adaptation of measurements were necessary, e.g. NR-U and Pre-configured gaps.

- RAN4 to reply to RAN1 that Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on timeline.

It is noted we have included a proposal for the draft LS in the Annex A of this paper.

TU allocation for RAN4 RRM work

Considering on one hand the many aspects for RRM requirements to be discussed in RAN4 related to XR enhancements and the current duration of online discussion estimated to be 1.25 TU, thus well exceeding 0.5 TU, and the tight time frame for Rel-19, the current TU allocation of 0.5 for RRM is considered clearly insufficient.

RAN4 to request a higher TU allocation for the NR_XR_Ph3 work at RAN#106, in particular to increase TU allocation from 0.5 to 1.0 TU starting from RAN4 #114.

- References

- RP-241771, Revised WID: XR (eXtended Reality) for NR Phase 3, RAN #105, Melbourne, Australia, September 9-12, 2024

- R1-2407561, LS on gaps/restrictions that are caused by RRM measurements, RAN4 #112-bis, Hefei, China, October 14 – 18, 2024

- R4-2416864, WF on RRM requirements for XR_Ph3, Nokia, RAN4 #112bis, Hefei, China, October 14 – 18, 2024

- RP-242096, Views on XR for NR Phase 3 WI, Nokia

- R4-2416468, Discussion on XR ph3 RRM requirements, Nokia, RAN4 #112bis, Hefei, China, October 14 – 18, 2024

- R4-2415667, Topic summary for [112bis][220] NR_XR_Ph3, Nokia, RAN4 #112bis, Hefei, China, October 14 – 18, 2024

- R4-2418890, XR simulation results, Nokia, RAN4 #113, Orlando, US, Nov 18th – 22nd, 2024

- Annex A: Draft LS response to RAN1

3GPP TSG-RAN WG4 Meeting #113	R4-24xxxxx

Orlando, Florida, USA, November 18 – 22, 2024

Title:	Response LS on gaps/restrictions that are caused by RRM measurements

Response to:	R4-2414910 (R1-2407561)

Release:	Rel-19

Work Item:	NR XR Phase 3 (NR_XR_Ph3-Core)

Source:	RAN4

To:	RAN1

Cc:

Contact person:	Rafael Paiva

Rafael . paiva (at) Nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN4 thanks RAN1 for the LS on gaps/restrictions that are caused by RRM measurements.

RAN4 confirms that Alt 1-1 is feasible from RAN4 perspective and RAN4 is working on the related requirements on minimum time offset.

# 2	Actions

To RAN1

ACTION:  RAN4 respectfully asks RAN1 to take the above information in consideration.

# 3	Dates of next TSG RAN WG 1 meetings

TSG-RAN WG4 Meeting #114		17 – 21 February 2025				Athens, GR

TSG-RAN WG4 Meeting #114-bis		07 – 11 April 2025				China