3GPP TSG-RAN WG4 Meeting #113	R4-2419331

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.16.2

Source:	Ericsson

Title:	Simulation results for MMSE-IRC receiver for 8Rx reception

Document for:	Information

# 1	Introduction

In RAN4 #112bis meeting, companies have agreed on the interference profile and parameter assumptions for the initial study on the PDSCH performance of both MRC and IRC with 8Rx reception in inter-cell interference and intra-cell inter-user interference scenarios.

In this contribution, we shared our simulation results for both scenarios to support further down-selection of candidate options.

# 2	PDSCH requirements with Inter-cell interference

In this section, we first exhibited the simulation assumption that we used for evaluation, then we shared our simulation results comparing the performance with baseline receiver and MMSE-IRC receiver.

## 2.1	Simulation assumption

In our initial evaluations, for other parameters that are not included in the WF, we mostly reused the parameter assumption listed in the table 5.2.2.1.15-2 in TS38.101-4 with slightly extension for 8Rx reception:

Table 2.1-1: Test parameters

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for test 1-6, Disable for test 7-12 |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for Test 1-6 N/A for Test 7-12 |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for Test 1-6 7.58 for Test 7-12 | 2.29 for Test 1-6 N/A for Test 2-1, 2-2 |
| SSB configuration | SSB position in burst |  | First SSB in Slot #0 | 1st SSB in Slot#0 for Test 1-6 2nd SSB in Slot #0 for Test 7-12 | 1st SSB in Slot#0 for Test 1-6 N/A for Test 7-12 |
|  | SSB periodicity | ms | 20 | 20 | 20 |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 |  |  |  |  |  |

## 2.2	Simulation results

For the simulation results below, we have used the baseline receiver.

Table 2.2-1 PDSCH simulation results for 2 and 1 interference cell (baseline)

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 2 interference cells |  |  |  |  |  |  |  |  |
| 1 | 1 | 10 / 15 | MCS17 | TDLC300-100 | 2x8, ULA Low | 8.65 | 11.07 | 2.42 |
| 2 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 7.26 | 13.22 | 5.96 |
| 3 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 9.21 | 14.47 | 5.26 |
| 4 | 1 | 10 / 15 | MCS19 | TDLC300-100 | 2x8, ULA Low | 10.89 | 12.40 | 1.51 |
| 5 | 2 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 4.02 | 10.27 | 6.25 |
| 6 | 2 | 10 / 15 | MCS19 | TDLA30-10 | 4x8, ULA Low | 9.03 | 14.53 | 5.50 |
| 1 interference cell |  |  |  |  |  |  |  |  |
| 7 | 1 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 4.93 | 7.90 | 2.97 |
| 8 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 6.20 | 11.10 | 4.90 |
| 9 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 7.30 | 12.83 | 5.53 |
| 10 | 1 | 10 / 15 | MCS19 | TDLA30-10 | 2x8, ULA Low | 6.50 | 9.23 | 2.7 |
| 11 | 2 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 2.76 | 8.27 | 5.5 |
| 12 | 2 | 10 / 15 | MCS19 | TDLA30-10 | 4x8, ULA Low | 7.53 | 12.85 | 5.3 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |

For the simulation results below, we have used the simplified receiver.

Table 2.2-2 PDSCH simulation results for 2 and 1 interference cell (simplified)

| Test num | Rank | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS (Table 1) | Propagation condition | Correlation matrix and antenna configuration | Reference value 70% max Tput |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 2 interference cells |  |  |  |  |  |  |  |  |
| 1 | 1 | 10 / 15 | MCS17 | TDLC300-100 | 2x8, ULA Low | 8.17 | 11.10 | 2.93 |
| 2 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 9.07 | 13.25 | 4.18 |
| 3 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 11.24 | 14.46 | 3.22 |
| 4 | 1 | 10 / 15 | MCS19 | TDLC300-100 | 2x8, ULA Low | 9.73 | 12.43 | 2.70 |
| 5 | 2 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 5.78 | 10.21 | 4.43 |
| 6 | 2 | 10 / 15 | MCS19 | TDLA30-10 | 4x8, ULA Low | 10.74 | 14.55 | 3.81 |
| 1 interference cell |  |  |  |  |  |  |  |  |
| 7 | 1 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 4.89 | 7.88 | 2.99 |
| 8 | 2 | 10 / 15 | MCS17 | TDLA30-10 | 2x8, ULA Low | 7.1 | 11.09 | 3.95 |
| 9 | 4 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 8.6 | 12.82 | 4.2 |
| 10 | 1 | 10 / 15 | MCS19 | TDLA30-10 | 2x8, ULA Low | 6.4 | 9.17 | 2.8 |
| 11 | 2 | 10 / 15 | MCS13 | TDLA30-10 | 4x8, ULA Low | 3.7 | 8.43 | 4.7 |
| 12 | 2 | 10 / 15 | MCS19 | TDLA30-10 | 4x8, ULA Low | 8.5 | 12.93 | 4.5 |
| Note 1: The propagation conditions for Cell 1, Cell 2 and Cell 3 are statistically independent. Note 2: Bandwidth/ Sub carrier spacing, Propagation Condition, Correlation matrix and antenna configuration parameters apply for each of Cell 1, Cell 2 and Cell 3. |  |  |  |  |  |  |  |  |

# 3	PDSCH requirements with Intra-cell inter-user interference

In this section, we first exhibited the simulation assumption that we used for evaluation, then we shared our simulation results comparing the performance with baseline receiver and MMSE-IRC receiver.

## 3.1	Simulation assumption

Table 3.1-1: Test parameters

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | 1000 | 1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 | 1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH for test 4 and 6; Orthogonal for test 1,2,3,5,7 |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 |  |
| Number of HARQ Processes |  |  | 4 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

## 3.2	Simulation results

For the simulation results below, we have used the baseline receiver.

Table 3.2-1 PDSCH simulation results

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 1 | 1+1 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 2.13 | 3.08 | 0.95 |
| 2 | 2+2 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 5.64 | 13.03 | 7.39 |
| 3 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | 13.9 | N/A | INF |
| 4 | 1+1 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 3.98 | 10.63 | 6.65 |
| 5 | 1+1 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 5.31 | 8.64 | 3.33 |
| 6 | 1+1 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 7.28 | 26.31 | 19.03 |
| 7 | 2+2 | 10 / 15 | MCS19 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 10.73 | N/A | INF |

For the simulation results below, we have used the simplified receiver.

Table 3.2-2 PDSCH simulation results

| Test num | Rank combination (target+ co-scheduled) | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate |  | Propagation condition | Correlation matrix and antenna configuration | Reference value (70% max Tput) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Target UE | Co-scheduled UE |  |  | MMSE-IRC SNR(dB) | MMSE SNR(dB) | Gain |
| 1 | 1+1 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 2.04 | 3.08 | 1.04 |
| 2 | 2+2 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 6.6 | 13.03 | 6.43 |
| 3 | 4+4 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 8x8, ULA Low | N/A | N/A | N/A |
| 4 | 1+1 | 10 / 15 | MCS13 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 3.9 | 10.71 | 6.81 |
| 5 | 1+1 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 5.41 | 8.64 | 3.23 |
| 6 | 1+1 | 10 / 15 | MCS17 | Random 16QAM symbols | TDLA30-10 | 2x8, ULA Low | 7.43 | 25.64 | 18.21 |
| 7 | 2+2 | 10 / 15 | MCS19 | Random 16QAM symbols | TDLA30-10 | 4x8, ULA Low | 12.42 | N/A | INF |

# 4	Summary

In this contribution, we shared the PDSCH simulation assumptions and results for both inter-cell and intra-cell interference scenarios.

# References

[1]	RP-2416555: WF for [112bis][320] NR_demod_Ph5_Part2_UE, Nokia, RAN4 #112bis, Hefei, Anhui province, China