3GPP TSG-RAN4 Meeting # 113 	R4-2419336

# Orlando, US, 18th - 22nd November

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0134> | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to 38.101-5: Correction on UE demodulation requirement for NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_enh_Perf |  |  |  |  |  | Date: |  |  | 2024-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | There are several ‘TBD’ in the chapter 8. Plus, the reference number is missing when refering to a spec. The description at the beginning of the chapter is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Replace ‘TBD’ with correct information - Add correct reference number - Correct the description at the beginning of the chapter |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for NTN are not complete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1, 8.1.3.4.2, 8.2.1.1.2, 11.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-5 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------------- Begin of change 1-------------------------------------------------------------

### 8.1.1	Relationship between minimum requirements and test requirements

The present document is a Single-RAT and interwork specification for NR UE, covering minimum performance requirements of both conducted and radiated requirements. Conformance to the present specification is demonstrated by fulfilling the test requirements specified in the conformance specification TS 38.521-5 [2].

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification TS 38.521-5 [2] defines test tolerances. These test tolerances are individually calculated for each test. The test tolerances are used to relax the minimum requirements in this specification to create test requirements.

The measurement results returned by the test system are compared – without any modification – against the test requirements as defined by the shared risk principle.

The shared risk principle is defined in Recommendation ITU-R M.1545 [3].

The applicability of each requirement is described under each sub-clause in [8.2.1] and [8.3.1].

-------------------------------------------------------------End of change 1----------------------------------------------------------------

------------------------------------------------------------- Begin of change 2-------------------------------------------------------------

##### 8.1.3.4.2	Noc for NR operating bands in FR1

The Noc power spectrum density shall be larger or equal to the minimum Noc power level for each operating band supported by the UE as defined in clause 8.1.3.4.2.1.

Unless otherwise stated, a fixed Noc power level of -145 dBm/Hz shall be used for all operating bands.

8.1.3.4.2.1	Derivation of Noc values for NR operating bands in FR1

The minimum Noc power level for an operating band, subcarrier spacing and channel bandwidth is derived based on the following equation:

NocBand_X, SCS_Y, CBW_Z = REFSENSBand_X, SCS_Y, CBW_Z – 10*log10(12*SCS_Y*nPRB) + D – SNRREFSENS + ∆thermal

where

-	REFSENSBand_X, SCS_Y, CBW_Z is the REFSENS value in dBm for Band X, SCS Y and CBW Z specified in Table 7.3.2-1

-	12 is the number of subcarriers in a PRB

-	SCS Y is the subcarrier spacing associated with the REFSENS value

-	nPRB is the maximum number of PRB for SCS Y and CBW Z associated with the REFSENS value, and is specified in Table 5.3.2-1

-	D is diversity gain equal to 3 dB

-	SNRREFSENS = -1 dB is the SNR used for simulation of REFSENS

-	∆thermal is the amount of dB that the wanted noise is set above UE thermal noise, giving a defined rise in total noise. ∆thermal = 16dB, giving a rise in total noise of 0.1dB, regarded as insignificant.

The calculated Noc value for the baseline of Band n256, 15 kHz SCS, 10 MHz CBW is -146.5 dBm/Hz.

An allowance of 1.5dB is made for future bands, giving an Noc power level of -145 dBm/Hz.

-------------------------------------------------------------End of change 2----------------------------------------------------------------

------------------------------------------------------------- Begin of change 3------------------------------------------------------------

### 8.2.1	General

#### 8.2.1.1	Applicability of requirements

##### 8.2.1.1.1	General

The minimum performance requirements are applicable to all FR1 operating bands defined in clause 5.2.

If same test is listed for different UE features/capabilities in Clauses 8.2.1.1.2, then this test shall apply for UEs which support all corresponding UE features/capabilities.

##### 8.2.1.1.2	Applicability of requirements for optional UE features

The performance requirements in Table 8.2.1.1.2-1 shall apply for UEs which support optional UE features only.

Table 8.2.1.1.2-1: Requirements applicability for optional UE features

| UE feature/capability [11] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| NR NTN access (nonTerrestrialNetwork-r17) | FR1 FDD | PDSCH | Clause 8.2.1.2.2.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4) |  |
| NR NTN scenario support (ntn-ScenarioSupport-r17) | FR1 FDD | PDSCH | Clause 8.2.1.2.2.1 (Test 1-1, Test 1-2, Test 1-3, Test 1-4) | The requirements apply only when ntn-ScenarioSupport-r17 is “ngso” or is not configured. |
| Increasing the number of HARQ processes (max-HARQ-ProcessNumber-r17) | FR1 FDD | PDSCH | Clause 8.2.1.2.2.1 (Test 1-3) |  |
| Disabled HARQ feedback for downlink transmission (harq-FeedbackDisabled-r17) | FR1 FDD | PDSCH | Clause 8.2.1.2.2.1 (Test 1-4) |  |
| Note: For UE supporting NR NTN access (nonTerrestrialNetwork-r17), the requirements in TS38.101-4 [6] also applies to UE according to applicability rules in TS38.101-4 [6] Clause 5.1, 6.1, 7.1 and 8.1 |  |  |  |  |

-------------------------------------------------------------End of change 3----------------------------------------------------------------

-------------------------------------------------------------Begin of change 4-------------------------------------------------------------

# 11	Demodulation performance requirements (Radiated requirements)

## 11.1	General

### 11.1.2	Applicability of minimum requirements

The radiatedminimum requirements specified in this specification shall be met in all applicable scenarios for FR2-NTN.

-------------------------------------------------------------End of change 4----------------------------------------------------------------