3GPP TSG-RAN WG4 Meeting #113	R4-2419338

Orlando, Florida, USA, 18th – 22nd November, 2024

Source:	BT, Ericsson

Title:	Further observations on the properties of MIMO channels in typical deployments

Agenda item:	7.14.1

Document for:	Information

Introduction

The Study on Spatial Channel Modelling for Demodulation Performance Requirements was approved at RAN#104 [1], and RAN4#112 and RAN4#112b approved way forward agreements [2] and [3] respectively.

In this contribution we provide observations of spatial characteristics of measured MIMO channels from typical deployments, focusing on the evolution of spatial preferences of channels in time. The purpose of this contribution is to support the group with identification of important properties of typical propagation environments to be reflected as close as reasonably possible in a spatial channel model. Other observations of basic properties of MIMO channels in sample typical deployments are provided in [4, 5].

Field measurements from typical deployments

Measurement setup and scope

Uplink sounding reference signals (SRS) were used for evaluation of respective MIMO channel between a typical commercial MIMO base station and a UE.  Measurements were conducted using a 40MHz channel in n78 band (3680 – 3720 MHz) in four locations A to D shown in Figure 1. Reported samples were collected in static positions during office hours, with some movement of people and vehicles in the coverage sector, yet not in the immediate vicinity of the measurement setup. The output of the measurements was reconstructed into channel gains per transmitter and receiver port pair (the H matrices), including all system gains and losses. The H matrices were post-processed, and in this contribution we report angles of arrival determined using MUSIC algorithm for four locations A to D.

Locations A and C can be characterised as having a dominant line-of-sight component, yet with several reflections from surrounding buildings. Locations B and D can be characterised as non-line-of-sight due to obstructions from surrounding buildings. However, the LoS or NLoS determination is indicative. The determination is based on analysis of power delay profiles from previous test campaigns utilising same locations. Sample typical PDP for downlink direction for Location A is provided in Figure 3 of [5].

Other useful analysis types for the purposes of SCM could be cluster angle distribution of one link, correspondence of AoA to clusters (same or different), existence of sub-clusters within a strong cluster which have similar angle and spread, among others. In this contribution though we focus on the temporal stability of principal AoAs.

Figure 1. Measurement point locations for the sample MIMO channels.

Angle of Arrival stability

Figures 2 and 3 illustrate the estimated AoAs over time. In this analysis, the focus was on exploring whether MIMO channels in typical field deployments have long-term stable spatial preferences. Presence of three dominant arrivals was assumed and a simple implementation of MUSIC algorithm was applied to measured MIMO channel matrices.

Figure 2. Three primary AoAs for SRS from UE in Locations A and C.

Figure 3. Three primary AoAs for SRS from UE in Location B and D.

Measurements demonstrate that there are multiple long-term stable AoAs observed in the MIMO channel between static receiver and transmitter. Observed fluctuations around observed trends across approx. 30sec of measurement duration are assumed to be in part due to the processing of data. Better estimation algorithms could be applied to the measured data, however this is outside of the scope of the contribution. Furthermore, the power of clusters is also relevant, since weak clusters would cause a large estimation error.  Finally, the spread of estimated trends for AoAs varies per location, as well as per assumed LoS and NLoS profile, as expected. For the purposes of SCM study, use of stationary UE or low speed UE could be beneficial to ensure spatial consistency of the model.

MIMO radio channels for stationary UE in typical deployments exhibit multiple long-term stable spatial directions of arrivals.

Conclusion

Observations and proposals for the SCM are summarised below.

- MIMO radio channels for stationary UE in typical deployments exhibit multiple long-term stable spatial directions of arrivals.

- Acknowledgements

The measurements and data processing for this contribution have been conducted by Ericsson Research UK and BT Research teams under UK DSIT REASON project and UKRI Future Leaders Fellowship (MR/Y020065/1).

References

- RP-241610, “3GPP Work Item Description: Study on spatial channel model for demodulation performance requirements”, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

- R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112, Aug 2024.

- R4-2416591, Way Forward for [112b][328] FS_NR_demod_SCM, RAN4#112b, Oct 2024.

- R4-2411557, “Scenarios and requirements for spatial channel modelling”, BT, RAN4#112, Aug 2024.

- R4-2415673, “Further observations from deployment and MU-MIMO use cases for SCM evaluation”, BT, Orange, RAN4#112b, Oct 2024.