3GPP TSG-RAN WG4 Meeting #113	R4-2419340

Orlando, USA, November 18 – 22, 2024

Source:	Nokia

Title:	HPUE justification in WID and interference profile

Agenda item:	7.16.1

Document for:	Discussion

Introduction

RAN4#112bis was the second meeting in RAN4 to discuss Rel-19 NR demodulation performance phase 5 Work Item (WI). Rel-19 NR demodulation performance phase 5 has two distinct topics which are treated independently, namely, UE Demodulation and BS Demodulation.

The way forward from RAN4#112bis discussion on BS Demodulation Performance for MMSE-IRC is documented in [1]. As shown in [1], there are several agreements made by companies. Nonetheless, several important issues are still open and need to be further discussed in RAN4#113.

In this contribution, we will discuss one of the open issues which seems to need a special attention as it might need a revision to the Work Item Description (WID) [2].

Discussion

Justification and Objective of the WI

In [2], the justification and objective of the WI are described. The justification is as follows:

# 3 Justification In Rel-19, to verify the UE and BS demodulation performance with advanced receiver and apply demodulation requirements to more scenarios of practical network, the following enhancements are identified: - The Rel-18 8Rx performance requirements (introduced under NR_ENDC_ RF_FR1_enh2 WI) do not cover scenarios of co-channel inter-cell and intra-cell inter-user interference. With 8 Rx, larger gain of MMSE-IRC is expected under interference scenarios, due to larger spatial domain freedom compared to 2/4 Rx. Therefore, there are benefits to define UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices under interference scenarios. - BS receiver with MMSE-IRC can efficiently suppress the uplink interference from UEs in neighbouring cells and thus improve uplink coverage and capability. In addition, the widely use of HPUE makes UL co-channel interference from the neighbouring cell UE severer than that in LTE system. There is necessity to introduce BS performance requirements for MMSE-IRC advanced receiver under inter-cell interference.

As shown above, the justification of this WI is the fact that the widely use of HPUE makes the UL inter-cell interference in NR to be more severe than in LTE. With such justification as above, the objective of this WI is defined as follows:

# 4 Objective ### 4.1 Objective of SI or Core part WI or Testing part WI ### 4.2 Objective of Performance part WI - Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline - Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions: - Receiver type: MMSE-IRC receiver - Scenarios: - FDD synchronous deployments - TDD synchronous deployments with aligned UL:DL configurations among cells - Slot-based transmission and aligned SCS among cells - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

As shown above, in the objective it is clearly stated that in defining the requirements, RAN4 could reuse LTE based interference profile in TR 36.884 [3] as a starting point, while other interference profiles are not precluded.

Open Issue from RAN4#112bis

The open issue from RAN4#112bis [1] that requires a special attention is as below:

Interference profile - On how to account for HPUE in existing LTE profiles: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - Proposals on how to account for HPUE in existing LTE profiles. - Proposal 1: Perform new system-level simulations with the inclusion of HPUEs - Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

In TR 36.884 [3], the values of the Dominant Interference Proportions (DIP) were obtained from system-level simulation. It is noted here that the system-level simulation in LTE in [3] did not consider HPUEs.

Ideally, as HPUEs is explicitly written in the justification of the WID [1], a new system-level simulation is needed. The new system-level simulation which takes into account the HPUEs will provide the proper DIP values for this WI. With such DIP values accounting HPUEs, the justification of the WI can be validated.

From previous discussions in RAN4, companies seem to be hesitant to conduct new system-level simulation to generate new DIP values accounting HPUEs to be used in link-level simulation to define the requirements. Hence, one proposal was to consider artificially increasing DIP/INR (Interference-to-Noise Ratio) with additional 3 or 7 dB.

The justification of the WI clearly states that HPUEs make the interference in NR to be more severe than in LTE.

New system-level simulation is needed to generate new DIP values by introducing HPUEs into the simulator.

Artificially increasing the DIP/INR to represent HPUEs needs a proper justification, which by itself requires simulation to support it. Furthermore, the proper simulation would be anyway a system-level simulation.

New system-level simulation shall be conducted by a sufficient number of companies to generate new DIP values accounting HPUEs to validate the justification of the WI.

If no new system-level simulation is conducted, RAN4 shall not artificially increasing the DIP/INR values from LTE without a proper justification.

If there is neither new system-level simulation nor a reasonable justification to add margin due to HPUEs, companies are invited to bring this topic to plenary to either remove HPUE from the WID or clearly state that LTE DIP values are to be reused with agreeable margin.

Conclusion

In this contribution, we provide our views on one particular open issue in Rel-19 NR Demodulation Performance Phase 5 UL BS MMSE-IRC, which might require a revision in the WID. The following Observations and Proposals were made:

Observation 1: The justification of the WI clearly states that HPUEs make the interference in NR to be more severe than in LTE.

Observation 2: New system-level simulation is needed to generate new DIP values by introducing HPUEs into the simulator.

Observation 3: Artificially increasing the DIP/INR to represent HPUEs needs a proper justification, which by itself requires simulation to support it. Furthermore, the proper simulation would be anyway a system-level simulation.

Proposal 1: New system-level simulation shall be conducted by a sufficient number of companies to generate new DIP values accounting HPUEs to validate the justification of the WI.

Proposal 2: If no new system-level simulation is conducted, RAN4 shall not artificially increasing the DIP/INR values from LTE without a proper justification.

Proposal 3: If there is neither new system-level simulation nor a reasonable justification to add margin due to HPUEs, companies are invited to bring this topic to plenary to either remove HPUE from the WID or clearly state that LTE DIP values are to be reused with agreeable margin.

- References

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, 3GPP RAN4#112bis, China Telcom, Hefei, China, October 14-18, 2024

- RP-241297, Revised WID on NR demodulation performance: Phase 5, 3GPP RAN#104, China Telecom, NTT DOCOMO, Shanghai, China, June 17-20, 2024

- 3GPP TR 36.884, Performance requirements of MMSE-IRC receiver for LTE BS, v13.1.0, 09-2016