3GPP TSG-RAN WG4 Meeting # 113 	    R4-2419362

Orlando, US, 18  – 22 November 2024

Agenda Item:	7.18.1

Source: 	Ericsson

Title:	General discussion on AI/ML for mobility

Document for:	Approval

Introduction

Starting from RAN4#112bis meeting, discussion on issues related to AI/ML for mobility has started in RAN4. Discussions on requirement and testability are presented in our companion papers and general aspects related to AI/ML for mobility are discussed in this paper.

Discussion

For AI/ML based mobility, to reduce the measurement overhead and measurement delay, prediction of measurements in time domain, frequency domain and spatial domain, and the gain these predictions bring to mobility is being evaluated by RAN2. Definition of different prediction types and the measurement model used by RAN2 are provided below for reference purposes.

Temporal prediction: Predict measurement of a cell for future time instances by using the measurements performed by UE on the same cell.

Spatial prediction: Predict a set of neighbor cell(s) by measuring a smaller set of cells.

Frequency prediction: Predict measurements of cells in one frequency based on the measurements performed by the UE on cells in another frequency.

Figure 1.  Measurement model as defined in TS 38.300 [2].

Studying the impact of different prediction types on RAN4 requirement is one of the objectives set by updated SID and the details of requirements will be discussed in the set agenda item for the discussion. In our view, it is important to establish a common understanding of the term “prediction” and “predicted measurement/event” to be used in RAN4 discussion before a detail analysis of impact on RAN4 requirements can be done.

The term “prediction” should refer to the process of using a trained AI/ML model for producing output(s). And the term “predicted measurement/event” should refer to the output of the AI/ML model. A high-level overview of prediction and predicted measurement/event is provided in Figure 2. Models for prediction of measurements for mobility will be deployed at the UE. Upon receiving request from the network, the UE will do some measurements and use them for prediction. Within this framework, the focus/scope of setting the requirement should not be on the details of the AI/ML model deployed to support mobility functionality rather the focus/scope of the setting requirement should be on the measurements performed by the UE before the prediction can start and the output of the model or predicted measurement/event.

Figure 2. Elaboration of prediction and predicted measurement/event.

Observation 1: Focus/scope of setting the requirement should not be on the details of the AI/ML model deployed to support mobility functionality.

Proposal 1: RAN4 to not define requirements for prediction as a process.

Another important aspect is related to the side condition for the measurements to be performed by the UE for prediction and the side condition for the predicted measurements. In RAN2 evaluations, measurements defined for the legacy L3-based mobility are used for prediction. For all those measurements, corresponding requirements and side-conditions under which the requirements are valid are well-defined in the existing specification. From RAN4 point of view, it is important to understand whether the existing side conditions are applicable when the legacy measurements are used for further predictions.

Impact of the error in the measurements on the predicted measurements/event is another important issue that needs to be evaluated. Due to the limited TU budget allocated to RAN4, it may not be feasible to do this type of evaluation in RAN4. According to the timeline agreed in RAN2 for the study item, RAN2 is expected to start evaluation for generalization capability of AI/ML model deployed to support mobility functionality in RAN2#129bis meeting [3]. It is therefore our proposal to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

Observation 2: Measurements defined for legacy L3-based mobility are used for prediction in RAN2 evaluations.

Observation 3: For the measurements used in RAN2 evaluation, requirements and the side-conditions are well defined in the existing RAN4 specifications.

Observation 4: Impact of the error in the measurements on the predicted measurements/event is important issue that needs to be evaluated.

Proposal 2: Study whether the existing side conditions are still applicable when the legacy measurements are used for further predictions.

Proposal 3: RAN4 to provide input to RAN2 to at least consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

Proposal 4: Discuss and identify RAN4 specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility.

Summary

In this contribution, general issues that have an impact on different prediction types being studied are highlighted. The discussions presented in this paper are summarized in terms of observations and proposals below.

Observation 1: Focus/scope of setting the requirement should not be on the details of the AI/ML model deployed to support mobility functionality

Proposal 1: RAN4 to not define requirements for prediction as a process.

Observation 2: Measurements defined for legacy L3-based mobility are used for prediction in RAN2 evaluations.

Observation 3: For the measurements used in RAN2 evaluation, requirements and the side-conditions are well defined in the existing RAN4 specifications.

Observation 4: Impact of the error in the measurements on the predicted measurements/event is important issue that needs to be evaluated.

Proposal 2: Study whether the existing side conditions are still applicable when the legacy measurements are used for further predictions.

Proposal 3: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

Proposal 4: Discuss and identify RAN4 specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility.

References

- R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).

- TS38.300, Stage 2: NR and NG-RAN Overall Description, 3GPP.

- R2-2404711, Discussion on work plan of AI mobility, OPPO, Nokia, MediaTek.