c3GPP TSG-RAN WG4 Meeting # 113 	    R4-2419364

Orlando, US, 18  – 22 November 2024

Agenda Item:	7.18.3

Source: 	Ericsson

Title:	On testability issues related to AI/ML for mobility

Document for:	Approval

Introduction

RAN4#112bis discussed the testability aspects related to AI/ML for mobility. Issues discussed and the agreements reached are listed in the WF document [1].

Discussions

In the last meeting, 2 intra-cell scenarios and 1 inter-cell scenario were agreed to be prioritized for the evaluation of impact on RAN4 requirement due to AI/ML based mobility. The table containing the prioritized scenarios is provided below for reference [1].

Agreement from RAN4#112bis:

RAN4 to start the discussion for scenario 2, 3, 4. Further clarification can be discussed for the scenarios.

| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| --- | --- | --- | --- | --- |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

Testing for intra-cell scenarios

Intra-cell scenarios characterize deployment of AI/ML model to predict RRM measurements in temporal domain, where the measurements performed within observation window (OW) are used by the UE to perdict measurements valid for time instances inside the prediction window (PW) for the same cell as shown in Figure 1. Like test setup for legacy mobility, test setup for validation of requirements for intra-cell predictions should also be based on two cells. The two-cell setup is needed to take interference coming from the cell other than the measured cell in to account during testing. The aim of testing should therefore be to validate that the measurements reported by the UE for a cell meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

To validate the measurements predicted by UE for the time instances inside the PW, ground truth of predicted values is required. This issue in our view is generic and is not limited to only testing for intra-cell scenarios. RAN4 should therefore take this issue into account and discuss considerations for deriving the ground truth value for validation of the accuracy of predicted measurements in general.

Figure 1. Measurement timeline for intra-cell temporal prediction scenario.

Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup.

Proposal 2: The aim of testing for intra-cell temporal domain prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

Proposal 3: RAN4 to discuss considerations for deriving the ground truth value for validation of the accuracy of predicted measurements.

Testing for inter-cell (inter-frequency) scenario

Scenario 3 is another high priority scenario agreed to be investigated to identify the impact on RAN4 requirements [1]. Scenario 3 characterizes deployment of AI/ML model to predict RRM measurement of a cell in one carrier frequency by exploiting the measurements performed by UE on cell in another frequency in FR1. The aim of testing should therefore be to validate that the measurements reported by the UE

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

This scenario is similar to the inter-frequency measurement scenario supported for non-AI/ML mobility, where the UE is configured to perform and report measurement for two different cells that are in two different carrier frequencies. Test setup defined to validate requirements for inter-frequency measurements can therefore be re-used for testing AI/ML based mobility for scenario 3. While testing for AI/ML based mobility, cell 1 can be considered the cell for measurement and cell 2 can be considered the target cell for which the UE predicts the measurement for.

Proposal 4: The aim of testing for inter-cell (inter-frequency) scenario should be to validate that the measurements reported by the UE:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Proposal 5: Existing test setup defined to validate requirements for inter-frequency measurements can be re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

- Cell 1 in the test can be considered the cell for measurement.

- Cell 2 in the test can be considered the target cell for measurement prediction.

Summary

In this contribution Ericsson’s view on testability issues related to AI/ML for mobility are highlighted and discussed. The proposals below summarize the discussion presented in this contribution.

Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup.

Proposal 2: The aim of testing for intra-cell temporal domain prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

- requirement for the measurements performed inside the OW,

- requirement for the measurements predicted for the time instances inside the PW.

Proposal 3: RAN4 to discuss considerations for deriving the ground truth value for validation of the accuracy of predicted measurements.

Proposal 4: The aim of testing for inter-cell (inter-frequency) scenario should be to validate that the measurements reported by the UE:

- meet the requirements for measured cell, and

- meet the requirements for the target cell.

Proposal 5: Existing test setup defined to validate requirements for inter-frequency measurements can be re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

- Cell 1 in the test can be considered the cell for measurement.

- Cell 2 in the test can be considered the target cell for measurement prediction.

References

- R4-2416855, WF on AI/ML Mobility, Moderator (Nokia).