3GPP TSG-RAN WG4 Meeting # 113							R4-2419367

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.21.3.2

Source: 	OPPO

Title: 	On device RF requirements

Document for:	Approval

- Introduction

In RAN4#112-bis meeting, the device RF requirements have been fully discussed. Many of the requirements have been left as TBD. It might be more appropriate to further discuss the requirements and if in the end no consensus is reached, then we can leave the requirement discussion further in the WI phase.

- Discussion

In the WF [1] for A-IoT RF requirements,

| A-IoT device TX (D2R) RF Requirement |  | Potentially needed or not |  |  |
| --- | --- | --- | --- | --- |
|  |  | Device 1 | Device 2a | Device 2b |
| Transmit output power | Maximum output power /output power | NO for maximum output power YES for backscattering power or power backscattering loss | NO for maximum output power YES for backscattering power or power backscattering loss | YES |
| Output power dynamic | Transmit OFF power | No | NO | YES, as RAN4#112 agreed |
|  | Transmit ON/OFF time mask | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
|  | Minimum output power | N/A | N/A | YES |
|  | Power control | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Transmit signal quality | Frequency error | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
|  | Modulation quality requirements | TBD | TBD | TBD |
|  | In band emissions | NO, as RAN4#112 agreed | TBD | TBD. |
|  | Carrier leakage | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
| Output RF spectrum emissions | Occupied bandwidth | YES | YES | YES |
|  | SEM | TBD | TBD | TBD |
|  | ACLR | TBD | TBD | TBD |
|  | Spurious emissions | TBD | TBD | TBD |
| Unwanted emissions |  | TBD | TBD | TBD |
| Transmit intermodulation |  | NO | NO | TBD |

For the frequency error of device 2a, we believe it is needed since the frequency accuracy is crucial for the time and frequency sync.

For the in-band emission, we believe it should be together to be considered with co-existence study since currently the flat power difference is used for co-existence simulation while this should be considered as an in-band emission for both device type 2a and 2b.

For SEM, it is used to limit the PA non-linearity and for device 1 and 2a, we believe it is not needed since only reflection applies. For device 2b, we believe it is needed.

For ACLR we believe it is not needed. In the co-existence simulation, the IBE is used instead of ACLR since the channel bandwidth of the A-IoT devices is too small and the IBE plays more important role in the co-existence simulation.

For spurious emission, it is always the regulation requirement and hence we believe it applies for all three types of devices.

With the above discussion, we propose the requirement applicability as below:

Proposal 1: To agree on the following table for TX requirements:

Table 1 Summary of other TX requirements’

|  | Device Type 1 | Device Type 2a | Device Type 2b |
| --- | --- | --- | --- |
| Frequency error | (Agreed) | needed | (Agreed) |
| In-band emission | Together with co-existence study | Together with co-existence study | Together with co-existence study |
| SEM | Not needed | Not needed | Needed |
| ACLR | Not needed | Not needed | Not needed |
| Spurious emission | Needed | Needed | Needed |

For the RX requirements, below table is captured from WF [1]:

| A-IoT device- RX Requirement | Potentially needed or not |  |  |
| --- | --- | --- | --- |
|  | Device 1 | Device 2a | Device 2b |
| Reference sensitivity | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Maximum input power | TBD | TBD | TBD |
| ACS | NO | NO | Depend on co-existence study |
| ACSC | NO | NO | NO |
| In-band blocking | NO | NO | TBD |
| Out-of-band blocking | NO | NO | TBD |
| Receiver intermodulation | NO | NO | TBD |
| Rx spurious emission | NO | NO | TBD |
| Spurious response | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
| Interference rejection | TBD | TBD | TBD |

For the maximum input power, it defines the RX power dynamic range hence it is needed.

For the in-band and out-if-band blocking, it is based on the legacy UE working environment. In such case, for device 2b, the blocking requirement is needed to prevent the receiver to be blocked.

For the RX IMD and RX spurious, they are needed for device 2b as similar to legacy IoT devices.

Proposal 2: To agree on the following table for RX requirements:

| Maximum input power | Needed for 2b |
| --- | --- |
| Receiver intermodulation | Needed for 2b |
| Rx spurious emission | Needed for 2b |
| In-band blocking | Needed for 2b |
| Out-of-band blocking | Needed for 2b |

3	Conclusions

In this paper, we give further discussion on device RF requirement the observations and proposals are as below:

Proposal 1: To agree on the following table for TX requirements:

Table 1 Summary of other TX requirements’

|  | Device Type 1 | Device Type 2a | Device Type 2b |
| --- | --- | --- | --- |
| Frequency error | (Agreed) | needed | (Agreed) |
| In-band emission | Together with co-existence study | Together with co-existence study | Together with co-existence study |
| SEM | Not needed | Not needed | Needed |
| ACLR | Not needed | Not needed | Not needed |
| Spurious emission | Needed | Needed | Needed |

Proposal 2: To agree on the following table for RX requirements:

| Maximum input power | Needed for 2b |
| --- | --- |
| Receiver intermodulation | Needed for 2b |
| Rx spurious emission | Needed for 2b |
| In-band blocking | Needed for 2b |
| Out-of-band blocking | Needed for 2b |

4 References

[1] R4-2417088	WF on RF requirement impact of A-IoT, Huawei