3GPP TSG-RAN WG4 Meeting #113							R4-2419371

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.5.2.2

Source: 	OPPO

Title: 	On UE capability and behavior for type 4a and 4b

Document for:	Approval

- Introduction

In RAN4#112-bis meeting, the UE capability and behaviour for type 4a and 4b has been fully discussed and a WF[1] has been agreed.

Discussion

The reference UE architecture topic has been discussed during the RAN4#112-bis meeting and the WF agreement is captured as below.

< Issue 2-1: How to differentiate UE Capability Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC >

Agreement:

- Legacy capabilities, such as MIMO-CapabilityDL-r10 on E-UTRA carriers

< Issue 2-2: UE Capability Type 4a(6Rx/UE) for NR-CA >

Agreement:

- Do not consider Type 4a for NR-CA

< Issue 2-3: BS Signaling to switch between Type 4a/4b and Type 1 4L/CC(collocated) >

Way Forward:

- Continue further discussion on BS signalling in the next meeting.

< Issue 2-4: BS Signaling to switch between Type 4a/4b and Type 1 6L/8L/CC(collocated) >

Way Forward:

- Continue further discussion on BS signalling in the next meeting.

< Issue 2-5: Switch between Type 4a/4b and single CC operation >

Agreement:

- No new NW signaling needed from Type 4 to single carrier fallback.

< Issue 2-6: UE behavior during initial connection between Type 1, Type 2 and Type 4a/4b >

Way Forward:

- Continue further discussion on BS signalling in the next meeting.

During the RAN4#112 meeting, there is another WF[2] that has captured the UE capabilities and is shown as below:

< Issue 2-3-1:  New UE Capability for Type 4a(6Rx/UE) and 4b(8Rx/UE) for EN-DC/NR-CA >

[Agreement:]

- Define one capability for NR CA and one UE capability for EN-DC

- Differentiate Type 4a and 4b by UE capability for EN-DC.

- Specific UE Capabilities’ design depends on RAN2.

So consider all the agreements that have been reached, we don’t need to further differentiate type 4a and 4b for UE capability. Since the type 4 NR CA capability provides the support or not for type 4b NR-CA capability. And the type 4 EN-DC capability + MIMO-CapabilityDL-r10 can differentiate type 4a EN-DC and type 4b EN-DC.

Proposal 1: Type 4 NR CA capability provides the support or not for type 4b NR-CA capability and there is no type 4a NR-CA scenario.

Proposal 2: Type 4 EN-DC capability + MIMO-CapabilityDL-r10 to differentiate type 4a EN-DC and type 4b EN-DC capability.

Besides the UE capability report, still the BS signalling is discussed and currently there is no agreement yet since the UE capability discussion has not been finished yet. With the above discussion and proposal, we further discuss the BS signalling issue.

Look back to type 2 design, the NW signalling is used to configure the UE working at type 2 non-collocated and it is configured in the CellGroupConfig IE. The detail IE description is captured as below. Furthermore, the BS signalling only apply when the maxMIMO-layer is set to 2 while when the maxMIMO-layer is set to 4, there is no requirement in such case. The reason is that without the network signalling, UE doesn’t know if it is in collocated or non-collocated scenario.

Observation 1: Without network signalling, UE doesn’t know if it is in collocated or non-collocated scenario for type 2.

For both NR-CA and EN-DC, if larger maxMIMO-layer numbers are set, say 6 or 8 then the UE should work under collocated scenario. If smaller maxMIMO-layer numbers are set, say 2 then the legacy Rel-18 NW signalling should apply. The case that needs to be further clarified is when maxMIMO-layer = 4.

Observation 2: For maxMIMO-layer =6 or 8, the UE should work under collocated scenario.

Observation 3: For maxMIMO-layer= 2, the Rel-18 legacy NW signalling should apply.

– CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels

with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the

configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

-- ASN1START

-- TAG-CELLGROUPCONFIG-START

-- Configuration of one Cell-Group:

CellGroupConfig ::= SEQUENCE {

nonCollocatedTypeNR-CA-r18 ENUMERATED { true } OPTIONAL, -- Need R

uplinkTxSwitchingMoreBands-r18 SetupRelease { UplinkTxSwitchingMoreBands-r18 } OPTIONAL -- Need M

]]

When maxMIMO-layer = 4, UE can work under non-collocated and collocated scenario with 4 layers. In this case, similar to type 2 UE, a further NW signalling is needed to differentiate the collocated and non-collocated deployment as well as the requirement.

Proposal 3: A NW signalling is needed when maxMIMO-layer=4.

For NR-CA, only type 4b is applied and hence the NW signalling can be simple as the type 2 NW signalling. However, as it is agreed to leave RAN2 to further define the signalling, still we need to tell RAN2 that for NR-CA the NW signalling only needs to differentiate type 4b non-collocated and collocated deployment.

Proposal 4: For NR-CA, the NW signalling only needs to differentiate type 4b non-collocated and collocated deployment.

For EN-DC, both type 4a and type 4b applied. However, the NW signalling still only needs to differentiate type 4 non-collocated and collocated deployment and leave the type 4a/4b to be further decided by the maxMIMO-layers.

Proposal 5: For EN-DC, the NW signalling only needs to differentiate type 4 non-collocated and collocated deployment.

3	Conclusions

In this paper, we give further discussion on the UE reference architecture, the observations and proposals are as below:

Observation 1: Without network signalling, UE doesn’t know if it is in collocated or non-collocated scenario for type 2.

Observation 2: For maxMIMO-layer =6 or 8, the UE should work under collocated scenario.

Observation 3: For maxMIMO-layer= 2, the Rel-18 legacy NW signalling should apply.

Proposal 1: Type 4 NR CA capability provides the support or not for type 4b NR-CA capability and there is no type 4a NR-CA scenario.

Proposal 2: Type 4 EN-DC capability + MIMO-CapabilityDL-r10 to differentiate type 4a EN-DC and type 4b EN-DC capability.

Proposal 3: A NW signalling is needed when maxMIMO-layer=4.

Proposal 4: For NR-CA, the NW signalling only needs to differentiate type 4b non-collocated and collocated deployment.

Proposal 5: For EN-DC, the NW signalling only needs to differentiate type 4 non-collocated and collocated deployment.

4 References

[1] R4-2417115 	WF on rest issues for intra-band non-collocated EN-DC/NR-CA, KDDI

[2] R4-2414453 	WF on Rel-19 non-collocated scenario, KDDI