3GPP RAN WG4 Meeting #113																	R4-2419379

Orlando, USA, 18th – 22nd November 2024

Agenda item:	7.8.3.1

Source:	Apple, Globalstar

Title:	Asymmetric band combinations with the 3MHz channel bandwidth

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. And since that time the smallest channel bandwidth was 5MHz, all NTN bands have had only 5MHz as the smallest bandwidth.

After the RAN#104 meeting a new WI was agreed [2], one of the aims of which is to enable the 3MHz channel for the NTN bands. Based on the WF document from the previous RAN4#112 meeting [3] an agreement was made to enable asymmetric bandwidth combinations with UL 3MHz and larger DL channel bandwidths. In this discussion paper we suggest further clarifications regarding potential asymmetric band combinations.

# 2	Asymmetric band combinations

During the previous RAN4#112 meeting an agreement was made to enable asymmetric combinations with the 3MHz UL channel. However, the agreement captured in [3] is somewhat ambiguous as it is not clear whether it captures the 5MHz DL channel or not.

******************** agreement from RAN4#112 in R4-2413528 (begin) ********************

  - Introduce 3 MHz channel bandwidth for UL in n254 in Table 5.3.6-1 as 3MHz UL + X MHz DL

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) | Asymmetric channel bandwidth combination set |
| --- | --- | --- | --- |
| n254 | 3 | 10,15 | 1 |
| NOTE 1: The assignment of the paired UL and DL channels are subject to a TX-RX separation as specified in clause 5.4.4. NOTE 2: As indicated in TS38.306 [11], it is mandatory for UEs to support asymmetric channel BCS0 if there is an asymmetric BCS0 defined for the band. |  |  |  |

******************** agreement from RAN4#112 in R4-2413528 (end) ********************

For instance, the original definition of band n254 had the 5MHz channel as the smallest bandwidth with 5+10MHz and 5+10MHz asymmetric band combinations. So, using band n254 as an example, with introduction of the 3MHz channel, the following band asymmetric band combinations can be considered: 3+5, 3+10, and 3+15MHz.

Proposal:	Asymmetric band combinations with the 3MHz UL channel include the 5MHz DL and larger (applicable) DL channel bandwidths.

# 3	Conclusions

In this discussion paper we suggested clarifying an agreement from the previous RAN4#112 meeting regarding asymmetric band combinations with 3MHz UL and XMHz DL channels. To our technical understanding "X" corresponds to 5MHz and larger (applicable) DL channel bandwidths.

Proposal:	Asymmetric band combinations with the 3MHz UL channel include 5MHz DL and larger applicable DL channel bandwidths.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

- R4-2413528, "[112][312] NR_IoT_NTN_less_than_5MHz_UERF ", Xiaomi