3GPP TSG-RAN4 Meeting #113	R4-2419391

# Orlando, United States, 18th - 22nd November, 2024

Agenda item:	7.17.4.2

Source:	Keysight Technologies UK Ltd

Title:	On CDL Test Setup Requirements FR2 AI/ML Beam Management

Document for:	Approval

# Introduction

This contribution addresses test system and testability considerations for AI/ML beam management use cases.

# Discussion

Not much progress has been made in terms of channel models and test systems in the last few meetings for beam management use cases [2], [3], [4]. Regarding the test setup requirements, the following agreements and actions were noted in the last WF.

- Issue 2-9: Test setup requirements Agreement: - For CDL or simplified CDL channel model, RAN4 to study the relationship between AoD range [and granularity], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For TDL channel model, RAN4 to study the relationship between [dynamic power range], number of beams and number of probes. - Set A and B should be generated in the chamber for ground-truth extraction. - For single AoA, FSS how to derive Set A

This contribution intends to provide additional views on test setup requirements and is taking into account discussions from [5], [6], [7]. These contributions also provide an extensive overview of FR2 OTA systems, CDL vs TDL channel models, etc.

# Test Setup Requirements: CDL Models

Based on our experience with developing beam management test cases, we believe a relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes with a sample system visualized in Figure 1, is suitable to meet the following assumptions for a single TRP or two TRPs with close angular proximity:

Emulation of Tx-beam powers and RX-PAS for 4 Tx-beams (dual polarization) [6] or 8 Tx-beams (single polarization) for CDL-A, B, C, D and E UMi/UMa/InO scenarios.

  - The emulation of Tx-beams with accurate RX-PAS AoA profiles to take into account the strongest beams at the center of the test zone.

- Additionally, to emulate set B Tx beams beyond 4 [up to 16] while exercising the AI/ML algorithms, power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics

- More importantly, to emulate set A Tx beams [up to 128], power offsets can be applied to the Tx-beams with the closest match of the 4 available RX-PAS profiles and exact power offset based on the 3D beam and CDL model characteristics

- The RX-PAS AoA profile for beams emulated with power offsets cannot be guaranteed

- Support AoDs/ZoDs with a full range (0° to 360°)

- Support AoAs/ZoAs with a limited range of 110° in azimuth and 25° in elevation. for a single UE orientation

- For accurate performance assessment of all or top-K Set A beams, testing will not require exercising the AI/ML algorithms on the UE-side and different UE orientations and/or sequential testing would be required when

  - More than 4 beams need to be assessed

  - N beams (with N≤4) have beam peaks with relative AoA/ZoA differences larger than the AoA/ZoA ranges supported by the system

Figure 1: Sample FR2 test system supporting AI/ML BM testing. Probes in blue correspond to the FR2 MIMO OTA probes.

Observation 1: A relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes is suitable to emulate 4 dual-polarized/8 single-polarized Tx-beam RX-PAS profiles, set B Tx beams up to [16] with power offsets, and set A Tx beams up to [192] with power offsets.

Proposal 1: UE/chipset/infra vendors to provide feedback on whether modelling 4 dual-polarized/8 single-polarized RX-PAS accurately with the remaining beams modelled with power offsets is sufficient

Observation 2: The extension of an FR2 MIMO OTA system to 8 probes is suitable to support unlimited range of AoDs/ZoDs and a range of AoAs/ZoAs of 110°/25°.

The minimum spacing (granularity) between typical mm-wave probes was determined to be ~3-4° [10] and the minimum probe spacing for the FR2 MIMO MPAC system [11], [12] is 10°.

Some of these testing aspects are already supported by Keysight’s S8708A Advanced Performance Test (APT) toolset [6].

Observation 3: Various beam management performance testing can readily be supported with a commercially available test system, i.e., complexity and time-to-market concerns should not be an issue.

Proposal 2: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system with a range of AoAs/ZoAs of 110°/25° as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity.

# Conclusion

The following observations and conclusions were made in this contribution.

Observation 1: A relatively simple and straightforward extension of an FR2 MIMO OTA system to 8 probes is suitable to emulate 4 dual-polarized/8 single-polarized Tx-beam RX-PAS profiles, set B Tx beams up to [16] with power offsets, and set A Tx beams up to [192] with power offsets.

Observation 2: The extension of an FR2 MIMO OTA system to 8 probes is suitable to support unlimited range of AoDs/ZoDs and a range of AoAs/ZoAs of 110°/25°.

Observation 3: Various beam management performance testing can readily be supported with a commercially available test system, i.e., complexity and time-to-market concerns should not be an issue.

Proposal 1: UE/chipset/infra vendors to provide feedback on whether modelling 4 dual-polarized/8 single-polarized RX-PAS accurately with the remaining beams modelled with power offsets is sufficient

Proposal 2: Consider an 8-probe OTA system with 2 additional probes when compared to the FR2 MIMO OTA system with a range of AoAs/ZoAs of 110°/25° as the starting point for AI/ML Beam management testing for 1 TRP and/or 2 TRPs with close angular proximity.

# References

- RP-240774, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #103, March 2024

- R4-2410570, WF on AI/ML, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #111, May 2024

- R4-2414323, WF on AI/ML, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112, August 2024

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated, 3GPP TSG-RAN WG4 Meeting #112bis, October 2024

- R4-2409770, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 Meeting #111, May 2024

- R4-2411254, OTA Test System/Testability Considerations for FR2 AI/ML Beam Management Use Cases, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112, August 2024

- R4-2416477, On Testability Aspects for FR2 AI/ML Beam Management, Keysight Technologies, 3GPP TSG-RAN4 Meeting #112bis, October 2024

- R4-2411587, Discussion on test setup for AI/ML based beam management, Rohde & Schwarz, 3GPP TSG-RAN4 Meeting #112, August 2024

- R4-2411279, Study on CDL channel model for AI/ML beam management in FR2, Anritsu Corporation, 3GPP TSG-RAN4 #112, August 2024

- R4-1904586, MPAC setup -Feasibility study, MVG Industries, 3GPP TSG-RAN WG4 #90-bis, April 2019

- TR 38.827, Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

- TS 38.151, Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) performance requirements