3GPP TSG-RAN WG4 Meeting#113							         		R4-2419411

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	Architecture and assumptions for 3Tx MPR

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.19.2	UE RF requirements	[NR_MIMO_Ph5-Core]

Document for:	Approval

# 1	Background

In RAN#112bis, a way forward [1] captured several agreements on 3Tx architecture and MPR assumptions. In this contribution, based on our previous work on 1Tx, 2Tx and 4Tx MPR and looking at the current MPR requirements, we discuss the PA architecture and anticipated impact on MPR.

# Discussion

## 2.1 Way forward inputs

The way forward [1] captured agreements on 3Tx architecture and MPR in italics below.

<Agreement>: PC for 3Tx requirement

- PC for 3Tx requirement: PC3, PC2, FFS on PC1.5

  - FFS on UE type, i.e., FWA and/or handheld UE

  - FFS on support of full power mode 0

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

- Full power mode 0 apply to PC2 and PC3 only.

  - FFS on for which UE types it should apply.

<Way forward>: FFS on New capability for power class per band per declared UL power mode for 3Tx

<Agreement>: Assumption for power splitting between antenna ports: Equal power split among non-zero ports according to RAN1 spec

<Agreement>: PA configurations: For MPR evaluation, the assumptions of PA architecture are:

- For PC3: 3x23dBm

- For PC2: 3x23dBm, 3x26dBm is not precluded and interested company can provide the evaluation

<Agreement>: Different MPR for different UE type: Define separate MPR requirements for handheld and FWA UE

<Way forward>: MPR derivation method for 3Tx: Companies are encouraged to evaluate MPR based on existing MPR requirements, e.g., PC, 2/4Tx, UE type, considering 3PA configuration agreed above

- Option 1: Reuse legacy MPR requirements for each power class (OPPO, Huawei, Nokia)

- Option 2: Further evaluate with 3PA configuration (Apple)

<Way forward >: FFS on Supported MIMO layer for ULFPTx mode 0

The way forward [1] provides conclusions on some aspects of PA architecture, but some of the conclusions do not seem very well aligned with how 2Tx and 4Tx were assessed.

## 2.2 Architecture assumptions

To assess which PA architectures are valid for different power classes and 3Tx, Table 1 below provides comparisons for the 3Tx architecture and MPR aspects versus the already existing 1Tx, 2Tx and 4Tx cases. The main assumption is that the architecture uses existing PA sizing of 20dBm (NRU), 23dBm and 26dBm.

Table 1: Architecture and MPR aspects per power class versus number of Tx.

| # Tx | power class | PC3 | PC2 | PC1.5 |
| --- | --- | --- | --- | --- |
|  | Pcmax [dBm] | 23 | 26 | 29 |
| 1 | min architecture | 23dBm | 26dBm | N/A |
| 2 | /PA [dBm] | 20 | 23 | 26 |
|  | min PA for full power mode 0 | 23+20dBm wo TxD 2x20dBm with 2Tx TxD | 26+23dBm wo TxD 2x23dBm with 2Tx TxD | 2x26dBm w/wo 2Tx TxD |
|  | specific MPR requirement | No same as 1Tx | Yes 10dB iso only | Yes 10&20dB iso |
| 3 | /PA [dBm] | 18.2 | 21.2 | 24.2 |
|  | min PA for full power mode 0 | one 23dBm PA wo TxD 2x20dBm w TxD | one 26dBm PA wo TxD 2x23dBm with w TxD | 2x26dBm with 2Tx TxD |
|  | min architecture | 23+2x20dBm wo TxD 3x20dBm with 2Tx TxD | 3x23dBm w/wo 2Tx TxD | 3x26dBm w/wo 2Tx TxD |
|  | initial BO/PA [dB] | 1.8dB |  |  |
|  | specific MPR requirement? | No same as 1Tx or better if yes? | Yes: smartphone=FWA => reduced MPR vs 2Tx | Yes: 10&20dB iso => reduced MPR vs 2Tx |
| 4 | min architecture | N/A |  | 4x23dBm |
|  | specific MPR requirement |  |  | Yes for FWA 10&20dB iso |

## 2.2 MPR RIDM aspects for 3Tx

Separately, to assess the impact of reverse IMD for 3Tx vs 2Tx, Table 2 collects the QPSK MPR for the different power classes, number of Tx, and allocation types.

Table 2: MPR table for QPSK modulation versus allocation, power class and number of Tx

| Modulation | power class | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| --- | --- | --- | --- | --- |
| DFT-s-OFDM QPSK MPR (dB) | PC2 1Tx | ≤ 3.5 | ≤ 1 | 0 |
|  | PC2 2Tx 10dB | ≤ 3.5 | ≤ 2 | 0.5 |
|  | PC1.5 2Tx 10dB | ≤ 6.5 | ≤ 2.5 | ≤ 0.5 |
|  | PC1.5 2Tx 20dB | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | PC1.5 4Tx 10dB | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | PC1.5 4Tx 20dB | ≤ 6.5 | ≤ 2 | ≤ 0 |
| CP-OFDM QPSK MPR (dB) | PC2 1Tx | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | PC2 2Tx | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | PC1.5 2Tx 10dB | ≤ 6.5 | ≤ 4.5 | ≤ 2 |
|  | PC1.5 2Tx 20dB | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | PC1.5 4Tx 10dB | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | PC1.5 4Tx 20dB | ≤ 6.5 | ≤ 4 | ≤ 1.5 |

For PC2, the difference between 1Tx PC2 and 2Tx PC2 MPR is between 1dB and 0.5dB:

=> for 3Tx, if we assume now that two PAs couples into one instead of one into one for 2Tx, the delta will be 1 to 1.8dB

For 3Tx cases, the PAs will benefit from at least 1.8dB of intrinsic back-off which corresponds to at least a 4dB gain on the interference level related to RIMD:

=> The 0.5-1dB impact for 2Tx can be assumed to be 0.4 to 0.8dB for 3Tx with 1.8dB intrinsic back-off.

Observation: The RIMD impact for 3Tx is about 0.8dB versus 1dB for 2Tx

## 2.3 Observations and proposals based on architecture and RIMD input

Observation for PC3:

- Compared to the 3Tx way forward, the 2Tx case had no assumption on the PA architecture since it was possible to meet the 1Tx MPR with PAs that would have less than 23dBm power capability.

- For the 3Tx case, it is also possible to meet the 1Tx MPR with at least two PAs that have less than 23dBm power capability.

Proposal for 3Tx PC3:

- If 1Tx MPR is reused as with 2Tx there is no need to make any assessment for MPR or any assumption on the PA architecture.

- If 3x20dBm or 23dBm+2x20dBm PAs are used, each PA will benefit from an intrinsic of at least 1.8dB back-off. Thus if we assume 0.8dB RIMD impact for 3Tx versus 0.5dB impact for 2Tx, any 1Tx MPR case that is less than 1.5dB MPR could use 0dB MPR.

- If 3x23dBm PAs are used, each PA will benefit from an intrinsic 4.8dB back-off. Thus, if we assume 0.8dB RIMD impact for 3Tx versus 0.5dB impact for 2Tx, any 1Tx MPR case that is less than 4.5dB MPR could use 0dB MPR.

- In our view, we do not see why 3x23dBm PA architecture should be mandated and would prefer to either reuse 1Tx PC3 MPR or assume 3x20dBm or 23+2x20dBm PAs.

Observation for PC2:

- Whether 2Tx TxD with two PAs or a “full” PA is used to achieve full power mode 0, the minimum architecture is with 3x23dBm PAs which benefits of an intrinsic 1.8dB back-off.

- As with 2Tx, we are not sure a 20dB isolation needs to be assessed for FWA since for the large FWA form factor, a 4x23dBm PC1.5 can already be implemented.

Proposal for 3Tx PC2:

- For 3Tx, only 10dB isolation is considered for both smartphone and FWA.

- Assuming 3x23dBm PAs, each PA will benefit from an intrinsic 1.8dB back-off. Thus, if we assume 0.8dB RIMD impact for 3Tx versus 0.5dB impact for 2Tx, any 2Tx PC2 MPR case that is less than 1.5dB MPR could use 0dB MPR.

Observation for PC1.5:

- Only 2Tx TxD can be used to achieve full power mode 0, the minimum architecture is with 3x26dBm PAs benefits of an intrinsic 1.8dB back-off.

- As with 2Tx PC2, we are not sure a 20dB isolation needs to be assessed for FWA since for large FWA form factor, a 4x23dBm PC1.5 can already be implemented.

Proposal for 3Tx PC1.5:

- For 3Tx, only 10dB isolation is considered for both smartphone and FWA.

- Assuming 3x26dBm PAs, each PA will benefit from an intrinsic 1.8dB back-off. Thus if we assume 0.8dB RIMD impact for 3Tx versus 0.5dB impact for 2Tx, any 1Tx MPR case that is less than 1.5dB MPR could use 0dB MPR.

Observation on full power mode 0:

- All power classes can achieve full power mode 0

- For PC3 and PC2 it can be achieved with either:

  - Having at least 1 PA with full power capability

  - Having at least two PAs with “half” power capability and TxD

- For PC1.5 it can be achieved with two 26dBm PAs and TxD

Proposal on full power mode 0:

- Full power mode 0 can be achieved for PC3 assuming either:

  - At least one 23dBm PA

  - At least two 20dBm PAs using 2Tx TxD.

- Full power mode 0 can be achieved for PC2 assuming either:

  - At least one 26dBm PA

  - 3x23dBm PAs and two of them using 2Tx TxD.

- Full power mode 0 can be achieved for PC1.5 assuming 3x26dBm PAs and two of them using 2Tx TxD.

Observation: Based on the above proposals we do not see the need for a new capability for power class per band per declared UL power mode for 3Tx.

# Conclusion

In this contribution, based on our previous work on 1Tx, 2Tx and 4Tx MPR, and looking at the current MPR requirements, we make the following proposals and observation.

Proposal for 3Tx PC3:

- If 1Tx MPR is reused as with 2Tx there is no need to make any assessment for MPR or any assumption on the PA architecture.

- If 3x20dBm or 23dBm+2x20dBm PAs are used, each PA will benefit from an intrinsic of at least 1.8dB back-off. Thus if we assume 0.8dB RIMD impact for 3Tx versus 0.5dB impact for 2Tx, any 1Tx MPR case that is less than 1.5dB MPR could use 0dB MPR.

- If 3x23dBm PAs are used, each PA will benefit from an intrinsic 4.8dB back-off. Thus, if we assume 0.8dB RIMD impact for 3Tx versus 0.5dB impact for 2Tx, any 1Tx MPR case that is less than 4.5dB MPR could use 0dB MPR.

- In our view, we do not see why 3x23dBm PA architecture should be mandated and would prefer to either reuse 1Tx PC3 MPR or assume 3x20dBm or 23+2x20dBm PAs.

Observation for PC2:

- Whether 2Tx TxD with two PAs or a “full” PA is used to achieve full power mode 0, the minimum architecture is with 3x23dBm PAs which benefits of an intrinsic 1.8dB back-off.

- As with 2Tx, we are not sure a 20dB isolation needs to be assessed for FWA since for the large FWA form factor, a 4x23dBm PC1.5 can already be implemented.

Proposal for 3Tx PC2:

- For 3Tx, only 10dB isolation is considered for both smartphone and FWA.

- Assuming 3x23dBm PAs, each PA will benefit from an intrinsic 1.8dB back-off. Thus, if we assume 0.8dB RIMD impact for 3Tx versus 0.5dB impact for 2Tx, any 2Tx PC2 MPR case that is less than 1.5dB MPR could use 0dB MPR.

Observation for PC1.5:

- Only 2Tx TxD can be used to achieve full power mode 0, the minimum architecture is with 3x26dBm PAs benefits of an intrinsic 1.8dB back-off.

- As with 2Tx PC2, we are not sure a 20dB isolation needs to be assessed for FWA since for large FWA form factor, a 4x23dBm PC1.5 can already be implemented.

Proposal for 3Tx PC1.5:

- For 3Tx, only 10dB isolation is considered for both smartphone and FWA.

- Assuming 3x26dBm PAs, each PA will benefit from an intrinsic 1.8dB back-off. Thus if we assume 0.8dB RIMD impact for 3Tx versus 0.5dB impact for 2Tx, any 1Tx MPR case that is less than 1.5dB MPR could use 0dB MPR.

Observation on full power mode 0:

- All power classes can achieve full power mode 0

- For PC3 and PC2 it can be achieved with either:

  - Having at least 1 PA with full power capability

  - Having at least two PAs with “half” power capability and TxD

- For PC1.5 it can be achieved with two 26dBm PAs and TxD

Proposal on full power mode 0:

- Full power mode 0 can be achieved for PC3 assuming either:

  - At least one 23dBm PA

  - At least two 20dBm PAs using 2Tx TxD.

- Full power mode 0 can be achieved for PC2 assuming either:

  - At least one 26dBm PA

  - 3x23dBm PAs and two of them using 2Tx TxD.

- Full power mode 0 can be achieved for PC1.5 assuming 3x26dBm PAs and two of them using 2Tx TxD.

Observation: Based on the above proposals we do not see the need for a new capability for power class per band per declared UL power mode for 3Tx.

# References

[1] R4-2417116 WF on UE RF requirements for MIMO enhancement, Samsung, RAN4#112bis