3GPP TSG-RAN WG4 Meeting # 113															R4-2419412

Orlando, USA, 18th – 22nd November, 2024

Agenda item:			7.1.1.3.4

Source:	Samsung

Title:	Views on SRS IL imbalance

Document for:	Discussion

1	Introduction

As one of objectives, ”6Rx for handheld and FWA UE” under Rel-19 WI on UE RF enhancements for NR FR1/FR2 and EN-DC (Phase 4) [1], the issue of insertion loss imbalance across SRS ports has been discussed to study whether the corresponding solution is required or not.

Even though RAN4 is still struggling for the justification, more than 10 solutions for the SRS IL imbalance issue have been proposed, which mostly are the same with theirs from the last release when RAN4 had similar arguments. However, in our view, RAN4 would repeat the Rel-18 situation until proven the real network performance issue as found in the past meetings. In this contribution, we would like to provide our view to conclude a unproductive dreary discussion based on the last WF [2].

2 Discussion

As mentioned earlier, also in the WID [1], the most important thing is that RAN4 has to identify the issue of the IL imbalance across SRS ports clearly from the real network operations. Otherwise, the meaningless arguments would repeat to find out a solution for the undefined problem as previous discussions in Rel-18, also as the first part of the WF, which are kept unchanged from the first meeting of Rel-19.

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19 Way Forward: RAN4 to further discuss the following options. - Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue. - Option 2: Continue to pursue a solution to the SRS IL imbalance issue based on compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain based on the outcome of Issue 4-2-1.

Proposal 1:		RAN4 has to identify the issue of the IL imbalance across SRS ports clearly from the real network operations.

Theoretically, such power imbalance between SRS antenna ports can be an issue itself. It is also possible that the SRS IL imbalance could affect overall uplink and downlink performance due to the low SRS power of Rx antenna ports if single PA based antenna switching is conducted. This would be one motivation from Rel-18 to strive for the solution to compensate the imbalance between the ports.

However, according to our prior study, we found that this kind of assumption or conclusion does not apply to real world as we have provided in [3-5]. In summary, we have got following facts from the measurement, which have not been discussed but repeated the aimless solution.

Observation 1:	No performance degradation can be shown from SRS IL imbalance up to 16 dB in real networks.

Observation 2:	Throughput loss is negligible even if 26 dB imbalance gap between SRS port (up to 0.5 dB) in real networks.

For the second part of the WF about candidate solutions, we would like to focus on the ‘candidate’ one since it is not easy to capture all proposals here.

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue Way forward: Companies are encouraged to provide comments and indication of support for the solution proposals listed below by RAN4#113. Proponent companies (in parentheses) for the candidate solution and solution proposals below are encouraged to provide expected benefit/performance gains and draftCRs so that the impacts to RAN1, RAN2, and RAN4 specifications (for RAN1 and RAN2, a description of the necessary changes is sufficient if draftCRs are not possible) can be further discussed. Other proposals including expected performance gains and draftCRs are not precluded. - Candidate solutions - Option 1: - UE performs self-compensation of SRS IL up to its maximum Tx power capabilities - UE provides assistance to the network on SRS IL compensation (semi-static and/or dynamic), e.g. - Per SRS resource power headroom reporting - Configured maximum output power per SRS resource reporting - SRS insertion loss value reporting (per SRS resource or per UE) - Other - Details of SRS IL self-compensation and assistance framework are FFS - Other options are not precluded

As we have kept asking in our contributions [3-5], it is still doubtful that how network can acknowledge the SRS power gap if it is from IL imbalance or from others, and how UEs can estimate the insertion loss value dynamically as it is not from their intention. In our understanding, the gap between SRS ports is more dependent on the channel fading, hand grip loss, or other issues than IL. These fundamental issues cannot be removed since many other underlying factors like path differences between antennas and LPAF (LNA/PA/Filter/switch) are still there affecting the imbalance even though some IL imbalance related reporting schemes or capabilities are introduced expecting the better network behavior. Then, it would give the same question about the purpose of the new capability if it cannot make any change of the SRS imbalance issue. Introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation. In our view, the SRS IL imbalance does not affect the practical system performance, nor any enhancement to resolve the issue would work effectively.

Observation 3:	Introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation given the measurement.

Nevertheless, we understand that RAN4 is now in a deadlock and it is time to close the time-consuming discussion with a clear conclusion. RAN4 should consider to set a deadline for proponent companies to provide expected benefit/performance gains and draft CRs to check any impacts to RAN1, RAN2, and RAN4 specifications. Another deadline can be set for the consensus to make any conclusion before the date.

Proposal 2:		RAN4 should consider to set a deadline for proponent companies to provide expected benefit/performance gains and draft CRs to check any impacts to RAN1, RAN2, and RAN4 specifications.

Proposal 3:		Another deadline can be set for the consensus to make any conclusion before the date.

3	Conclusion

In this contribution, we continued to provide our analysis and view on the proposed solutions. It is also proposed a possible way for RAN4 at this stage. Following observations and proposals are provided to be considered by this meeting.

Proposal 1:		RAN4 has to identify the issue of the IL imbalance across SRS ports clearly from the real network operations.

Observation 1:	No performance degradation can be shown from SRS IL imbalance up to 16 dB in real networks.

Observation 2:	Throughput loss is negligible even if 26 dB imbalance gap between SRS port (up to 0.5 dB) in real networks.

Observation 3:	Introducing new capability for IL imbalance reporting would be a meaningless solution for improving accuracy of the downlink channel estimation given the measurement.

Proposal 2:		RAN4 should consider to set a deadline for proponent companies to provide expected benefit/performance gains and draft CRs to check any impacts to RAN1, RAN2, and RAN4 specifications.

Proposal 3:		Another deadline can be set for the consensus to make any conclusion before the date.

Reference

[1] RP-241656	New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

[2] R4-2417209	WF on 6Rx UE requirements

[3] R4-2409460	Views on SRS IL imbalance

[4]	R4-2412136	Views on SRS IL imbalance issue based on performance measurements

[5]	R4-2416003	Views on SRS IL imbalance