3GPP TSG RAN WG4 Meeting #113	R4-2419429

Orlando, USA, 18 Nov. 2024 – 22 Nov. 2024

Agenda Item:	4.5.2

Source: 	Ericsson

Title:	Discussion on NR-EUTRA HO MIB decoding delay

Document for:	Discussion

# Introduction

In this contribution, we provide our views on MIB decoding delay for NR-EUTRA HO.

# Discussion

In RAN4#112 meeting, the following CR on NR-E-UTRAN HO requirement’ is agreed.

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms. If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of [50]ms.

One open issue in the CR which need further discussion is the MIB decoding delay. Currently it is mentioned as [50ms] in the agreed CR. In LTE MIB transmission periodicity is assumed to be 10ms and same MIB content is repeated for 4 frames. That means MIB is transmitted every 40 ms and repeated every 10 ms.

Figure 1: MIB tranmsision in LTE

Under low SNR regions, when UE needs to decode the MIB, the repetitions will help as the same MIB is repeated. UE is expected to use soft combining to improve the SNR and decode the MIB. In the above figure, MIB is transmitted at SFN 0 and same MIB is repeated at SFN1, 2 and 3. When the SNR is between -2dB and -8 dB (between Insync and out of sync region), we think UE may need 40ms to decode MIB. Since the HO requirements do not apply below -2dB, 1 MIB decoding period should be considered in the MIB decoding delay. Since the SNR level was not specified in the E-UTRA HO requirements, we can specify it as when the signal quality is sufficient for successful detection on the first attempt.

Hence, we propose,

- If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of 10 ms when the signal quality is sufficient for successful detection on the first attempt.

# Summary and conclusions

- If PRACH occasion is configured in radio frames with even SFN and the relative time difference between radio frame i in the current cell and the target cell is larger than or equal to 153600⋅Ts, the UE is allowed an additional MIB reception delay of 10 ms.

# References

[1]	R4-2413910