3GPP TSG-RAN WG4 Meeting #113 	R4-2419436

# Orlando, USA, 18th – 22nd , November 2024

Agenda Item:	4.5.2

Source: 	Ericsson

Title:	Discussion on interruption requirements for RLF/BFD for deactivated SCG for Rel-17 MR-DC

Document for:	Discussion

# Introduction

In this paper we provide our view on Rel-17 SCG activation interruption agreement shall be executed.

# Discussion

## Interruption requirement for SCG activation/deactivation

For Rel-17 deactivation SCG, a new parameter meascylePscell was introduced to enable both L3 measurement and the RLF/BFD measurement when SCG is being deactivated. This measurement is also agreed will cause interruption to the LTE/NR MCG.

During RAN4 103-e WF R4-2210605 agreement in regarding interruption requirements due to L3 measurement, RLM/BFD measurement as bellow:

## Sub-topic 2-3: Interruption requirements Issue 2-3-1: Interruption due to PSCell activation/deactivation Agreements achieved on GTW Agreement: - If PSCell is activated from a deactivated status - Existing requirements for interruption due to PSCell addition/release can be used as baseline, i.e., 1ms interruption length Issue 2-3-2: interruption due to PSCell activation/deactivation in asynchronous deployment. < Agreement>: For SCG activation/deactivation in ENDC, -When SCG is activated/deactivated, there are no active serving cells in the SCG. The interruption on LTE MCG can refer to clause 7.32.2.4 (Interruptions at SCell addition/release) in TS 36.133. For SCG activation/deactivation in NR-DC, the interruption requirements can refer to existing interruptions at PSCell addition/release specified in clause 8.2.4.2.1 in TS38.133. Issue 2-3-3: Interruption due to L3 measurement on deactivated PSCell, if RLM/BFD is configured < Agreement>: The agreement made in [R4-2207962] is confirmed and bracket is removed, i.e., 1%. - If RLM/BFD is configured, the current interruption requirement during Scell dormancy applies ([1]%). Issue 2-3-4: Interruption requirement due to RLM and BFD on deactivated PSCell < Agreement>: The agreement made in [R4-2207962] is confirmed and bracket is removed, i.e., 0.5%.

Currently in TS 38.133 two chapter of interruption requirements clause 8.2.4.2.17 and clause 8.2.4.2.18 reflect agreements in Issue 2-3-3 the interruption caused due to L3 measurement and Issue 2-3-4 interruption due to RLM and BFD separately.

However, in TS 36.133 only one chapter of the interruption requirement clause 7.32.2.20 which is interruptions due to RRM measurement on deactivated NR SCG was introduced. Question remains how the agreement shall be interpreted and captured in the specification.

For agreement in Issue-2-3-3, the interruption due to L3 measurement on deactivated PScell, if RLM/BFD is configured, the current interruption requirement during Scell dormancy applies which is 1% .

To our understanding, the deactivated PScell measurement all L3 measurements and RLM/BFD measurements both follow the separately configured parameter measCyclePcell using the same measurement resources SSB.

If the RLM/BFD is configured, the interruption due to L3 measurement shall be included since the UE need to follow the configuration and do the measurement according to the measCyclePScell during deactivated SCG states.

Our interpretation the 1% is a total interruption include the RRM and RLM/BFD measurement.

The agreement in Issue2-3-4 is to discuss within this 1% the RLM/BFD measurement can only cause up to 0.5% interruption.

This is also being reflected on the NR specification TS38.133 clause 8.2.3.2.17.

If the UE is not configured with RLM or BFD on the deactivated PSCell, interruptions on PCell or activated SCell(s) due to measurements on the deactivated PSCell are allowed with up to 0.5% probability of missed ACK/NACK feedback when the configured measCyclePSCell is 640ms or longer.

This correction can be seen from CR 7362 in detail.

- Proposal 1:  RAN4 update the TS36.133 interruption requirement based on the 0.5% for interruption caused by RLM/BFD measurement.

# Summary

The following have been proposed in the current contribution:

	Proposal 1:  RAN4 update the TS36.133 interruption requirement based on the 0.5% for interruption caused by RLM/BFD measurement. 2

# References

[1] R4-2210605 “WF on R17 further Multi-RAT Dual-Connectivity enhancements”, Huawei, May, 2022