# 3GPP TSG RAN WG4 Meeting #113 	R4-2419437

# Orlando, USA, 18th – 22nd , November 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 7362 | rev |  | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to add interruption requirement due to RLM/BFD measurement for deactivated PScell |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NR_DC_enh2 |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The interruption requirement for deactivated Pscell should be in two parts, one due to the L3 measurement, the other is the interruption due to RLM&BFD. However the TS36.133 only include interruption due to RRM measurement in the clause 7.32.2.20, the interruption due to RLM/BFD is not reflected. This is not aligned with TS38.133 clause 8.2.4.2.17 and clause 8.2.4.2.18. Reference agreement: WF R4-2210605 in RAN4#103e agreement: < Agreement>: Issue 2-3-3: Interruption due to L3 measurement on deactivated PSCell, if RLM/BFD is configured The agreement made in [R4-2207962] is confirmed and bracket is removed, i.e., 1%. - If RLM/BFD is configured, the current interruption requirement during Scell dormancy applies ([1]%). < Agreement>: Issue 2-3-4: Interruption requirement due to RLM and BFD on deactivated PSCell The agreement made in [R4-2207962] is confirmed and bracket is removed, i.e., 0.5%. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the interruption requirement due to RLM and BFD on deactivated Pscell. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.32.2.21 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<START OF CHANGES>>

##### 7.32.2.21 Interruptions during RLM/BFD measurements on deactivated PScell

When PSCell is configured with bfd-and-RLM with value true, if NR PScell is deactivated, the UE is for the purpose RLM/BFD measurements on the deactivated PSCell allowed to cause interruptions to activated serving cell(s) which can either be E-UTRA Pcell or Scell(s) in MCG.

The rate of ACK/NACK feedback loss on any activated serving cell resulting from RLM/BFD measurements on deactivated PSCell shall not exceed 0.5%.

<<END OF CHANGES>>