3GPP TSG-RAN WG4 Meeting #113	R4-2419443

Orlando, USA, 18th – 22nd, November 2024

Agenda Item:	7.20.3.1

Source: 	Ericsson

Title:	Discussion on RRM requirements for UE-to-UE CLI handling (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#112 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

# 2.1 RRM impacts of UE-to-UE CLI handling

### L1-CLI RSSI measurement methods

#### Issue 2-1-6: Measurement resources Recommended WF - RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements.

There are some progresses in RAN1 for CLI- RSSI measurement and these can be reflected on the latest RAN1 agreements

RAN1#118 bis agreement: For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband. For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband).

In Method #1, the RSSI measurement resource is configured within the downlink (DL) subband, and RAN1 agreed to use the DL reference timing for this approach. Additionally, it’s important to note that the RSSI frequency resources are determined by excluding any frequency resources outside the DL usable PRBs.

The wideband RSSI measurement report will not differentiate the frequency location of the RSSI measurement resource within the DL subband.

Given these two factors, the Rel-19 L1-RSSI measurement resource configuration and reporting mechanism closely resemble those of the Rel-16 L3-RSSI measurement.

In the Rel-16 L3-RSSI measurement, a single measurement sample is used. Considering that the Rel-19 CLI measurement is an L1 measurement, multiple measurement samples would add to the measurement delay, which contradict with the original purpose of introducing new L1 measurement quantity. Therefore, multiple sample measurements for RSSI in Rel-19 is unnecessary.

- Proposal 1: L1-CLI-RSSI for method #1 measurement period requirements shall be defined based on 1 sample.

One difference here to notice is the CLI-RSSI measurement timing.

For Legacy Rel-16, the RSSI measurement timing is specified as the same as SRS-RSRP with adding a constant offset.

9.7.1 Introduction The UE capable of performing CLI measurements shall be able to measure SRS-RSRP and CLI-RSSI which are defined in TS38.215 [4] within active DL BWP. The measurements requirements in this clause apply for TDD mode only. CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resouce is configured within active BWP When the UE measures SRS-RSRP and CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

From our understanding, the RSSI and SRS-RSRP measurements differ from the UE's perspective.

For RSSI measurement, the network configures the UE with a specific list of RSSI reference signals (RSSI-RS), and the UE measures based on the network configuration. This approach aligns with the latest RAN1 agreement to use downlink (DL) timing as a reference.

In contrast, the SRS-RSRP measurement involves both an aggressor UE and a victim UE. Here, the SRS is transmitted by the aggressor UE and received by the victim UE for measurement. The timing difference between the transmission from the aggressor and its reception by the victim needs to be accounted for in this measurement.

In Rel-16, the timing error introduced in RSSI measurement did not go through a simulation campaign, so the added timing error impact remains unclear. However, since this timing error is relatively small compared to the cyclic prefix (CP), and for the accuracy requirements considered, we can conclude that the effect of omitting this timing error would likely be minimal.

- Proposal 2: For L1-CLI-RSSI, the DL reference timing should be used for method #1 when setting RAN4 measurement requirement.

- Proposal 3: For L1-CLI-RSSI, reuse the Rel-16 CLI-RSSI requirement for method#1 with update of the measurement reference timing.

RSSI measurement method # 3 and two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband are both for further discussion within RAN1, RAN4 can further discuss RSSI measurement requirement when RAN1 have reached conclusion of method#3.

- Proposal 4: RAN4 can further discuss the CLI-RSSI requirement if RAN1 have reached conclusion of method #3.

### Measurement Reporting and report mapping

#### Issue 2-1-9: Measurement reporting Recommended WF: - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting - FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement - FFS whether L1-RSRP measurement reporting requirement can be re-used.

The RAN1 #117 agreement narrows down measurement reporting to aperiodic, while considering periodic and semi-persistent reporting. It's important to note that the previous RAN1 #116 bis agreement aimed to integrate CLI measurement into the CSI measurement framework.

This differs from the Rel-16 CLI reporting, which supports periodic, event-triggered, and event-triggered periodic reporting. Therefore, new requirements must be specified by RAN4 to support at least aperiodic reporting.

RAN1#116 bis agreement on Measurement reporting - CSI measurement procedure integrating CLI measurement. RAN1#117 agreement on Measurement reporting - Aperiodic reporting Note: Periodic and semi-persistent reporting can be considered RAN#118 bis agreements on measurement resource and report configuration In CSI-ReportConfig, reuse resourcesForChannelMeasurement providing the ID of a CSI-ResourceConfig that contains configuration of measurement resources for L1-SRS-RSRP and/or L1-CLI-RSSI.

The aperiodic CLI report configuration will reuse the legacy L1-RSRP CSI-ReportConfig to point the CSI-ResourceConfigure that containing the measurement resources set.

Also, the aperiodic reporting will be based on a slot offset being configured.  We believe the existing L1-RSRP aperiodic measurement reporting requirements can serve as a baseline.

- Proposal 5: For L1 CLI measurement reporting, the existing L1-RSRP aperiodic reporting requirement can be re-used.

#### Issue 2-1-13: Report mapping Recommended WF - FFS whether RAN4 needs to discuss report mapping for L1 based UE-to-UE CLI measurement. FFS whether R16 report mapping can be re-used.

The L1-RSRP measurement reporting requirements are defined alongside L3-RSRP, with a resolution of 1 dB.

The existing Rel-16 L3 CLI SRS-RSRP is specified within the range of -140 dBm to -44 dBm, with a resolution of 1 dB, accompanied by a specific mapping table that mirrors L1-RSRP, with a different range as outlined in TS 38.133, clause 10.1.22.1.2.

Furthermore, Rel-16 SRS-RSSI is defined within the range of -100 dBm to -25 dBm, with a resolution of 1 dB, as specified in TS 38.133, clause 10.1.22.2.2.

Currently RAN4 is running simulation for L1-SRS-RSRP measurement accuracy with output for the lower and upper bound of the report range.

It is also important to note that RAN1 is currently discussing report mapping, considering aspects such as mapping format, mapping order etc. RAN4 will need to align the final reporting range requirement with RAN1 to ensure there is no impact on the UCI payload.

- Proposal 6: For measurement report mapping range the Rel-19 CLI measurement reporting should be based on the simulation outcome.

- Proposal 7: RAN4 confirm the final reporting range after simulation with RAN1 for no potential UCI payload impact.

### Scheduling availability

#### Issue 2-1-11: Scheduling and measurement restriction Recommended WF - RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed. - FFS the impact of measurement methods and Rx beam assumption.

Our working assumption is that the victim UE may utilize a different timing than the DL reference timing of the serving cell while the aggressor UE transmits the SRS within the timing advance (TA) when both UEs are operating within the same cell.

Given that the SBFD UE supports only half-duplex operation, it is not feasible to transmit UL during CLI measurements.

Since the SBFD UE only support half-duplex, it is not possible to transmit UL during the CLI measurement.

For L1-SRS-RSRP, currently only measurement methods#2 is supported from RAN1, the victim UE will measure the aggressor UE within the UL subband.

Based on the latest RAN1 #117 agreements:

- Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set) i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns - Rx beams configuration for UE-to-UE CLI measurement (if spatial domain coordination is supported)

The existing legacy RS patterns are expected for Rel-19, the L1-SRS-RSRP will have same impact to the DL reception in comparing with the Rel-16 for at least FR1.

Regarding L1-CLI-RSSI, for at least method #1. Similar measurement behavior is expected as Rel-16.

- Proposal 8: For L1-SRS-RSRP, the Rel-16 scheduling availability for FR1 can be reused as a baseline.

- Proposal 9: The L1-CLI-RSSI method #1, the Rel-16 scheduling availability for FR1 can be reused as a baseline.

### RX beam configuration impact

#### Issue 2-1-5: Rx beam Recommended WF - RAN4 to wait for RAN1 conclusion on the Rx beam for L1 based UE-to-UE CLI measurement. Meanwhile, RAN4 to discuss the impacts of Rx beam configuration/determination on the requirements.

Based on the RAN1 #117 agreement for FR2, the RX beam configuration may be specified, also potentially UE capability will be introduced, the scheduling restriction requirements can wait for RAN1 progress.

If spatial domain-based schemes are supported for UE-to-UE CLI handling for FR2, the following are recommended to be specified - Rx beams configuration for UE-to-UE CLI measurement - Note: The above is subject to a separate optional UE capability.

Latest RAN1 118 bis meeting agreements in regarding the TCI states indication

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

The current Rel-16 CLI assumes CLI measurement resources are QCL-ed with Type D to the latest PDSCH or CORESET, meaning the UE uses the same RX beam as the latest PDSCH or PDCCH. One can view this as an implicit indication of QCL Type D.

The latest RAN1 agreement specify the configuration of the explicitly configure a CLI measurement resource with a TCI state (with QCL Type-D) to indicate the RX beam that the UE can use for CLI measurement only possible when unifiedTCI-StateType is configured. This is inherited from the legacy CSI-RS Rel-17 unified TCI framework.

From measurement timing aspect, currently the L1-SRS-RSRP is investigating the single sample measurement accuracy with higher SNR condition. The L1-CLI-RSSI shall also follow the Rel-16 CLI RSSI single sample measurement. The list of TCI states indicate multiple resources will not have impact on the measurement delay if only single sample will be agreed.

For this reason, we think the current approach of Rx beam indication based on the latest PDSCH or CORESET is sufficient.

For RAN4, it is reasonable to start with the Rel-16 assumption for L1-SRS-RSRP and L1-CLI-RSSI measurement requirement, as the RX beam configuration is tied to separate UE capability agreements in RAN1.

- Proposal 10: RAN4 can reuse the Rel-16 legacy QCL assumption for L1-SRS-RSRP and L1-CLI-RSSI measurement requirement.

### Measurement timing

Here we share the first round of simulation based on ideal AWGN channel and single sample results.

|  | - 15KHz FR1 | - 30KHz FR1 | - 60KHz FR1 | - 60KHz FR2 | - 120KHz FR2 |
| --- | --- | --- | --- | --- | --- |
| - Case 1 | - 5%: -0.14 dB - 95%: 0.74 dB | - 5%: -0.49 dB - 95%: 0.67 dB | - 5%: -1.32 dB - 95%: 0.58 dB | - 5%: -0.93 dB - 95%: 0.62 dB | - 5%: -2.71 dB - 95%: 0.52 dB |
| - Case 2 | - 5%: 0.03 dB - 95%: 0.76 dB | - 5%: -0.06 dB - 95%: 0.74 dB | - 5%: -0.28 dB - 95%: 0.71 dB | - 5%: -0.02 dB - 95%: 0.74 dB | - 5%: -0.20 dB - 95%: 0.73 dB |

- Observation 1: Single shot measurement with ideal channel we can observe delta RSRP is within 1dB in most of the Subcarrier spacing for both FR1 and FR2

- Observation 2: Only FR2 120KHz SCS observe high delta RSRP with consider the cell phase error.

- Proposal 11: RAN4 further investigate how cell phase error impact with realistic TDL channel.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: L1-CLI-RSSI for method #1 measurement period requirements shall be defined based on 1 sample.

	Proposal 2: For L1-CLI-RSSI, the DL reference timing should be used for method #1 when setting RAN4 measurement requirement.

	Proposal 3: For L1-CLI-RSSI, reuse the Rel-16 CLI-RSSI requirement for method#1 with update of the measurement reference timing.

	Proposal 4: RAN4 can further discuss the CLI-RSSI requirement if RAN1 have reached conclusion of method #3.

	Proposal 5: For L1 CLI measurement reporting, the existing L1-RSRP aperiodic reporting requirement can be re-used.

	Proposal 6: For measurement report mapping range the Rel-19 CLI measurement reporting should be based on the simulation outcome.

	Proposal 7: RAN4 confirm the final reporting range after simulation with RAN1 for no potential UCI payload impact.

	Proposal 8: For L1-SRS-RSRP, the Rel-16 scheduling availability for FR1 can be reused as a baseline.

	Proposal 9: The L1-CLI-RSSI method #1, the Rel-16 scheduling availability for FR1 can be reused as a baseline.

	Proposal 10: RAN4 can reuse the Rel-16 legacy QCL assumption for L1-SRS-RSRP and L1-CLI-RSSI measurement requirement.

	Proposal 11: RAN4 further investigate how cell phase error impact with realistic TDL channel.

# References

- R4-2416858 “WF on SBFD RRM”, Huawei, October ,2024

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024