# 3GPP TSG-RAN WG4 Meeting #113	R4-2419448

# Orlando, USA, November 18 – 22, 2024

Agenda Item:	7.1.1.1.1

Source: 	Huawei, Hisilicon

Title:	Discussion on SAR solution for Rel-19 HPUE

Document for:	Approval

1	Introduction

It was agreed in the RAN4#112 [1] that the general aspects of HPUE CA would be discussed, including: duty cycle solution for SAR compliance. This contribution is focused on SAR solution for Rel-19 HPUE CA.

2	Discussion

2.1 SAR solution for intra-band CA with PC1.5

In Rel-18, duty-cycle based SAR solution was discussed in 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC with 3Tx PC2/PC1.5. Considering the SAR solution in Rel-19 should be applied to specific band combinations to be requested by operators, referring to Rel-18 3Tx, duty-cycle based solution should be considered. Besides, UE implemented P-MPR can be considered as baseline SAR solution.

Proposal 1: UE implemented P-MPR is considered as baseline SAR solution. The duty-cycle based SAR solution can be optionally supported.

For intra-band CA/EN-DC, we listed the scenarios for high power UE that have been supported in the previous release and included in Rel-19 scope.

Table 1 Scenarios for intra-band EN-DC/CA high power UE that have been supported in the previous release and included in Rel-19 scope

|  |  | PC2 |  | PC1.5 |  |
| --- | --- | --- | --- | --- | --- |
|  |  | TDD | FDD | TDD | FDD |
| Intra-band EN-DC | contiguous | Support | Not Support | Support | Not Support |
|  | non-contiguous | Support | Not Support | Support | Not Support |
| Intra-band CA | contiguous | Support | Not Support | Included in current Rel-19 scope w. or w/o UL MIMO | Not Support |
|  | non-contiguous | Support | Not Support | Included in current Rel-19 scope w/o UL MIMO | Not Support |

It is shown in the table above, the intra-band EN-DC with PC1.5 and intra-band CA with PC2 are both supported in the previous releases. And the SAR solutions for intra-band CA with PC1.5 can refer to those designed for the former two if no critical issue are identified.

The duty-cycle based SAR solution for intra-band contiguous and non-contiguous CA with PC1.5 can be defined as follows in sub-clause 6.2A.1.1 and 6.2A.1.2.

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.1-1/2 and the supported power class enables the higher maximum output power than that of the default power class: - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and half the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 23dBm or lower; - shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 25% (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or - if the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is not absent and the percentage of total uplink symbols transmitted on all UL CCs in a certain evaluation period is larger than maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306 (The exact evaluation period is no less t24han one radio frame); or - if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.1/2 is 26dBm or lower; - shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2; - else shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.1/2.

With this example, the SAR solution to intra-band CA in PC2 and PC1.5 can be unified to one, based on the current structure of the SAR solution to PC2. And there is no difference of SAR solution between intra-band contiguous and non-contiguous CA.

Proposal 2: The duty-cycle based SAR solution for PC2 and PC1.5 intra-band CA can be unified based on the current structure of the SAR solution for PC2. And there is no difference of the SAR solution between intra-band contiguous and non-contiguous CA.

2.2 SAR solution for inter-band CA with PC1.5 and inter-band EN-DC with PC1.5/PC2

For inter-band CA, we listed the scenarios in Table 3 for high power UE that have been supported in the previous release and included in Rel-19 scope.

Table 3 illustrates the scenarios for PC1.5 inter-band CA with possible interests from operators, including PC2+PC2 for 2Tx, and PC2+PC1.5 for 3Tx.

The duty-cycle based SAR solution for 2Tx inter-band CA with PC1.5 can be defined as follows in sub-clause 6.2A.1.3.

If a UE supports a different power class than the default UE power class for the band combination listed in Table 6.2A.1.3-1 and the supported power class enables the higher maximum output power than that of the default power class: – if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or – if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 23dBm or lower; – shall apply all requirements for the default power class to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is not absent and the average percentage of uplink symbols transmitted in a certain evaluation period is larger than 0.5*maxUplinkDutyCycle-interBandCA-PC2 as defined in TS 38. 306 (The exact evaluation period is no less than one radio frame); or – if 10 log10 ∑ pEMAX,c or PEMAX,CA which defined in clause 6.2A.4.1.3 is 26dBm or lower; – shall apply all requirements for the power class 2 to the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3; – else; – shall apply all requirements for the supported power class and set the configured transmitted power as specified in clause 6.2A.4.1.3 (regardless of the average percentage of uplink symbols if the field of UE capability maxUplinkDutyCycle-interBandCA-PC2 is absent). The average percentage of uplink symbols is defined as 50% (DutyNR, x /maxDutyNR,x + DutyNR, y /maxDutyNR,y, ). DutyNR, x, DutyNR, y represent the actual percentage of uplink symbols transmitted in the same evaluation period (The exact evaluation period is no less than one radio frame) for NR Band x, NR Band y respectively; maxDutyNR,x, represent the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 as defined in TS 38.306, if UE supports power class 1.5. maxDutyNR,x, maxDutyNR,y represent the field of UE capability maxUplinkDutyCycle-PC2-FR1 per band as defined in TS 38.306, otherwise. For NR Band x or NR Band y, – if power class of band x in the band combination is power class 1.5 and the corresponding UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent; – the corresponding maxDutyNR,x is equal to 0.5*maxUplinkDutyCycle-PC2-FR1 – else if the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x is equal to 25%; – if power class of one or both of the bands within the band combination is power class 2 and the corresponding UE capability maxUplinkDutyCycle-PC2-FR1 is absent; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 50%; – else if the band is configured with power class 3; – the corresponding maxDutyNR,x or maxDutyNR,y is equal to 100%.

With this example, the SAR solution to inter-band CA in PC2 and PC1.5 can be unified to one, based on the current structure of the SAR solution to PC2 without the differentiation of duplex mode of the two bands in one band combination. The SAR solutions specified for PC1.5 inter-band CA with 2Tx can be applied to that with 3Tx.

Proposal 3: The duty-cycle based SAR solution for inter-band CA in PC2 and PC1.5 can be unified based on the current structure of the SAR solution for PC2 without differentiation of duplex mode. The SAR solutions specified for PC1.5 inter-band CA with 2Tx can be unified with that for 3Tx.

For inter-band EN-DC, we listed the scenarios for high power UE that have been supported in the previous release and included in Rel-19 scope.

Table 2 Scenarios for inter-band EN-DC high power UE that have been supported in the previous release and included in Rel-19 scope

|  |  |  | PC2 |  |  | PC1.5 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FDD + TDD | TDD + TDD | FDD + FDD | FDD + TDD | TDD + TDD | FDD + FDD |
| Inter-band EN-DC | 2Tx | Support | 23 + 26 23 + 23 | 23 + 23 |  |  |  |  |
|  |  | Included in current Rel-19 scope |  |  | 23 + 23 |  | 26 + 26 |  |
|  | 3Tx | Support | 23 + 26 | 23 + 23 23 + 26 |  |  |  |  |
|  |  | Included in current Rel-19 scope |  |  |  |  |  |  |

Table 2 illustrates the scenarios for PC1.5/PC2 inter-band EN-DC with possible interests from operators, including PC3+PC3 for FDD+FDD band combinations with 2Tx, and PC2+PC2 for TDD+TDD band combinations with 2Tx [6].

Observation 1: The SAR solution to inter-band EN-DC with PC3 FDD+ PC3 FDD may be supported in Rel-19.

In Rel-16, two UE capabilities of duty cycle were introduced for inter-band EN-DC SAR solution, including maxUplinkDutyCycle-interBandENDC-TDD-PC2 and maxUplinkDutyCycle-interBandENDC-FDD-TDD-PC2. The two UE capabilities of duty cycle are applicable to TDD+TDD and FDD+TDD, separately. Thus a duty cycle capability for 2Tx FDD+FDD inter-band EN-DC band combination with PC2 may need to be introduced in Rel-19. However, in RAN4#112bis, the WF [4] was approved that no new IE introduction for EN-DC PC1.5 and PC2. In this case, 2Tx FDD+FDD inter-band EN-DC band combination with PC2 and PC1.5 may not be supported in Rel-19.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission under different EUTRA TDD uplink-downlink configurations so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for inter-band TDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3 [4]. If the field is absent, 30% shall be applied to all EUTRA TDD uplink-downlink configurations. If eutra-TDD-Configx is absent, 30% shall be applied to the corresponding EUTRA TDD uplink-downlink configuration. Value n20 corresponds to 20%, value n40 corresponds to 40% and so on. | BC | No | TDD only | FR1 only |
| maxUplinkDutyCycle-interBandENDC-FDD-TDD-PC2-r16 Indicates the maximum percentage of symbols during a certain evaluation period that can be scheduled for NR uplink transmission and EUTRA FDD uplink transmission so as to ensure compliance with applicable electromagnetic energy absorption requirements provided by regulatory bodies. This field is only applicable for inter-band FDD+TDD EN-DC power class 2 UE as specified in TS 38.101-3 [4]. This capability signalling comprises of maxUplinkDutyCycle-FDD-TDD-EN-DC1 and maxUplinkDutyCycle-FDD-TDD-EN-DC2 which indicate the maxUplinkDutyCycle capability of NR band corresponding to different LTE reference configurations as described in TS 38.101-3 [4], clause 6.2B.1.3. Value n30 corresponds to 30%, value n40 corresponds to 40% and so on. | BC | No | N/A | FR1 only |

Observation 2: 2Tx FDD+FDD inter-band EN-DC band combination with PC2 and PC1.5 may not be supported in Rel-19 without a new duty cycle capability.

Besides, the 2Tx inter-band EN-DC with PC2 TDD+ PC2 TDD and the corresponding SAR solution may need to be supported in Rel-19.

Observation 3: The 2Tx inter-band EN-DC with PC2 TDD+ PC2 TDD in PC1.5 and the corresponding SAR solution may be supported in Rel-19.

Although the 3Tx inter-band EN-DC with PC2/PC1.5 are also included in Rel-19 WID scope, no demand from operators is collected until now. Therefore, it is not necessary to consider the SAR solution to 3Tx inter-band EN-DC with PC2/PC1.5 at this stage.

Observation 4: It is not necessary to further consider SAR solution to 3Tx inter-band EN-DC with PC2/PC1.5 in Rel-19, given that no new demand different from that in Rel-18 from operators until now.

With the analysis above, the SAR solution might need to be specified for the following cases:

- SAR solutions to 2Tx/3Tx inter-band CA with PC1.5 applied to any duplex modes of the two bands in one band combination.

- SAR solution to 2Tx inter-band EN-DC with PC3 FDD+ PC3 FDD.

- 2Tx and PC1.5 inter-band EN-DC with PC2 TDD+ PC2 TDD.

Proposal 4: The duty-cycle based SAR solution may be specified for the following cases:

|  |  | PC2 | PC1.5 |
| --- | --- | --- | --- |
| Inter-band CA | 2Tx |  | needed |
|  | 3Tx |  |  |
| Inter-band EN-DC | FDD + FDD | needed |  |
|  | TDD + TDD |  | needed |

Proposal 5: If no consensus on the SAR solution, leave the issue to UE implementation (i.e., P-MPR).

3	Conclusion

In this paper, we have shared our views on the general aspects of PC1.5 inter-band UL CA and EN-DC.

The following observations and proposals are made on the SAR solutions:

Proposal 1: UE implemented P-MPR is considered as baseline SAR solution. The duty-cycle based SAR solution can be optionally supported.

Proposal 2: The duty-cycle based SAR solution for PC2 and PC1.5 intra-band CA can be unified based on the current structure of the SAR solution for PC2. And there is no difference of the SAR solution between intra-band contiguous and non-contiguous CA.

Proposal 3: The duty-cycle based SAR solution for inter-band CA in PC2 and PC1.5 can be unified based on the current structure of the SAR solution for PC2 without differentiation of duplex mode. The SAR solutions specified for PC1.5 inter-band CA with 2Tx can be unified with that for 3Tx.

Observation 1: The SAR solution to inter-band EN-DC with PC3 FDD+ PC3 FDD may be supported in Rel-19.

Observation 2: 2Tx FDD+FDD inter-band EN-DC band combination with PC2 and PC1.5 may not be supported in Rel-19 without a new duty cycle capability.

Observation 3: The 2Tx inter-band EN-DC with PC2 TDD+ PC2 TDD in PC1.5 and the corresponding SAR solution may be supported in Rel-19.

Observation 4: It is not necessary to further consider SAR solution to 3Tx inter-band EN-DC with PC2/PC1.5 in Rel-19, given that no new demand different from that in Rel-18 from operators until now.

Proposal 4: The duty-cycle based SAR solution may be specified for the following cases:

|  |  | PC2 | PC1.5 |
| --- | --- | --- | --- |
| Inter-band CA | 2Tx |  | needed |
|  | 3Tx |  |  |
| Inter-band EN-DC | FDD + FDD | needed |  |
|  | TDD + TDD |  | needed |

Proposal 5: If no consensus on the SAR solution, leave the issue to UE implementation (i.e., P-MPR).

References

- R4-2414277 WF on HPUE for CA in TN, Samsung, RAN4#112

- R4-2409643 Discussion on PC1.5 for intra-band CA, Huawei, HiSilicon, RAN4#111

- R4-2409644 Discussion on PC1.5 for inter-band UL CA and EN-DC, Huawei, HiSilicon, RAN4#111

- R4-2417098 WF on HPUE RF requirements, RAN4#112bis