# 3GPP TSG-RAN WG4 Meeting #113		R4-2419456

Orlando Meeting, Nov. 18th – Nov 22nd, 2024

Source:	Nokia

Title:	On UE RF specifications table improvement

Agenda item:	9.1.1.4

Document for:	Discussion

Introduction

During RAN#103 a WF was agreed to improve the RAN4 specifications with relevance for this agenda focus on the UE RF specification in TS 38.101-1, 38.101-2 and 38.101-3. The RAN WF is included below for reference [1]:

- The RAN4 Rel-19 specifications are expected to be available by December 2024. - RAN4 will organize the discussions for improving the specifications in Q2 and Q3 2024 in RAN4 meeting(s), and report to RAN#104 and RAN#105 - Focus on 38.133 and 38.101-1/38.101-2/38.101-3, not covering other specifications in this RAN task - Motivation of the work: - Try to improve the above specifications for Rel-19 for 5G in the short term - Try to conclude on guidance including the structure, drafting rule to ensure the quality of specifications for UE RF and RRM. - Set up one dedicated agenda to collect the input from companies for specification improvement - Companies are expected to point out the key issues and also provide the concrete solutions. - No corresponding CR is expected before September - Schedule the specific time slot for the single discussions on the specification improvement in RAN4 main session starting from April - Identify the key issues and root reasons behind - Summarize the candidate solutions for the next action - Further discuss and decide how to capture the outcome of this RAN task in RAN#105

At RAN4#110bis the RAN WF above was further discussed and relate to UE RF a RAN4 WF [2] was agreed include the following way-forward:

<Way forward/Agreement>: - Following proposals to be discussed in UE RF Spec specification improvement agenda: Proposals in R4-2405036 - Proposal 1: RAN4 to discuss if information/requirements spread across multiple tables in the TS can be merged into a single table. - Proposal 3: RAN4 to discuss if the listing of UE relaxation per band combination can be collected into a single table.

Based on the WF a discussion was initiated at RAN4#110bis in [3]. At RAN4#111 there where unfortunately no further discussions on the proposals in [6] why they were repeated at RAN4#112 in [7]. This meeting we further discuss the proposals for improving the RAN4 UE RF specification.

Discussion

Length/size of tables related to band combinations

Listing of supported band combinations

When opening RAN4 UE specification TS 38.101-1 it is evident that the dominant part of the specification content is tables listing band combinations. The first tables list simply what DL and UL configurations are supported and as one moves on in the specification additional information is added on e.g. supported channel bandwidths within the listed DL and UL bands. Listing the same band combinations multiple times just adding additional information is needed simply because it is not currently practical to condense all the information and requirements for a single DL configuration into a single table.

Currently it is not possible to condense all the information and requirements for a single DL configuration into a single table.

As a result, RAN#102 commissioned an activity to identify a new methodology with a small working party in conjunction with ETSI MCC to handle capturing a large number of band combinations. The status of this work was first presented at RAN#103 in [4] and latest in [8]. The intention of this work is in long term to shift the listing of the band combinations from the Word-based specification to a database managed by MCC and then simply reference from the written Word-based specification.

The long-term goal is to move the listing of band combinations to a database managed by MCC.

However, due to the complexity of the information related to the band combinations, this may take significant time and RAN4 can in the meantime investigate whether additional simplification and removal of redundancy can be achieved.

Definition of UE relaxations per band combination

A starting point to reduce the length and number of tables for band combinations is, as suggested in [3, 6, 7], to investigate if some of the information/requirements now spread across multiple tables listing band combinations can be merged into a single table listing the band combination.

Multiple tables are now listing band combinations meaning that there are numerous long tables in the specification.

UE relaxations per band combination, e.g. ΔTIB,c and ΔRIB,c and even MSD in many cases, have the same value dependent on the type of issue, e.g. harmonics, creating the need for the relaxation. Even so, these relaxation values are currently listed per band combination in multiple tables for each type of issue.

Currently the RAN4 UE RF specification has separate tables for each UE relaxation type, e.g. MSD due to harmonica mixing issues.

A suggestion is to list relaxation per UE issue/type in a single table and then list all the types of issues for the specific band combinations in list of supported band combinations. By grouping a list of issues per band combination would provide an overview of the individual band combinations combined need for UE relaxation.

Providing a list of supported band combinations together with their “issues” requiring relaxation would provide an overview instead of spreading the information over multiple tables in the specification.

In a practical example for TS 38.101-1 only targeting Clause 7 this would mean introducing the following 8 as shown in the following in Table 1-8. The Reasoning for the values proposed in these tables is given in [3, 6, 7] which shows the statistics and investigations conducted for the currently defined UE relaxations in TS 38.101-1.

With the agreement of the LowerMSD capability signaling only a worst-case MSD value is required for each MSD type and order, including band-group relations Table 1 to Table 8 in the following has been made using the highest MSD for each MSD type and MSD order. If the MSD is expected to be much better than specified in the generic requirements it is expected that UE vendors signal the improved MSD Class for the combination.

Statistics and investigations conducted for the currently defined UE relaxations in TS 38.101-1 clause 7 is presented in [3, 6, 7].

Proposed new table structure for 38.101-1 presented in Table 1-9:

Table 1: Band group definition

| FR1 band group range |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | FR1-1 | FR1-2 | FR1-3 | FR1-4 | FR1-5 |
| Range (MHz) | 600-1000 | 1400-2200 | 2300-2700 | 3300-5000 | 5150-7125 |

In the industry these band groups have also been described as FR1-1 = LB, FR1-2 = MB, FR1-3= HB while FR1-4 and FR1-5 combines into UHB used in the following tables.

Table 2: Uplink harmonic requirements.

| PC3 |  | PC2 |  | PC1.5 |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | Order | MSD | Order | MSD |
| UL2 | 23.9dB | UL2 | 27.1dB | UL2 | 32.3dB |
| UL3 | 9dB | UL3 | N/A | UL3 | N/A |
| UL4 | 11dB | UL4 | N/A | UL4 | N/A |
| UL5 | 10.5dB | UL5 | N/A | UL5 | N/A |

Table 3: Harmonic mixing requirements PC3.

| PC3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Order | MB/LB | HB/LB | HB/MB | HB/HB | UHB/LB | UHB/MB | UHB/HB | UHB/UHB |
| UL1/DL2 | 3.7dB | N/A | 6.7dB | N/A | N/A | 8.3dB | N/A | N/A |
| UL1/DL3 | 26.5dB | 24.3dB | N/A | N/A | N/A | 13.5dB | N/A | N/A |
| UL1/DL4 | N/A | 5.7dB | N/A | N/A | 5.7dB | N/A | N/A | N/A |
| UL1/DL5 | N/A | N/A | N/A | N/A | 31dB | N/A | N/A | N/A |
| UL2/DL3 | N/A | N/A | N/A | N/A | N/A | N/A | 10.4dB | 22.6dB |
| UL3/DL4 | N/A | N/A | N/A | 4.5dB | N/A | N/A | N/A | N/A |
| UL4/DL3 | N/A | N/A | N/A | N/A | N/A | N/A | 8.3dB | N/A |

Table 4: Harmonic mixing requirements PC2/PC1.5.

| PC2/PC1.5 |  |  |  |
| --- | --- | --- | --- |
| Order | UHB/LB | UHB/MB | UHB/HB |
| UL1/DL2 | N/A | 9.2dB/11.9dB | N/A |
| UL1/DL4 | 8.1dB/10.7dB | N/A | N/A |
| UL1/DL5 | 34dB/37dB | N/A | N/A |
| UL2/DL3 | N/A | N/A | 13.2dB/16.1dB |

Table 5: Harmonic mixing requirements PC5.

| PC5 |  |  |
| --- | --- | --- |
| Order | UHB/HB | UHB/UHB |
| UL1/DL2 | 5.3dB | N/A |
| UL2/DL3 | N/A | 20.3dB |

Table 6: IMD requirements for PC3-PC5.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 30dB | IMD4 | 8.6dB | IMD2 | 30dB |
| IMD3 same b.gr | 25dB | IMD5 same b.gr | 22dB | IMD3 same b.gr | 25dB |
| IMD3 diff. b.gr | 15dB | IMD5 diff. b.gr | 17.5dB | IMD3 diff b.gr. | 17.5dB |
| IMD4 | 8.6dB | IMD7 same b.gr | 8.5dB | IMD4 | 12dB |
| IMD5 | 5.5dB | IMD7 diff. b.gr | 3.2dB | IMD5 | 6dB |
|  |  | IMD9 | 2.8dB | IMD7 | 2.5dB |

Table 7: IMD requirements for PC2.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 36dB | IMD4 | 18.6dB | IMD2 | 36.5dB |
| IMD3 same b.gr | 25dB | IMD5 same b.gr | 32.5dB | IMD3 same b.gr | 26.5dB |
| IMD3 diff. b.gr | 15dB | IMD5 diff. b.gr | 22dB | IMD3 diff b.gr. | 22dB |
| IMD4 | 19dB | IMD7 same b.gr | 13.5dB | IMD4 | 22dB |
| IMD5 | 14dB | IMD7 diff. b.gr | 9dB | IMD5 | 21dB |
|  |  | IMD9 | 3dB | IMD7 |  |

Table 8: IMD requirements for PC1.5.

| 2 CA IMD |  |  |  | 3 CA IMD |  |
| --- | --- | --- | --- | --- | --- |
| Order | MSD | 2CA (NC_ULCA) | MSD | Order | MSD |
| IMD2 | 37.5dB | IMD4 | N/A | IMD2 | N/A |
| IMD3 same b.gr | N/A | IMD5 same b.gr | N/A | IMD3 same b.gr | N/A |
| IMD3 diff. b.gr | N/A | IMD5 diff. b.gr | N/A | IMD3 diff b.gr. | N/A |
| IMD4 | 26.5dB | IMD7 same b.gr | N/A | IMD4 | N/A |
| IMD5 | 30dB | IMD7 diff. b.gr | N/A | IMD5 | N/A |
|  |  | IMD9 | N/A | IMD7 | N/A |

Referring to the relaxations listed in Table 2-8, sometimes dependent on the band group listed in Table 1, it would be possible to list all e.g. 2 band combinations as shown in Table 9 below. Note that the development of the IMD test point equations is still ongoing, hence shown in gray.

Table 9: 2 UL CA configurations with MSD requirements.

| NR CA band combination (CA_nX-nY) | NR Rx band (nZ) | Uplink Harmonic | Harmonic Mixing | Cross Band Interf. | Order | NC ULCA | IMD test point equation UL Band(x,y) to DL Band(z) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n1-n3 | n1 |  |  |  | IMD3 same |  | I2*nX-nYI=nZ |
| CA_n1-n3 | n3 |  |  | 1,>2 |  |  |  |
| CA_n1-n8 | n1 |  |  |  | IMD4 |  | I2*nX-2*nYI=nZ |
| CA_n1-n28 | n1 | UL3 |  |  |  |  |  |
| CA_n1-n38 | n1 |  |  | >2 |  |  |  |
| CA_n1-n38 | n38 |  |  | >2 |  |  |  |
| CA_n1-n40 | n1 |  |  | >2 |  |  |  |
| CA_n1-n40 | n40 |  |  | >2 |  |  |  |
| CA_n1-n41 | n41 |  |  | >2 |  |  |  |
| CA_n1-n46 | n1 |  |  |  | IMD5 |  | InY-4*nXI=nZ |
| CA_n1-n77 | n1 |  |  |  | IMD24 | IMD5 diff | InY-nXI=nZ; nY-2*2CCBW=nZ |
| CA_n1-n77 | n77 | UL2 |  |  |  |  |  |
| CA_n1-n78 | n1 |  |  |  | IMD4 | IMD7 | I3*nX-nYI=nZ; nY-3*2CCBW=nZ |
| CA_n1-n102 | n1 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n1-n105 | n105 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n2-n38 | n2 |  |  | >2 |  |  |  |
| CA_n2-n41 | n2 |  |  | >2 |  |  |  |
| CA_n2-n48 | n2 |  |  |  | IMD4 |  |  |
| CA_n2-n48 | n48 | UL2 |  |  |  |  |  |
| CA_n2-n66 | n2 |  |  |  | IMD3 same |  |  |
| CA_n2-n66 | n66 |  |  |  | IMD5 |  |  |
| CA_n2-n71 | n2 | UL3 |  |  |  |  |  |
| CA_n2-n71 | n71 |  | UL1/DL3 |  |  |  |  |
| CA_n2-n772,3 | n2 |  | UL1/DL2 |  | IMD24, IMD4 | IMD7 |  |
| CA_n2-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n2-n78 | n2 |  |  |  | IMD24 |  |  |
| CA_n2-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n3-n5 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n5 | n5 |  |  |  | IMD2 |  |  |
| CA_n3-n7 | n3 |  |  | >2 |  |  |  |
| CA_n3-n7 | n7 |  |  |  | IMD4 |  |  |
| CA_n3-n8 | n3 |  |  |  | IMD5 |  |  |
| CA_n3-n8 | n8 |  |  |  | IMD4 |  |  |
| CA_n3-n18 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n20 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n20 | n20 |  |  |  | IMD4 |  |  |
| CA_n3-n26 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n26 | n26 |  | UL1/DL2 |  | IMD24 |  |  |
| CA_n3-n34 | n3 |  |  | >2 |  |  |  |
| CA_n3-n38 | n3 |  |  |  | IMD4 |  |  |
| CA_n3-n41 | n3 |  |  | >2 | IMD3 diff, IMD4 | IMD9 |  |
| CA_n3-n41 | n41 |  |  | >2 |  |  |  |
| CA_n3-n74 | n74 |  |  | >2 |  |  |  |
| CA_n3-n75 | n75 |  |  | >2 |  |  |  |
| CA_n3-n772,3 | n3 |  |  |  | IMD24, IMD4 | IMD7 |  |
| CA_n3-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n3-n782,3 | n3 |  |  |  | IMD24, IMD4 | IMD7 |  |
| CA_n3-n782,3 | n78 | UL2 |  |  |  |  |  |
| CA_n5-n7 | n5 |  |  |  | IMD3 diff |  |  |
| CA_n5-n14 | n5 |  |  |  | IMD3 same |  |  |
| CA_n5-n14 | n5 |  |  |  | IMD3 same |  |  |
| CA_n5-n41 | n28 |  |  | 2 |  |  |  |
| CA_n5-n41 | n5 |  | UL1/DL3 |  | IMD3 diff |  |  |
| CA_n5-n41 | n41 | UL3 |  |  |  |  |  |
| CA_n5-n66 | n5 |  |  |  | IMD24 |  |  |
| CA_n5-n71 | n5 |  |  | >2 |  |  |  |
| CA_n5-n71 | n71 |  |  | >2 |  |  |  |
| CA_n5-n772,3 | n5 |  |  |  | IMD24, IMD4 | IMD4 |  |
| CA_n5-n772,3 | n77 | UL4, UL5 |  |  |  |  |  |
| CA_n5-n78 | n5 |  |  |  | IMD4 | IMD4 |  |
| CA_n5-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n7-n8 | n7 | UL3 |  |  |  |  |  |
| CA_n7-n26 | n7 |  |  |  | IMD5 |  |  |
| CA_n7-n26 | n26 |  |  |  | IMD3 diff |  |  |
| CA_n7-n40 | n7 |  |  | >2 | IMD3 same |  |  |
| CA_n7-n40 | n40 |  |  | >2 |  |  |  |
| CA_n7-n464 | n7 |  |  |  | IMD24 |  |  |
| CA_n7-n464 | n46 |  | UL1/DL2 |  |  |  |  |
| CA_n7-n66 | n7 |  |  |  | IMD4 |  |  |
| CA_n7-n71 | n7 | UL4 |  |  |  |  |  |
| CA_n7-n77 | n7 |  |  | >2 | IMD4 | IMD5 diff |  |
| CA_n7-n78 | n7 |  |  | >2 |  |  |  |
| CA_n7-n105 | n7 | UL3 |  |  |  |  |  |
| CA_n8-n20 | n8 |  |  |  | IMD3 same |  |  |
| CA_n8-n20 | n20 |  |  |  | IMD3 same |  |  |
| CA_n8-n41 | n8 |  |  |  | IMD3 diff |  |  |
| CA_n8-n41 | n41 | UL3 |  |  |  |  |  |
| CA_n8-n77 | n77 | UL4 |  |  |  |  |  |
| CA_n8-n782 | n8 |  |  |  | IMD4 |  |  |
| CA_n8-n782 | n78 | UL4 |  |  |  |  |  |
| CA_n8-n792,3 | n8 |  | UL1/DL5 |  | IMD5 |  |  |
| CA_n8-n792,3 | n79 | UL5 |  |  |  |  |  |
| CA_n12-n48 | N48 | UL5 |  |  |  |  |  |
| CA_n12-n66 | n66 | UL3 |  |  |  |  |  |
| CA_n12-n71 | n12 |  |  | 2,>2 |  |  |  |
| CA_n12-n772,3 | n12 |  | UL1/DL5 |  |  |  |  |
| CA_n12-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n12-n78 | n12 |  | UL1/DL5 |  |  |  |  |
| CA_n12-n78 | n78 | UL5 |  |  |  |  |  |
| CA_n13-n772,3 | n13 |  | UL1/DL5 |  |  |  |  |
| CA_n13-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n14-n772,3 | n14 |  | UL1/DL5 |  |  |  |  |
| CA_n14-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n18-n28 | n28 |  |  | 1,2 |  |  |  |
| CA_n18-n41 | n18 |  | UL1/DL3 |  |  |  |  |
| CA_n18-n77 | n18 |  |  |  | IMD4, IMD5 |  |  |
| CA_n18-n77 | n77 | UL5 |  |  |  |  |  |
| CA_n18-n78 | n18 |  |  |  | IMD4 |  |  |
| CA_n20-n40 | n20 |  | UL1/DL3 |  |  |  |  |
| CA_n20-n78 | n20 |  |  |  | IMD4 |  |  |
| CA_n20-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n24-n77 | n24 |  |  |  | IMD4 |  |  |
| CA_n25-n38 | n25 |  |  | >2 |  |  |  |
| CA_n25-n41 | n25 |  |  | >2 |  | IMD7 |  |
| CA_n25-n48 | n25 |  |  |  | IMD4 |  |  |
| CA_n25-n66 | n25 |  |  |  | IMD3 same |  |  |
| CA_n25-n66 | n66 |  |  |  | IMD3 same, IMD5 |  |  |
| CA_n25-n71 | n25 | UL3 |  |  |  |  |  |
| CA_n25-n71 | n71 |  | UL1/DL3 |  |  |  |  |
| CA_n25-n772,3 | n25 |  | UL1/DL2 |  | IMD24, IMD4 | IMD7 |  |
| CA_n25-n772,3 | n77 | UL2 |  |  |  |  |  |
| CA_n25-n78 | n25 |  |  |  | IMD24 | IMD7 |  |
| CA_n25-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n26-n29 | n29 |  |  | >2 |  |  |  |
| CA_n26-n48 | n26 |  | UL1/DL4 |  |  |  |  |
| CA_n26-n66 | n26 |  |  |  | IMD24 |  |  |
| CA_n26-n70 | n26 |  |  |  | IMD24 |  |  |
| CA_n26-n77 | n26 |  | UL1/DL4 |  |  |  |  |
| CA_n26-n78 | n26 |  | UL1/DL4 |  | IMD4 |  |  |
| CA_n26-n78 | n78 | UL4 |  |  |  |  |  |
| CA_n28-n40 | n28 |  | UL1/DL3 |  |  |  |  |
| CA_n28-n50 | n28 |  |  |  | IMD2, IMD4 |  |  |
| CA_n28-n50 | n50 | UL2 |  |  |  |  |  |
| CA_n28-n71 | n28 |  |  | >2 |  |  |  |
| CA_n28-n71 | n71 |  |  | 2 |  |  |  |
| CA_n28-n74 | n28 |  |  |  | IMD2, IMD4 |  |  |
| CA_n28-n74 | n74 | UL2 |  |  | IMD4, IMD5 |  |  |
| CA_n28-n75 | n75 | UL2 |  |  |  |  |  |
| CA_n28-n772,3 | n28 |  |  |  | IMD2, IMD5 | IMD4 |  |
| CA_n28-n772,3 | n77 | UL5 |  |  |  |  |  |
| CA_n28-n782 | n28 |  |  |  | IMD5 |  |  |
| CA_n28-n782 | n78 | UL5 |  |  |  |  |  |
| CA_n28-n94 | n94 | UL2 |  |  |  |  |  |
| CA_n29-n48 | n29 |  | UL1/DL5 |  |  |  |  |
| CA_n29-n71 | n29 |  |  | 2 |  |  |  |
| CA_n29-n772,3 | n29 |  | UL1/DL5 |  |  |  |  |
| CA_n30-n66 | n66 |  |  | >2 |  |  |  |
| CA_n30-n772,3 | n30 |  | UL2/DL3 |  | IMD4 | IMD7 |  |
| CA_n34-n41 | n34 |  |  | >2 |  |  |  |
| CA_n34-n41 | n41 |  |  | >2 |  |  |  |
| CA_n38-n66 | n66 |  |  | >2 |  |  |  |
| CA_n38-n78 | n38 |  |  | >2 |  |  |  |
| CA_n38-n78 | n78 |  |  | >2 |  |  |  |
| CA_n39-n41 | n39 |  | UL3/DL4 | >2 |  |  |  |
| CA_n39-n41 | n41 |  |  | >2 |  |  |  |
| CA_n40-n41 | n40 |  |  | 2 |  |  |  |
| CA_n40-n41 | n41 |  |  | 2 |  |  |  |
| CA_n40-n772 | n40 |  | UL2/DL3 |  |  |  |  |
| CA_n40-n772 | n77 |  |  | >2 |  |  |  |
| CA_n40-n78 | n40 |  | UL2/DL3 | >2 |  |  |  |
| CA_n41-n48 | n41 |  |  | >2 |  |  |  |
| CA_n41-n48 | n48 |  | UL4/DL3 | >2 |  |  |  |
| CA_n41-n66 | n66 |  |  | >2 |  | IMD5 same |  |
| CA_n41-n70 | n70 |  |  | >2 |  |  |  |
| CA_n41-n71 | n41 | UL4 |  |  |  |  |  |
| CA_n41-n71 | n71 |  |  |  | IMD4 |  |  |
| CA_n41-n772 | n41 |  | UL2/DL3 | >2 |  |  |  |
| CA_n41-n772 | n77 |  |  | >2 |  | IMD9 |  |
| CA_n41-n78 | n41 |  | UL2/DL3 | >2 |  | IMD9 |  |
| CA_n41-n78 | n78 |  |  | >2 |  |  |  |
| CA_n46-n484 | n46 |  |  | >2 |  |  |  |
| CA_n46-n484 | n48 |  | UL2/DL3 | >2 |  |  |  |
| CA_n46-n77 | n46 |  | UL2/DL3 |  |  | IMD5 diff, IMD6, IMD7 |  |
| CA_n46-n77 | n77 |  |  | >2 |  |  |  |
| CA_n46-n784 | n46 |  | UL2/DL3 | >2 |  | IMD6, IMD7 |  |
| CA_n46-n784 | n78 |  |  | >2 |  |  |  |
| CA_n48-n66 | n48 | UL2 |  |  |  |  |  |
| CA_n48-n66 | n66 |  |  |  | IMD5 |  |  |
| CA_n48-n70 | n70 |  |  |  | IMD24 |  |  |
| CA_n48-n964 | n48 |  | UL2/DL3 | >2 |  |  |  |
| CA_n48-n964 | n96 |  |  | >2 |  |  |  |
| CA_n66-n71 | n66 |  |  |  | IMD4 |  |  |
| CA_n66-n77 | n66 |  |  |  | IMD2, IMD5 | IMD5 diff, IMD7 |  |
| CA_n66-n77 | n77 | UL2 |  |  |  |  |  |
| CA_n66-n78 | n66 |  |  |  | IMD5 | IMD7 |  |
| CA_n66-n78 | n78 | UL2 |  |  |  |  |  |
| CA_n66-n85 | n66 | UL3 |  |  | IMD4 |  |  |
| CA_n67-n78 | n67 |  | UL1/DL5 |  |  | IMD4 |  |
| CA_n70-n71 | n70 |  |  |  | IMD4 |  |  |
| CA_n70-n71 | n71 | UL3 |  |  |  |  |  |
| CA_n70-n77 | n70 | UL2 |  |  | IMD2, IMD5 |  |  |
| CA_n70-n78 | n70 | UL2 |  |  | IMD2, IMD5 |  |  |
| CA_n71-n77 | n71 |  |  |  | IMD5 | IMD4 |  |
| CA_n71-n78 | n71 |  |  |  | IMD5 |  |  |
| CA_n71-n85 | n71 |  |  | >2 |  |  |  |
| CA_n71-n85 | n85 |  |  | 1,2 |  |  |  |
| CA_n77-n85 | n77 | UL5 |  |  |  |  |  |
| CA_n77-n85 | n85 |  | UL1/DL5 |  | IMD5 |  |  |
| CA_n78-n79 | n78 |  |  | >2 |  |  |  |
| CA_n78-n79 | n79 |  |  | >2 |  |  |  |
| CA_n78-n92 | n78 | UL4 |  |  |  |  |  |
| CA_n78-n94 | n78 | UL4 |  |  |  |  |  |
| CA_n78-n102 | n102 |  |  |  |  | IMD4 |  |
| CA_n78-n105 | n78 | UL5 |  |  |  |  |  |
| CA_n78-n105 | n105 |  |  |  | IMD5 |  |  |
| Note 2: Power class 2 supported. Note 3: Power class 1.5 supported. Note 4: Power class 5 supported. |  |  |  |  |  |  |  |

It should be noted that the information now captured in Table 1-9 currently in TS 38.101-1 within Clause 7 takes up 23 pages meaning if RAN4 adopts this approach 18 could be saved. If RAN4 than also incorporates the ΔRIB,c values into the table additionally 3 pages can be saved in the specification just in Clause 7.

RAN4 could reduce the length of TS 38.101-1 by 21 pages using the approach presented here.

Given the number of saved pages together with the possibility of providing an overview of UE relaxations per band combinations it is suggested that RAN4 proceeds developing the presented unified table approach.

RAN4 shall further develop the unified table approach for UL configurations as presented in this Toc and adopt this in the specification.

Additionally it can be investigated of the band combinations lister per DL configuration can be simplified in similar manner as by merging the information in Clause 5 for DL interruption with the relaxations listed in Clause 6, i.e. ΔTIB,c, per band combination.

RAN4 shall further investigate whether a unified tabled can be developed for DL configurations.

Conclusion

In this contribution, we discuss suggestions for improving the RAN4 UE RF specification in TS 38.101-1, 38.101-2 and 38.101-3. The following Observations and Proposals were made:

Observation 1: Currently it is not possible to condense all the information and requirements for a single DL configuration into a single table.

Observation 2: The long-term goal is to move the listing of band combinations to a database managed by MCC.

Observation 3: Multiple tables are now listing band combinations meaning that there are numerous long tables in the specification.

Observation 4: Currently the RAN4 UE RF specification has separate tables for each UE relaxation type, e.g. MSD due to harmonica mixing issues.

Observation 5: Providing a list of supported band combinations together with their “issues” requiring relaxation would provide an overview instead of spreading the information over multiple tables in the specification.

Observation 6: Statistics and investigations conducted for the currently defined UE relaxations in TS 38.101-1 clause 7 is presented in [3, 6, 7].

Observation 7: RAN4 could reduce the length of TS 38.101-1 by 21 pages using the approach presented here.

Proposal 1: RAN4 shall further develop the unified table approach for UL configurations as presented in this Toc and adopt this in the specification.

Proposal 2: RAN4 shall further investigate whether a unified tabled can be developed for DL configurations.

- References

- RP-240782, Moderator's summary on RAN4 spec quality, RAN4 chair (Huawei).

- R4-2406709, WF on UERF_Spec_Improvement, Qualcomm

- R4-2405036, On UE RF specifications Improvement, Nokia

- RP-240263, Progress update on CA framework overhaul, Nokia

- R4-2409191, On RAN4 basket WI work planning, Nokia

- R4-2409190, On UE RF specifications table improvement, Nokia

- R4-2413338, On UE RF specifications table improvement, Nokia

- R4-2415389, Progress update and next steps for CA framework overhaul, Nokia