# 3GPP TSG-RAN WG4 Meeting # 113						R4-2419478

# Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			TP to TR 38.769:  RF overview for UE as reader

Agenda item:		7.21.3.3

Document for:		Approval

# Introduction

In this paper, we present our overview for RF requirements overview for UE as reader.

# Discussion

For UE as reader RF requirement discussion, there are many aspects similar with the BS side. These aspects are frequency bands discussion and some of transmitter and receiver requirements.

Current the coexisting scope includes the scenarios and cases which describes the different factors to be considered for the UE as reader in D2T2-A2. These factors are listed in Table 1 below. As RAN4 agree that there is no frequency shifting for D2R transmission from device side, the UL spectrum will be used for D2R when CW transmission occurs in UL spectrum. The UE as reader can transmit CW transmission or not depending on its capability. Comparing to the A-IoT BS deployment where the separate CW node may be deployed co-located with A-IoT BS, it may not so easy to deploy one UE as reader and another co-located CW node, in this sense, we suggest focusing on the UE as reader can transmit the CW signal.

The UE as reader can transmit CW transmission or not depending its capability

Table 1: Factors impacting the UE as A-IoT reader operation

|  | Spectrum for R2D | Spectrum for D2R | Spectrum for CW transmission | UE as reader to transmit CW | Comment |
| --- | --- | --- | --- | --- | --- |
| UE as reader | UL spectrum | UL spectrum | UL spectrum | Yes | D2T2-A2 |
|  | UL spectrum | DL spectrum | DL spectrum | No | D2T2-B |

If UE as reader has capability to transmit the CW signal, CW signal needs to be transmitted and the CW waveform and generation has been discussed in RAN1 with below agreements:

RAN1#116:

Agreement For R19 A-IoT study item, at least single-tone unmodulated sinusoid waveform is a candidate waveform for carrier wave for D2R backscattering. Agreement For R19 A-IoT study item, multi-tone waveforms for carrier wave for D2R backscattering can be studied.

RAN1#116bis:

Agreement For CW waveform for D2R backscattering, multiple unmodulated single-tone is studied compared to single-tone in R19 SI. - Two unmodulated single-tones as a starting point - FFS: Other number of tones - FFS: how large gap is needed between tones Agreement For CW waveform for D2R backscattering, contiguous multi-tone OFDM signal is not studied in R19 SI. Agreement Study at least the following characteristics of unmodulated single-tone and multiple unmodulated single-tone CW waveforms for backscattering: - For D2R - Reception performance - Spectrum utilization of backscattered signal corresponding to the CW waveforms - CW interference suppression at D2R receiver - Including complexity and CW cancellation capability value/range (if any) - For scenarios ’A1’, ’A2’ and ’B’ - Relative complexity of CW generation

RAN#103:

Proposal 3v2 (endorsed) - Regarding the objective in the SID: Study necessary characteristics of carrier-wave waveform for a carrier wave provided externally to the Ambient IoT device, including for interference handling at Ambient IoT UL receiver, and at NR basestation. - This objective allows studying CW waveform characteristics which would need control of the CW node(s), e.g. waveform characteristics that impact interference such as when CW is transmitted or not transmitted, power, bandwidth, spectrum, etc.

From the RAN1 and RAN agreements, it can be observed that the CW waveform is defined with unmodulated sinusoid waveform, and maximum number of CW signal is limited to two. As the CW transmission is controlled in terms of the timing, power, bandwidth, and spectrum, CW signal is relevant and RAN4 can continue to specify the RF characteristic of CW signal based on these basic assumptions.

CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

CW transmission’s timing, power, bandwidth, spectrum is controlled.

CW RF characteristic can be studied in RAN4 according to the RAN1 and RAN agreements.

Because the simultaneous CW and R2D transmission may interfere the R2D signal reception at device 1/2a. Therefore, the R2D and CW transmission is TDM and CW signal transmission requirement should be discussed separately with R2D transmission.

CW and R2D transmission should be TDM to prevent the CW interfering with the R2D reception at device.

CW and R2D transmission requirement can be discussed separately.

For A-IoT R2D transmitter requirement, the R2D signal design very similar with LP-WUS signal design, for example, the OOK1 and OOK4 design are following the LP-WUS signal design and RAN1 agreement is quoted as below. Though there is no LP-WUS transmission from UE side, we think the R2D signal still can be referred to the LP-WUS discussion on RF specification.

RAN1 has below agreement regarding the R2D signal waveform and it can be observed that the OFDM-based waveform is agreed and from this perspective, the legacy UE transmitter requirement can be reused.

No need to discuss the A-IoT R2D transmitter requirement but depending on the LP-WUS work item conclusion.

RAN1#116:

Agreement A-IoT DL study includes an OFDM-based waveform from A-IoT R2D (reader-to-device) perspective. - Depending on what modulation(s) are decided to be studied: - Study whether/how to handle CP at transmitter/device/design - Study other characteristics of the OFDM waveform, e.g.: - CP-OFDM - DFT-s-OFDM - Etc. - The type of OFDM waveform is transparent to A-IoT device. Other waveforms from DL transmitter’s perspective can be proposed, and further discussion will consider whether or not they are included in the study.

Agreement

A-IoT DL study includes OOK from DL transmitter’s perspective.

- For an OFDM waveform, assume OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869.

  - FFS value(s) of M.

  - FFS: Any changes needed from the definitions in TR 38.869.

  - FFS: Exact definition of chip

- If other DL waveforms are included, further elaboration of the transmitter’s OOK generation would be needed.

RAN1#116:

Agreement A-IoT DL study includes OOK from DL transmitter’s perspective. - For an OFDM waveform, assume OOK-1 for single-chip per OFDM symbol transmission, and OOK-4 for M-chip per OFDM symbol transmission, starting from definitions in TR 38.869. - FFS value(s) of M. - FFS: Any changes needed from the definitions in TR 38.869. - FFS: Exact definition of chip - If other DL waveforms are included, further elaboration of the transmitter’s OOK generation would be needed.

The RF requirement relating to the CW signal should be discussed separately for transmitter requirement as the TDM operation between R2D and D2R.  In this sense, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission are relevant requirement. As this is a new waveform (unmodulated sinusoid), the modulated signal quality is not applied. However, the MPR/A-MPR needed to comply the different regulator requirement needs to be further studied. The high power CW tone transmission may have multiple IM product with the LO leakage and most likely some power backoff would be needed.

For transmitter requirement relating to CW transmission, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and the intermodulation requirement should be applied.

More study is needed for MPR/A-MPR for CW transmission.

RAN4 can discuss whether MPR/A-MPR simulation can be done in work item phase instead.

MPR/A-MPR discussion can be postponed to work item phase.

UE can transmit higher PSD signal as UE has a power control function for UL in NR.  In this sense, UE as reader may needs less transient time when switching from R2D signal to CW signal. Though the CW signal bandwidth needs to be defined differently for BS, it may not be so for UE as reader.  However, as this is study phase and RF architecture to transmit CW signal for UE as reader is not discussed yet, we think it may be good to keep it open for transient time needed between R2D and CW for UE as reader.

Transient time between R2D to CW transmission can be postponed to work item phase.

A new timing mask could be specified to test R2D and CW transmission for outpower dynamic for UE as reader that can transmit CW signal.

Another issue is unwanted emission generated by the phase noise and the transmitter non-linearity, the unwanted emission level may appear higher than the backscatter signal at the antenna. This is illustrated in Figure 1. The phase noise profile in TR 38.803 is used and it can be observed that around -50 dBm at the frequency offset of the 300kHz. If the reader REFSENS is below -70 dBm, such level of emission will drown the backscatter signal. If reader could equip with CW cancellation with a local CW copy, the unwanted emission caused by phase noise may be attenuated also. To enable the CW cancellation function, reader needs some time to train the circuit to generate the reverse CW leakage with the amplitude and negative phase.

Figure 1: the CW transmission with phase noise

For receiver requirement, as the coexisting scenario have both inside and outside topology, it should discuss whether two different set of the RF requirements should be defined corresponding to the either inside topology or outside topology. For inside topology, the receiver needs to tolerate the high leakage CW power either from the transmitter if UE as reader can transmit CW signal. For outside topology, CW transmission is far away from the UE as reader (D2T2-B), so the received CW power would be less issue for receiver to decode the backscattered signal and linearity requirement on receiver is relaxed. In our companion discussion paper for CW cancellation, the additional CW cancellation circuit would be expected if the linearity of the receiver would be compromised. Such would not be needed for outside topology where the leakage power from CW transmission is in par or smaller than backscattered signal. These considerations motivates different receiver architecture depending whether CW inside topology or outside topology is deployed.

Define two sets of the A-IoT UE as reader receiver requirement corresponding to inside topology and outside topology respectively.

The benefit of the defining two sets of receiver requirement can be the reader range. For the REFSENS designed for inside topology, the SNR degradation would be allowed as the high leakage power desensitize the receiver. For the REFSENS designed for outside topology, the need of further relaxing the REFSENS may not be needed therefore larger Reader to device range could be achieved. There are more factors to consider for REFSENS discussion, for example, the energy harvesting sensitivity of device 1 may limit the R2D distance and the min received power may be decided by R2D distance. Such discussion should be in the context of the link budget. Another example, the coexisting may set the limit on the cell range and therefore, the reader to device distance further be limited. For these factors to impact the REFSENS definition, RAN4 can identity these factors in study phase and further address it in work item phase to define the REFSENS.

Two different sets of receiver requirement may target to different reader to device range.

Both link budget and coexisting impact the REFSENS definition.

REFSENS definition depend on link budget analysis and coexisting scenario conclusion.

The REFSENS defined for inside topology should be tested with a CW signal.

REFSENS for inside topology should be tested with the CW signal.

- For the receiver requirements defined for outside topology, it can be further discussed whether there is no need to test it with a CW signal.

Further discuss if there is a need to test the REFSENS with CW signal or not.

- For ACS and inbound block requirements, it depends on the coexisting study. It may be necessary to discuss whether the UE as reader only operate for A-IoT RAT but not the other RAT at the same time. This is because when transmitting the CW signal while receiving the backscattered signal, the maximum input power requirement for legacy RAT cannot be met. We suggest no other RAT would be allowed when UE is operating as a reader, otherwise, the will be more impact for legacy requirement especially the strong leakage of CW may desensitize other RAT receiving.

Simultaneous other RAT and A-IoT operation is not allowed for UE as reader.

TP starts here:

### 6.7.3	Intermediate node (UE)

- Transmitter power

For the intermediate node that has capability to transmit CW signal, the MPR/A-MPR needs to be studied as the CW waveform defined with unmodulated sinusoid waveform is new waveform.

- Output power dynamics

For the intermediate node that has capability to transmit CW signal, the transient time between R2D to CW transmission needs to be further discussed.

Reference sensitivity

For inside topology, the receiver needs to tolerate the high leakage CW power from the transmitter when intermediate node can transmit CW signal and desensitization of receiver may be allowed with a CW cancellation function enabled in such case. For outside topology, the received CW power would be less and in this case, receiver to decode the backscattered signal without desensitization of receiver caused by CW leakage. Therefore, for REFSENS requirements, two sets of requirements can be defined, corresponding to inside topology and outside topology respectively.

# Conclusions

In this contribution, we present our overview on the RF aspects for UE as reader with below proposal and observations:

Proposal-1: The UE as reader can transmit CW transmission or not depending its capability

Proposal-2:No need to discuss the A-IoT R2D transmitter requirement but depending on the LP-WUS work item conclusion.

Observation 1 CW is unmodulated sinusoid signal and can be one or two tones with FFS frequency gap.

Observation 2 CW transmission’s timing, power, bandwidth, spectrum is controlled.

Observation 3 CW RF characteristic can be studied in RAN4 according to the RAN1 and RAN agreements.

Observation 4 CW and R2D transmission should be TDM to prevent the CW interfering with the R2D reception at device.

Observation 5 CW and R2D transmission requirement can be discussed separately.

Proposal-3: For transmitter requirement relating to CW transmission, the transmitted power, the ON/OFF power, the transmitter signal quality (frequency error requirement), unwanted emission and the intermodulation requirement should be applied.

Proposal-4: More study is needed for MPR/A-MPR for CW transmission.

Proposal-5: MPR/A-MPR discussion can be postponed to work item phase.

Proposal-6: Transient time between R2D to CW transmission can be postponed to work item phase.

Proposal-7: A new timing mask could be specified to test R2D and CW transmission for outpower dynamic for UE as reader that can transmit CW signal.

Proposal-8: Define two sets of the A-IoT UE as reader receiver requirement corresponding to inside topology and outside topology respectively.

Observation 6 Two different sets of receiver requirement may target to different reader to device range.

Observation 7 Both link budget and coexisting impact the REFSENS definition.

Proposal-9: REFSENS definition depend on link budget analysis and coexisting scenario conclusion.

Proposal-10: REFSENS for inside topology should be tested with the CW signal.

Proposal-11:Further discuss if there is a need to test the REFSENS with CW signal or not.

Proposal-12: Simultaneous other RAT and A-IoT operation is not allowed for UE as reader

# References

- R4-2410597, WF on impacts of A-IoT on RF requirements, Huawei

- RP-234058, New SID: Study on solutions for Ambient IoT (Internet of Things) in NR