# 3GPP TSG-RAN WG4 Meeting # 113						R4-2419494

# Orlando, US,18th–22nd November 2024

Source:		Ericsson

Title:			RedCap UE RF impact

Agenda item:		7.26.2.1

Document for:		Approval

# Introduction

In this paper, we present our views on the RedCap UE RF remaining issue.

# Discussion

In WF[1], the remaining issue is below:

## Issue 1-1: PC3 output power for HD-FDD (e)RedCap

Agreement: Nominal power for PC3 is 23 dBm.

Last meeting, it is agreed that nominal power of a PC3 should not be changed for HD-FDD (e)RedCap UE. Some UE vendor claim that there could be 0.8 dB gain on Tx power resulting from transmitter/receiver filter insert loss reduction. As this is NTN network, it is a benefit on the network performance when UE transmit higher power and it is worth to continue to discuss how to enable this in RAN4. Such power increase is not because of PVT and product spread and therefore, tolerance should not be used but we think it can be classified as “power boosting” domain. In Rel-18, the power boosting framework is done for MPR reduction work. Though the power boosting is for MPR reduction, it could be extended to cover other aspect for power boosting. Current Pcmax configuration formular is below:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

Pcmax configuration could be applied to (e) RedCap UE

ΔPPowerBoost is defined as  [0.8] dB for power class 3, when all of the following conditions are met:

-	If the (e)RedCap UE indicates support for UE capability [powerBoosting-RedCap-HD-FDD-r19] , and if IE [powerBoostRedCapHDFDD r19] is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

There is also notes in Table 6.2.2-1 in TS 38.101-1 and therefore, the same study should further needed to investigate if the MPR would be impacted by such power boosting.

Introduce the power boosting feature for HD-FDD (e) Redcap UE

Further discuss whether MPR would be impacted when reference power level increase to [ΔPPowerBoost - ΔPPowerClass]

## Issue 1-2: HD-FDD refsens for 2 Rx

Last meeting, the n256 REFSENS is agreed but not for n255. The issue is that some companies want to reuse the TN scaling factor for n255 but this will result in no tightening as the REFSENS of 5MHz equals to -100 dBm.

Option 1a: per band exception and selected band for different scaling factor as below:

The HD-FDD 5MHz REFSENS tightening from FD-FDD is proposed as in the table below.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

As this is NTN network and in UE normal operation, a low DL signal power will be received so it will be advantage on the network performance if RAN4 can tighten the n255 REFSENS with 0.5 dB.

Tightening the n255 REFSENS for HD-FDD with 0.5 dB.

## Issue 1-3: Simultaneous operation with GNSS

Open items:

- Strive towards common solution for (e)RedCap NR NTN and non-RedCap NR NTN

- Consider asking RAN1/2 to ensure IDC framework allows appropriate configurations to ensure GNSS location acquisition and NR NTN can operate in parallel in bands with UL close to GNSS frequencies

- Encourage further evaluations in RAN4 including but not limited to

  - Whether potential new solution(s) apply only from rel-19 or are early implementable

  - Whether UE speed needs to be considered

The work item of Rel-18 IDC enhancements targets at resolving the UE’s internal interference (i.e. from the UE’s Tx to the UE’s Rx) between NR and other RATs (e.g. WLAN, BlueTooth, UWB and so on), by using FDM solution, TDM solution and autonomous denial solution.

In our understanding, the IDC capability was introduced in Rel-18 and therefore, such feature should be available to NR NTN and NR RedCap UE when UE support it.  The IDC related UE capabilities is listed below.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| inDeviceCoexInd-r16 Indicates whether the UE supports reporting of affected NR carrier frequencies in IDC assistance information as specified in TS 38.331 [9]. | UE | No | No | No |
| inDeviceCoexIndAutonomousDenial-r18 Indicates whether the UE supports IDC autonomous denial as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| inDeviceCoexIndFDM-r18 Indicates whether the UE supports reporting of affected NR carrier frequency ranges in IDC assistance information as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |
| inDeviceCoexIndTDM-r18 Indicates whether the UE supports reporting of IDC TDM assistance information as specified in TS 38.331 [9]. A UE supporting this feature shall also support inDeviceCoexInd-r16. | UE | No | No | No |

The IDC capabilities is introduced in Rel-16 and IDC FDM/TDM in rel-18.

To solve the same frequency interference issue for GNSS and NR NTN, the IDC TDM or FDM solution could be configured. As the IDC TDM/FDM solution is introduced in NR Rel-18, the early released NR NTN in Rel-17 cannot report this capability and indicate to network the needs of the IDC configuration. Therefore, for early release NR NTN UE, it is implementation specific to solve this simultaneous operation of GNSS and NR NTN.

There is no IDC solution for Rel-17 NR NTN UE.

NTN NR UE and NTN (e)Redcap UE should support inDeviceCoexIndTDM-r18 and/or  inDeviceCoexIndFDM-r18 for the band in which  GNSS and NR NTN cannot simultaneously operate.

In IoT NTN, the GNSS measurement gap is introduced in Rel-18. The GNSS gap can be trigged by network or autonomously performed by UE, a short description of the GNSS measurement gap function is quoted below[2]. The IoT NTN UE supporting GNSS measurement gap feature are not expected to have the simultaneous GNSS and NR operation and therefore, UE configured with GNSS gap can be performed independently from NR UL transmission. The GNSS fix may takes longer time in the order of seconds and therefore, GNSS measurement gap is efficient solution for IoT devices.  The RedCap UE is target to the small factor devices and therefore, the power saving aspect should be also prioritized for the NR NTN RedCap. Therefore, there is also a benefit on power saving if GNSS measurement gap feature could be supported for NR NTN RedCap UE.

As currently GNSS measurement gap feature is only specified for IoT NTN UE and to extend the feature to NR NTN RedCap UE needs the WID updates. RAN4 can collect companies views on this and if consensus reached, WID updates could be done in December plenary.

Collect companies view on the introduction of the GNSS measurement gap feature to NR NTN RedCap UE.

Improved GNSS operations: During long connection times and for reducing GNSS measurements impact on UE power consumption, a UE supporting improved GNSS operations can be triggered to perform or configured to autonomously perform its GNSS acquisition If ul-TransmissionExtensionEnabled is configured, a time duration starts at the end of GNSS validity duration and a UE can perform autonomously GNSS acquisition using an autonomous gap starting at the end of the time duration, or the UE can be triggered to perform GNSS acquisition before the end of the time duration. This allows a UE in RRC_CONNECTED to apply UE pre-compensation of satellite delay and Doppler to maintain UL synchronization for UL transmission in an NTN cell. While network can trigger connected UE to perform GNSS measurement using DL MAC CE, UE autonomous GNSS-triggering measurement will be configured by RRC dedicated signalling. If there is neither network trigger nor network configuration of UE autonomously GNSS measurement, UE moves to RRC_IDLE after GNSS becomes invalid. UE will report the GNSS validity duration by using a UL MAC CE and assumes the GNSS location is valid upon successful GNSS measurement. Network enables this reporting of GNSS position fix duration, in SIB2 and in dedicated signalling (for handover).

In WID[3], there is no 38.307 as one of the impacted specification so the release independent aspect for RedCap needs to be discussed.

In 38.307, the additional operating band for RedCap is added as release independent feature for TN. As the NTN feature is separate clause in 38.307, the additional operating band for RedCap for NTN should be added under the NTN feature. For example:

# Release independent features for NR UE supporting satellite access operation

## 9.1	Additional NR operating bands for NR NTN in frequency range 1

Requirements for NR UE supporting satellite access operation, for additional NTN operating bands of TS 38.101-5 in Rel-P [8] are introduced via this clause.

Table 9.1-1: Additional NR operating bands for NR NTN in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- |
| NTN operating bands | FDD | Rel-17 | Table F.1.1-1, Table F.1.2-1, Table F.1.3-1 |

Table 9.1-2: NR NTN UE power class in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the power class was introduced) |
| --- | --- | --- | --- |
| Power Class 3 | FDD | Rel-17 | Table F.1.1-1 |

Table 9.1-3: NR NTN UE channel bandwidth in FR1-NTN

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the UE channel bandwidth was introduced) |
| --- | --- | --- | --- |
| UE channel bandwidth | FDD | Rel-17 | Table F.1.1-1 |

RAN1 agrees to introduce specfication change on the RedCap on HD-FDD operation, such agreement are listed below:

Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary

For any possible enhancement to the TA report, it is for further study, and it was captured as follows:

Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied

As the scope of the RAN1 scope is HD-FDD, in the release independent discussion, it can be discussed whether the HD-FDD should be release independent from Rel-19 and other duplex mode should be release independent from the first release, e.g FDD and TDD from Rel-17, HD-FDD from Rel-19.

## 9.2 Additional operating bands for Redcap and eRedCap for NR frequency range 1

Requirements for a Redcap and eRedcap UE for additional operating bands within FR1 of TS 38.101-5 in Rel-P [2] are introduced via this clause.

Table 9.2 -1: NR NTN RedCap operating band

| Feature | Duplex-mode | Release independent from |  | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- | --- |
| Redcap operating band | FDD | Rel-17 |  |  |
|  | TDD | Rel-17 |  |  |
|  | HD-FDD | Rel-19 |  |  |

Table 9.2 -2: NR NTN eRedCap operating band

| Feature | Duplex-mode | Release independent from |  | Requirements to be fulfilled (see TS 38.307 of the release in which the band was introduced) |
| --- | --- | --- | --- | --- |
| eRedcap operating band | FDD | Rel-18 |  |  |
|  | TDD | Rel-18 |  |  |
|  | HD-FDD | Rel-19 |  |  |

Though it could be argued if the 38.307 should be updated when additional new band is added, this indeed cause confusion for RedCap release independent feature update in 38.307 in later release. We suggest RAN4 discuss this and see if the 38.307 should be added in a updated WID.

RAN4 discuss if the 38.307 should be updated for RedCap NTN as release independent feature (operating band).

In this contribution, we present our view on the remaining issue for  Redcap UE RF:

Proposal-1: Introduce the power boosting feature for HD-FDD (e) Redcap UE

Proposal-2: Further discuss whether MPR would be impacted when reference power level increase to [ΔPPowerBoost - ΔPPowerClass]

Proposal-3: Tightening the n255 REFSENS for HD-FDD with 0.5 dB.

Observation 1 The IDC capabilities is introduced in Rel-16 and IDC FDM/TDM in rel-18.

Observation 2 There is no IDC solution for Rel-17 NR NTN UE.

Proposal-4: NTN NR UE and NTN (e)Redcap UE should support inDeviceCoexIndTDM-r18 and/or  inDeviceCoexIndFDM-r18 for the band in which  GNSS and NR NTN cannot simultaneously operate.

Proposal-5: Collect companies view on the introduction of the GNSS measurement gap feature to NR NTN RedCap UE.Collect companies view on the introduction of the GNSS measurement gap feature to NR NTN RedCap UE.

Proposal-6: RAN4 discuss if the 38.307 should be updated for RedCap NTN as release independent feature (operating band).

# References

- R4-2416586, Way Forward for [112][311] NR_NTN_Ph3_UE_RF, Qualcomm Inc

- TR 21.918

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3 , Huawei