3GPP TSG-RAN WG4 Meeting #113	R4-2419527

# Orlando, United States, 18 – 22 November 2024

Agenda item:		6.20.2

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for PC2 and 40 MHz

Document for:		Approval

## 1	Introduction

Requirements for PC2 for n28 were discussed lengthily in rel-18, but eventually postponed to rel-19. New WI including also 40 MHz CBW was approved in [1]. This document discussed UE RF requirements, many of which can be leveraged from rel-18 work, as well as new requirements for 40 MHz CBW.

## 2	Discussion

UE-to-UE coexistence requirements

There are multiple stringent UE-to-UE coexistence requirements for n28, as shown in Table 7.

Table 7: n28 UE coexistence requirements

| n28, n83 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 74, 75, 76 NR Band n77, n78, n100, n101 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 39, 40, 41, 52, 72, 73 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |

Meeting these requirements without any A-MPR is more difficult with wider channel bandwidth, as the emission region is relative terms closer to the transmission bandwidth when the channel bandwidth gets wider. In previous meeting we asked for further time to check these, and we can now confirm no additional accommodations are needed.

Proposal 1: No specification update to related to UE-to-UE coex requirements is needed

Center-frequency restrictions when UE supports 40 MHz CBW

Current gNB specifications already support 40 MHz CBW at the special raster point of 723.04 MHz in uplink. This point enables using 30 MHz CBW RB aligned within 40 MHz CBW and meeting minimum guard band, given that 30 MHz CBW minimum guard band is 40 kHz wider than 40 MHz CBW minimum guard band. It would be reasonable that also UE can be configured to the same center frequency to keep the smooth co-existence of different channel bandwidths.

If some deployments use only 40 MHz channel bandwidths, it would be possible to also place 40 MHz channel bandwidth immediately adjacent to low band edge. Given that the enhanced channel raster with 10 kHz channel step size is mandatory for n28 starting from rel-18, it would be possible to enable 40 MHz CBW channel raster points from 723.00 to 723.04 MHz. Given that all regions where 40 MHz may be possible may not have the flexibility to adjust the raster point by 40 kHz, it would be beneficial to include at least 723.00 MHz in the raster.

Proposal 2: Enable channel raster points for 40 MHz channel bandwidth from 723.00 MHz to 723.04 MHz unless.

When UE supports 40 MHz channel bandwidth, it has an RF filter with at least 40 MHz passband. Therefore, narrower channel bandwidths can be flexible positioned within this passband, as they do not have the restriction from 2x30 MHz duplexer. To enable more flexibility to network operations, it is proposed to enable flexibility to narrower channel bandwidths within the 40 MHz channel bandwidth. The changes to needed related table notes were tentatively agreed in [2], and now these changes can be confirmed. Additionally, RAN4 should consider whether the both agreed notes are merged together to a single note.

Proposal 3: For UEs supporting 40 MHz channel bandwidth, narrower channel bandwidths can be freely placed within the 40 MHz channel bandwidth as long as they are fully contained within 703 to 743.04 MHz.

Proposal 4: Confirm the wording on the notes in previous way forward and further consider merging them together under a single note.

## 3	Conclusion

In this contributions RF requirements for PC2 operation and 40 MHz CBW for n28 were discussed. Following observations and proposals were made:

Proposal 1: No specification update to related to UE-to-UE coex requirements is needed

Proposal 2: Enable channel raster points for 40 MHz channel bandwidth from 723.00 MHz to 723.04 MHz unless.

Proposal 3: For UEs supporting 40 MHz channel bandwidth, narrower channel bandwidths can be freely placed within the 40 MHz channel bandwidth as long as they are fully contained within 703 to 743.04 MHz.

Proposal 4: Confirm the wording on the notes in previous way forward and further consider merging them together under a single note.

## 4	References

[1]  RP-241660, New WID: Introduction of Power Class 2 and UE 40MHz Channel Bandwidth in NR band n28, CMCC CBN

[2]  R4-2417082, PC2 and 40MHz CBW for n28, CMCC