3GPP TSG-RAN WG4 Meeting #113	R4-2419528

# Orlando, United States, 18 – 22 November 2024

Agenda item:		6.17.2

Source: 		Qualcomm Inc.

Title: 		Co-existence considerations for the new S-band

Document for:		Discussion

## 1	Introduction

The WI on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) can be found in [1]. The objectives include co-existence analysis, for example UE-UE co-existence, for adjacent band per RAN4 scope. In this contribution these co-existence aspects are discussed.

## 2	Discussion

In previous meeting a WF was agreed [2]. One of the key agreements captured in the WF is on UE-to-UE coexistence between n252 UL and n2/b2/n25/b25 DL:

As seen, the target is now to define a requirement that allows the NTN service to work effectively across the full n252 band, whilst still protecting the TN UE at least in the majority of cases.

Meeting the link budget is essential for the NTN service to work effectively and therefore we did some initial analysis for expected A-MPR. Two emission limits were chosen as candidates to limit the total simulation effort and to be able to show a trend of how the A-MPR would change. The candidate values we used were -50 dBm/MHz and -30 dBm/MHz. Otherwise, the simulation assumptions were

- PA calibrated to 30 dB ACLR with 100 RB DFT-s-OFDM QPSK waveform

- -28 dBc IQ-image and DC-leakage

- -60 dBc CIM3

- Only DFT-s-OFDM QPSK waveform was considered.

- Emission limit applied below 1995 MHz

Focus was on narrow allocations, generally wide allocations (10 RB or more) might need more A-MPR.

The observations for different channel positions and carrier bandwidths are summarized in Table 1.

Table 1: Initial views on A-MPR for -50 dBm/MHz and -30 dBm/MHz limits below 1995 MHz

| CBW [MHz] | Carrier frequency [MHz] | -50 dBm/MHz limit | -30 dBm/MHz limit |
| --- | --- | --- | --- |
| 10 | 2005 | - For narrow allocations below RBstart = 8 and above RBstart = 43, up to 10 dB A-MPR needed - For narrow CIM5 allocations with RB start 8 to 14, approximately 4 dB A-MPR - No A-MPR needed for small set of narrow RB allocations with RBstart 15 to 43 | - For narrow allocations A-MPR is only needed at low edge of the channel below RBstart=8, the A-MPR is approximately 4 dB - Other narrow allocations above RBstart=8 need no A-MPR |
| 10 | 2010 | - For narrow allocations A-MPR is only needed at low edge of the channel below RBstart=9 the A-MPR is approximately 4 dB - Other narrow allocations above RBstart=9 need no A-MPR | - No A-MPR need was observed for narrow allocations |
| 10 | 2015 | - For narrow allocations A-MPR is only needed at low edge of channel, at RBstart=3 or below that. A-MPR is approximately 4 dB | - No A-MPR need was observed for narrow allocations |
| 5 | 2002.5 | - Approx 4 dB A-MPR needed for narrow allocations at RBstart=3 and below | - No A-MPR needed for narrow allocations |
| 5 | 2007.5 | - No A-MPR needed for narrow allocations | - No A-MPR needed for narrow allocations |

Some observations can be made from the data. Firstly, if the emission requirement is set at -50 dBm/MHz, A-MPR will be needed both for 10 MHz and 5 MHz CBW at low edge of the band. For 10 MHz CBW some A-MPR is needed also at the top end of the band. If operator has flexibility on the channel bandwidth arrangement within the band, then it is possible to reduce the number of impacted allocations by placing narrower CBW closer to the emission limit, but this does not completely remove the need for A-MPR.

Observation 1: -50 dBm/MHz emission limit below 1995 MHz will result in A-MPR, and depending on the channel bandwidth arrangement the A-MPR can be large, around 10 dB, effectively making some RB allocations unusable. Some A-MPR is needed for 10 MHz CBW even when placed at the top end of n252.

If the emission limit is set to -30 dBm/MHz, A-MPR is needed only for some RB allocations in 10 MHz CBW, and only in case 10 MHz CBW is placed at the low end of n252.

Observation 2: -30 dBm/MHz emission limit below 1995 MHz will result in A-MPR only for some RB allocations and only in 10 MHz CBW. This happens only when 10 MHz CBW is placed at the low end of n252.

Generally, -30 dBm/MHz emission limit more closely matches the WF on allowing NTN service to work effectively across the full n252 band.

Based on the deterministic analysis provided in previous meetings, approximately 10 meter UE-to-UE separation distance with -30 dBm/MHz limit corresponds to -50 dBm/MHz limit and 1 meter UE-to-UE separation.

Observation 3: A relaxed emission limit allows more flexibility for NTN UL operation. Approximately 10 meter UE-to-UE separation distance with -30 dBm/MHz limit corresponds to -50 dBm/MHz limit and 1 meter UE-to-UE separation.

## 3	Conclusion

In this contribution co-existence between new NTN S-band and adjacent 3GPP bands was discussed. Following observations were made.

Observation 1: -50 dBm/MHz emission limit below 1995 MHz will result in A-MPR, and depending on the channel bandwidth arrangement the A-MPR can be large, around 10 dB, effectively making some RB allocations unusable. Some A-MPR is needed for 10 MHz CBW even when placed at the top end of n252.

Observation 2: -30 dBm/MHz emission limit below 1995 MHz will result in A-MPR only for some RB allocations and only in 10 MHz CBW. This happens only when 10 MHz CBW is placed at the low end of n252.

Observation 3: A relaxed emission limit allows more flexibility for NTN UL operation. Approximately 10 meter UE-to-UE separation distance with -30 dBm/MHz limit corresponds to -50 dBm/MHz limit and 1 meter UE-to-UE separation.

## 4	References

- RP-241885, Revised WID on Introduction of another NR NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL), Qualcomm Incorporated

- R4-2417206, WF on NTN and IoT NTN bands, Moderator (Inmarsat)