3GPP TSG-RAN WG4 Meeting #113	R4-2419531

# Orlando, United States, 18 – 22 November 2024

Agenda item:		7.8.3.2

Source: 		Qualcomm Inc.

Title: 		UE RF requirements for NTN less than 5 MHz

Document for:		Approval

## 1	Introduction

3 MHz channel bandwidth was specified for TN in rel-18. In the work item approved for NTN, 3 MHz channel bandwidth is to be added with RAN1-design based on rel-18, with no expected RAN1 impact. In this contribution UE RF requirements for 3 MHz channel bandwidth are discussed.

## 2	Discussion

The agreements reached in RAN4#112bis are captured in [1]. This document addresses A-MPR, reference sensitivity and optional vs. mandatory support for 3 MHz CBW.

A-MPR

A-MPR evaluations were done with following assumption in simulations

- PA calibrated to meet 30 dBc ACLR 1 dB MPR in 20 MHz channel and 100 RB DFT-s-OFDM signal

- DC-leakage and IQ-image set to -28 dBc

- CIM3 of -60 dBc

NS_02N

NS_02N is specified for n255 with stringent emission requirements below 1610 MHz. No A-MPR is allowed for wider channel bandwidths. With 3 MHz channel bandwidth the emissions will be contained closer to the transmission, and no A-MPR is needed also for 3 MHz.

Proposal 1: No A-MPR is needed for 3 MHz channel bandwidth with NS_02N

NS_03N

NS_03N is specified for n254 for operation under FCC regulatory domain. n254 UL is immediately adjacent to the emission limit, and therefore meeting the requirement is hard. A-MPR is specified for 5 MHz and wider channel bandwidths.

Based on the evaluations small A-MPR is needed to meet the requirements. The new requirement is highlighted in yellow in below tables.

Table 1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3 MHz | fc < 1613.9 |  | >= 1.8 | A3 |
| 5MHz | 1612.5 <= fc < 1613.9 | <= 0.36 | <= 0.36 | A1 |
|  |  |  | >= 2.88 | A2 |
|  | 1613.9 <= fc < 1615.7 |  | >= 3.24 | A3 |
| 10MHz | 1615 <= fc < 1620.1 | <= 1.8 | <= 5.04 | A4 |
|  |  | <= 1.8 | > 5.04 | A5 |
|  |  | > 7.2 | > 0 | A6 |
|  |  | > 1.8 | >= 2.88 | A2 |
|  | 1620.1 <= fc < 1621.5 |  | <= 6.48 | A6 |
|  |  | <= 0.36 | <= 0.36 | A1 |
|  | fc = 1621.5 |  | >= 7.2 | A1 |
| 15MHz | all | <= 3.6 | <= 5.04 | A4 |
|  |  | <= 3.6 | > 5.04 | A5 |
|  |  | > 10.44 |  | A6 |
|  |  | > 3.6 | >= 4.32 | A2 |

Table 2: A-MPR for NS_03N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
|  | 256QAM | 4.5 | 6 | 4.5 | 8 | 9 | 4 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 256QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

Proposal 2: Adopt NS_03N A-MPR requirements as shown in Table 1 and Table 2

NS_04N

NS_04N is specified for n254 for operation in ETSI regulatory domain. For NS_04 there is a both additional spectrum emission mask as well as additional spurious emission limits both at low and high edge of the operating band. The spurious emission limit above the band is not limiting output power for NS_04N, as NS_04N is intended to be valid only when carrier is contained within 1610 – 1618.25 MHz. Based on our studies the additional SEM mask is not limiting the output power. Therefore, the required A-MPR for 3 MHz is the same for NS_03N.

Proposal 3: NS_04N A-MPR requirements for 3 MHz CBW are the same as for NS_03N

NS_05N

NS_04N is specified for n254 for operation in ETSI regulatory domain. For NS_05 there is a both additional spectrum emission mask as well as additional spurious emission limits both at low and high edge of the operating band. The spurious emission limit below the band is not limiting output power for NS_05N, as NS_05N is intended to be valid only when carrier is within 1618.25 – 1626.5 MHz. Therefore, simulations were done at the top end of the band center frequency of 1625.0 MHz. No need for A-MPR was observed.

Proposal 4: No A-MPR is needed for NS_05N

NS_24

NS_24 is specified for n256 for protection of band 34. The emission limit is very stringent at -50 dBm/MHz for 2010 to 2025 MHz. The requirement applies only when there is at least 5 MHz offset from the upper edge of the channel bandwidth to the protected frequencies. Therefore, 2003.5 MHz was used as center frequency for the evaluation.

A-MPR results for NS_24 are presented in Table 3 and Table 4

Table 3: A-MPR for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 3 MHz | 1997.5 < Fc ≤ 2003.5 |  | ≥1.44 | A8 |  |  |  |  |  |  |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <3.78 |  | A5 | ≥3.78 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 4: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 | ≤ [3.5] |
| DFT-s-OFDM QPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 | ≤ [4] |
| DFT-s-OFDM 16 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 | ≤ [4.5] |
| DFT-s-OFDM 64 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 10 | ≤ 3.5 | ≤ [5] |
| DFT-s-OFDM 256 QAM | ≤ 11 | ≤ 5 |  | ≤ 8.5 | ≤ 20 | ≤ 10 |  | ≤ [5] |
| CP-OFDM QPSK | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 | ≤ [6] |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 | ≤ [6] |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 | ≤ [6] |
| CP-OFDM 256 QAM | ≤ 13 | ≤ 6.5 |  | ≤ 8.5 | ≤ 20 | ≤ 12 |  | ≤ [6.5] |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |

Proposal 5: Adopt A-MPR for NS_24 as presented in Table 3 and Table 4

Refsens

With 3 MHz channel bandwidth there are 15 resource blocks available and therefore total received power with same PSD is reduced by 2.2 dB compared to 5 MHz channel bandwidth. The reduced total power also means that any impairments have more impact to the signal quality. For NR NTN, maximum input power and in-band blocking interferer signal level are reduced to -40 dBm. This means that NR NTN may not be as well optimized for maximum linearity as receiver for TN bands.

For 3 MHz channel bandwidth, most LTE bands apply 1.7 dB lower power for 3 MHz channel bandwidth than for 5 MHz channel bandwidth. This is because Rx IIP2 effects become more impactful. Rx IIP2 needs to be considered in setting the wanted signal power level.

Observation 1: Rx IIP2 needs to be considered in setting wanted signal level for 3 MHz refsens.

Proposal 6: Define refsens for 3 MHz channel bandwidth as shown in Table 5:

Table 5: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -101.7 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

Optional vs. mandatory support for 3 MHz

NR NTN was originally defined in rel-17 when the narrowest channel bandwidth was 5 MHz. UEs supporting NR NTN based on the rel-17 specifications will not find NTN cells from the new sync raster nor will they support configuration of 3 MHz channel bandwidth. For these reasons, networks will need to operate with the assumption that e.g. initial access would take place with the legacy sync raster, as otherwise some UEs will not be able to connect. Additionally, support for 3 MHz does not add any critical functionality which would be essential for NR NTN system to work.

For these reasons, it is preferable to allow optional support for 3 MHz channel bandwidth like is typical for new features added in later releases. RAN2 has already defined capabilities how UE indicates support of 3 MHz channel bandwidth, and it is possible that no updates in RAN2 are needed.

Proposal 7: Support of 3 MHz channel bandwidth is optional for UE.

## 3	Conclusion

In this contribution UE RF requirements for less than 5 MHz for NTN were discussed. Following observations and proposals were made

Proposal 1: No A-MPR is needed for 3 MHz channel bandwidth with NS_02N

Proposal 2: Adopt NS_03N A-MPR requirements as shown in Table 1 and Table 2

Table 1: A-MPR regions for NS_03N

| Channel BW | Carrier Center Frequency | RB_start*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3 MHz | fc < 1613.9 |  | >= 1.8 | A3 |

Table 2: A-MPR for NS_03N

|  | Modulation | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | 2.5 | 3.0 | 1.0 | 4.0 | 6.5 | 1.5 |
|  | QPSK | 2.5 | 4.0 | 2.5 | 6.0 | 7.0 | 2.0 |
|  | 16QAM | 3.0 | 4.5 | 3.0 | 6.5 | 7.5 | 2.5 |
|  | 64QAM | 3.5 | 5 | 3.5 | 7 | 8 | 3 |
|  | 256QAM | 4.5 | 6 | 4.5 | 8 | 9 | 4 |
| CP-OFDM | QPSK | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 16QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 64QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |
|  | 256QAM | 3.5 | 6.0 | 4.0 | 8.0 | 10.0 | 4.0 |

Proposal 3: NS_04N A-MPR requirements for 3 MHz CBW are the same as for NS_03N

Proposal 4: No A-MPR is needed for NS_05N

Proposal 5: Adopt A-MPR for NS_24 as presented in Table 3 and Table 4

Table 3: A-MPR for NS_24

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 3 MHz | 1997.5 < Fc ≤ 2003.5 |  | ≥1.44 | A8 |  |  |  |  |  |  |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 4: A-MPR for modulation and waveform type

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer | Outer |
| DFT-s-OFDM PI/2 BPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 | ≤ [3.5] |
| DFT-s-OFDM QPSK | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 | ≤ [4] |
| DFT-s-OFDM 16 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 18 | ≤ 10 | ≤ 3.5 | ≤ [4.5] |
| DFT-s-OFDM 64 QAM | ≤ 11 | ≤ 5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 10 | ≤ 3.5 | ≤ [5] |
| DFT-s-OFDM 256 QAM | ≤ 11 | ≤ 5 |  | ≤ 8.5 | ≤ 20 | ≤ 10 |  | ≤ [5] |
| CP-OFDM QPSK | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 | ≤ [6] |
| CP-OFDM 16 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 | ≤ [6] |
| CP-OFDM 64 QAM | ≤ 13 | ≤ 6.5 | ≤ 4 | ≤ 8.5 | ≤ 19 | ≤ 12 | ≤ 5.5 | ≤ [6] |
| CP-OFDM 256 QAM | ≤ 13 | ≤ 6.5 |  | ≤ 8.5 | ≤ 20 | ≤ 12 |  | ≤ [6.5] |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |

Observation 1: Rx IIP2 needs to be considered in setting wanted signal level for 3 MHz refsens.

Proposal 6: Define refsens for 3 MHz channel bandwidth as shown in Table 5:

Table 5: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -101.7 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -101.2 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 7: Support of 3 MHz channel bandwidth is optional for UE.

## 4	References

[1]  R4-2416587, Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi