3GPP TSG-RAN WG4 Meeting #113	R4-2419541

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.26.2.1

Source:	MediaTek Inc.

Title:	Discussion on Rel-19 NR-NTN RedCap UE RF requirements

Document for:	Discussion

- Introduction

In RAN#102 plenary meeting, the Rel-19 WID of Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in [1]. One of the main objectives is to support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN band.

In RAN4#111 and 112bis meetings, the WF [2-3] for guiding further discussion on residual issues were proposed.

In this contribution, our views about the residual issues are provided.

- Discussion

2.1		Guidance for Rel-19 phase 3 NR-NTN UE RF requirements

The Rel-19 WID [1] objectives indicate that NTN for NR Phase 3 supports Redcap and eRedcap with NR NTN operating in FR1-NTN band. The objectives are shown below.

4.1 Objective of SI or Core part WI or Testing part WI Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

The Rel-19 NR_NTN_Ph3_UERF WF [2] in RAN4#111 provided the guidance as follows.

Issue 1-2: HD-FDD refsens for 2 Rx Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement.

Issue 1-5: Maximum output power options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

The Rel-19 NR_NTN_Ph3_UERF WF [3] in RAN4#112bis provided the guidance as follows.

Issue 1-1: PC3 output power for HD-FDD (e)RedCap Agreement: Nominal power for PC3 is 23 dBm. Issue 1-2: HD-FDD refsens for 2 Rx Agreement: Agree n256 Refsens as below. Operating Band; SCS (kHz); 5 MHz (dBm); 10 MHz (dBm); 15 MHz (dBm); 20 MHz (dBm). 15; -100.0; -96.8; -95.0; -93.8. n256; 30; -97.1; -95.1; -94.0. 60; -97.5; -95.4; -94.2. Issue 1-3: Simultaneous operation with GNSS Open items: - Strive towards common solution for (e)RedCap NR NTN and non-RedCap NR NTN - Consider asking RAN1/2 to ensure IDC framework allows appropriate configurations to ensure GNSS location acquisition and NR NTN can operate in parallel in bands with UL close to GNSS frequencies - Encourage further evaluations in RAN4 including but not limited to - Whether potential new solution(s) apply only from rel-19 or are early implementable - Whether UE speed needs to be considered

2.2		HD-FDD refsens for 2 Rx

Regarding HD-FDD REFSENS for 2 Rx, because NTN connection performance between SAN and UE is important, some comments indicate that it is desired to achieve more than 0.5 dB tightening on FR1 NTN bands. However, some companies prefer to keep the 0.5 dB tightening on band n256. Regarding n255 and n254, there are more arguments.

According to face-to face discussion in RAN4#111 to RAN4#112bis meetings, regarding NTN FR1bands, it is observed that 2RX HD-FDD REFSENS could be further optimized compared to 2RX FD-FDD RFESENS.

Observation 1: Regarding NTN HD-FDD REFSENS for 2Rx, because NTN connection performance between SAN and UE is important and not as good as TN connection performance, some comments indicate that it is desired to achieve more than 0.5 dB tightening for FR1 NR-NTN bands n255 and n254.

Observation 2: In TS 38.101-5, 2Rx FD-FDD REFSENS are -100dBm/5MHz and -99.5dBm/5MHz for NTN n255 and n254, respectively.

Observation 3: Switching from FD-FDD mode to HD-FDD mode would improve REFSENS due to lower front-end insertion loss.

Proposal 1: Regarding NTN band n255 2RX HD-FDD REFSENS, 0dB tightening seems not to optimize NTN REFSENS compared to FD-FDD operation. Consider two options below.

- Option 1: To take average for n255 2RX HD-FDD REFSENS based on companies (e.g., Apple, Qualcomm, Sony, Xiaomi, MTK, Nokia) contributions before R4#113. The average is -100.2dBm/5MHz.

- Option 2: Regarding n255 2RX HD-FDD REFSENS based on companies (e.g., Apple, Qualcomm, Sony, Xiaomi, MTK, Nokia, Inmarsat, Thales, OPPO) comments before R4#113, consider -100.4dBm/5MHz.

Proposal 2: Regarding NTN band n254 2RX HD-FDD REFSENS, 0dB tightening seems not to optimize NTN REFSENS compared to FD-FDD operation. Consider two options below.

- Option 1: To take average for n254 2RX HD-FDD REFSENS based on companies (e.g., Apple, Qualcomm, Sony, Xiaomi, MTK, Nokia) contributions before R4#113. The average is -100.1dBm/5MHz.

- Option 2: Regarding n254 2RX HD-FDD REFSENS based on companies (e.g., Apple, Qualcomm, Sony, Xiaomi, MTK, Nokia, Inmarsat, Thales, OPPO) comments before R4#113, consider -100.3dBm/5MHz.

2.3		Maximum output power

It is observed that the HD-FDD RF front-end insertion loss (FE IL) is lower compared to FD-FDD FE IL and can improve the RX REFSENS and TX maximum output power (MOP) simultaneously.

Observation 4: HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

From EC Decision 2016/687[4], it is observed that a tolerance of up to + 2 dB is to take account of operation under extreme environmental conditions and production spread. Regarding UE TX output power, the power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be mobile or nomadic.

Observation 5: From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

From ETSI EN 301 908[5], it is observed that PC3 upper tolerance is specified as +2.7dB or +3dB.

Observation 6: From ETSI EN 301 908, PC3 TX MOP tolerance upper bound can be specified higher (i.e., +2.7dB or +3dB).

It is observed that NTN antenna gain for smartphone is -5.5dBi and can be used for NTN-UE EIRP requirement.

Observation 7: NTN antenna gain for smartphone is -5.5dBi and can be used for NTN-UE EIRP requirement.

Observation 8: The actual NTN-RedCap PC3 HD-FDD Tx upper bound can be 25.8dBm (i.e., 23.8dBm+2dB) but exceeds conventional PC3 FD-FDD TX upper power bound of 25dBm.

Proposal 3: Regarding solution for NTN-RedCap PC3 HD-FDD TX output power upper bound higher than 25dBm, the options for solution are as follows. The solution is also applicable for solving the same issue when considering other higher power classes (e.g., PC2, PC1.5).

- Option 1: Add additional note to increase the upper tolerance for NTN-RedCap HD-FDD operation in applicable regions.

| - NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26 | +2x | 23 | +2x, -2 |
| n255 | 26 | +2x | 23 | +2x, -2 |
| n254 | 26 | +2x | 23 | +2x, -2 |
| NOTE X: When UEs recognizes and supports NTN-RedCap HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD]. |  |  |  |  |

- Option 2: Add additional UE capability and NW signal to control NTN-RedCap HD-FDD power increment.

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26 | +2x | 23 | +2x, -2 |
| n255 | 26 | +2x | 23 | +2x, -2 |
| n254 | 26 | +2x | 23 | +2x, -2 |
| NOTE X: When NTN-RedCap UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB. The reference power is increased by ΔPNTNHdFdd. |  |  |  |  |

- Option 3: Add new power classes (e.g., PC2.5_NTN-RedCap-HD-FDD, PC1.75_NTN-RedCap-HD-FDD).

3 Conclusion

In this paper, our views on NTN-RedCap UE RF requirements in HD-FDD mode are provided as follows:

Observation 1: Regarding NTN HD-FDD REFSENS for 2Rx, because NTN connection performance between SAN and UE is important and not as good as TN connection performance, some comments indicate that it is desired to achieve more than 0.5 dB tightening for FR1 NR-NTN bands n255 and n254.

Observation 2: In TS 38.101-5, 2Rx FD-FDD REFSENS are -100dBm/5MHz and -99.5dBm/5MHz for NTN n255 and n254, respectively.

Observation 3: Switching from FD-FDD mode to HD-FDD mode would improve REFSENS due to lower front-end insertion loss.

Proposal 1: Regarding NTN band n255 2RX HD-FDD REFSENS, 0dB tightening seems not to optimize NTN REFSENS compared to FD-FDD operation. Consider two options below.

- Option 1: To take average for n255 2RX HD-FDD REFSENS based on companies (e.g., Apple, Qualcomm, Sony, Xiaomi, MTK, Nokia) contributions before R4#113. The average is -100.2dBm/5MHz.

- Option 2: Regarding n255 2RX HD-FDD REFSENS based on companies (e.g., Apple, Qualcomm, Sony, Xiaomi, MTK, Nokia, Inmarsat, Thales, OPPO) comments before R4#113, consider -100.4dBm/5MHz.

Proposal 2: Regarding NTN band n254 2RX HD-FDD REFSENS, 0dB tightening seems not to optimize NTN REFSENS compared to FD-FDD operation. Consider two options below.

- Option 1: To take average for n254 2RX HD-FDD REFSENS based on companies (e.g., Apple, Qualcomm, Sony, Xiaomi, MTK, Nokia) contributions before R4#113. The average is -100.1dBm/5MHz.

- Option 2: Regarding n254 2RX HD-FDD REFSENS based on companies (e.g., Apple, Qualcomm, Sony, Xiaomi, MTK, Nokia, Inmarsat, Thales, OPPO) comments before R4#113, consider -100.3dBm/5MHz.

Observation 4: HD-FDD FE IL is obviously lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

Observation 5: From EC Decision 2016/687, at least a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

Observation 6: From ETSI EN 301 908, PC3 TX MOP tolerance upper bound can be specified higher (i.e., +2.7dB or +3dB).

Observation 7: NTN antenna gain for smartphone is -5.5dBi and can be used for NTN-UE EIRP requirement.

Observation 8: The actual NTN-RedCap PC3 HD-FDD Tx upper bound can be 25.8dBm (i.e., 23.8dBm+2dB) but exceeds conventional PC3 FD-FDD TX upper power bound of 25dBm.

Proposal 3: Regarding solution for NTN-RedCap PC3 HD-FDD TX output power upper bound higher than 25dBm, the options for solution are as follows. The solution is also applicable for solving the same issue when considering other higher power classes (e.g., PC2, PC1.5).

- Option 1: Add additional note to increase the upper tolerance for NTN-RedCap HD-FDD operation in applicable regions.

| - NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26 | +2x | 23 | +2x, -2 |
| n255 | 26 | +2x | 23 | +2x, -2 |
| n254 | 26 | +2x | 23 | +2x, -2 |
| NOTE X: When UEs recognizes and supports NTN-RedCap HD-FDD operation in XXX region, the upper tolerance requirement can be increased from +2dB to [+2.7dB or +TT-NTN-HDFDD]. |  |  |  |  |

- Option 2: Add additional UE capability and NW signal to control NTN-RedCap HD-FDD power increment.

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n256 | 26 | +2x | 23 | +2x, -2 |
| n255 | 26 | +2x | 23 | +2x, -2 |
| n254 | 26 | +2x | 23 | +2x, -2 |
| NOTE X: When NTN-RedCap UE operates in HD-FDD and indicates support for UE optional capability x1-r19 and if IE x2-r19 is set to 1. The MOP upper tolerance is further increased by ΔPNTNHdFdd from 2dB. The reference power is increased by ΔPNTNHdFdd. |  |  |  |  |

- Option 3: Add new power classes (e.g., PC2.5_NTN-RedCap-HD-FDD, PC1.75_NTN-RedCap-HD-FDD).

4 Annex

TS 38.101-5 Table 5.2.2-1: NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

TS 38.101-5 Table 7.3.2-1: Two antenna port reference sensitivity QPSK REFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |
|  | 30 |  | -96.6 | -94.6 | -94 |  |  |
|  | 60 |  | -97 | -94.9 | -94.2 |  |  |
| n255 | 15 | -100 | -96.8 | -95 | -93.8 |  |  |
|  | 30 |  | -97.1 | -95.1 | -94 |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |
| n254 | 15 | -99.5 | -96.3 | -94.5 |  |  |  |
|  | 30 |  | -96.6 | -94.6 |  |  |  |
|  | 60 |  | -97 | -94.9 |  |  |  |

EC Decision 2016/687

The power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be fixed or installed and as total radiated power (TRP) (2) for terminal stations designed to be mobile or nomadic.

Total radiated power (TRP) is a measure of how much power the antenna actually radiates. The TRP is defined as the integral of the power transmitted in different directions over the entire radiation sphere.

ETSI EN 301 908

4.2.2 	Transmitter Maximum Output Power

4.2.2.1 	Transmitter maximum output power for Single Carrier

4.2.2.1.1 	Definition

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub-frame (1 ms).

4.2.2.1.2 	Limits

The UE maximum output power shall be within the shown value in table 4.2.2.1.2-1.

Table 4.2.2.1.2-1: UE power classes

| E-UTRA Band | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| 1 | 23 | ±2,7 |
| 3 | 23 | ±2,7 (see note) |
| 7 | 23 | ±2,7 (see note) |
| 8 | 23 | ±2,7 (see note) |
| 20 | 23 | ±2,7 (see note) |
| 22 | 23 | +3,0/-4,5 |
| 28 | 23 | +2,7/-3,2 |
| 31 | 23 | ±2,7 |
| 33 | 23 | ±2,7 |
| 34 | 23 | ±2,7 |
| 38 | 23 | ±2,7 |
| 40 | 23 | ±2,7 |
| 42 | 23 | +3,0/-4,0 |
| 43 | 23 | +3,0/-4,0 |
| 65 | 23 | ±2,7 |
| 68 | 23 | ±2,7 |
| NOTE: For transmission bandwidths (ETSI TS 136 521-1 [1], clause 5) confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1,5 dB (tolerance = +2,7/-4,2). |  |  |

5 Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- R4-2410574, “WF on Rel-19 phase 3 NR-NTN UE RF requirements”, Qualcomm

- R4-2416586, “Way Forward for [112bis][310] NR_NTN_Ph3_UE_RF”, Qualcomm

- EC Decision 2016/687

- ETSI EN 301 908-13 V13.2.0 (2021-11)