3GPP TSG-RAN WG4 Meeting #113									                        R4- 2419548

Orlando, USA, Nov. 18 – 22, 2024

Agenda item:			7.10.2.2

Source:	Ericsson

Title:	Discussion on RF requirement of ATG UE inter-band CA

Document for:	Approval

# Introduction

In 3GPP release 18, RAN4 has completed the standardization work for the air-to-ground (ATG) network WI, the main work focused on the FR1 single carrier scenario. To enhance the throughput to meet growing ATG capacity demand and fully utilize operators’ spectrum, the RAN plenary meeting #103 has approved a new WI on enhancements for the air-to-ground network for NR [1].

# Discussion

In the last meeting, some agreements addressed the general concerns [2]. Part of the agreements regarding the inter-band are shown below:

Issue 2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- The conclusions about allowing n39 UL for CA_n3A-n39A if any should only apply to ATG and will not have any impact on existing NR specification for legacy NR UE.

- Further check if the performance of these parameters in the following table can be achieved in ATG UE with 5MHz frequency gap in n39 filter.

| - Parameters | Values |
| --- | --- |
| Antenna Isolation | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 50dB |
| n39 Tx filter rejection at 1805~1880MHz | 50dB |

- Other RF architectures or parameters are not precluded.

Issue 2-2: BCS for DL CA_n3_n39

WF:

Further consider the following table, if the feasibility is confirmed for uplink of n39.

Table 1.  Supported bandwidths per CA band combination of band n3+n39

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | n3, [n39] | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

Issue 2-3: MSD for CA_n3A-n39A with n39 UL

WF:

- Further check the MSD value with 5MHz frequency gap in 1880~1885MHz, if the feasibility is confirmed for uplink of n39.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1827.5 | 5 | [TBD] | ACLR2 |

- Other test points are not precluded.

Numerous unresolved matters remain that require attention, and the upcoming session will delve into the RF requirement for ATG UE inter-band CA.

  - Inter-band CA

### Whether to allow n39 UL for CA_n3A-n39A

Issue 3-2-1: whether to allow n39 UL for CA_n3A-n39A

Agreement:

- Whether to allow n39 UL for CA_n3A-n39A depend on companies’ input.

  - The filter with [X] MHz frequency gap can be considered.

The bands n3 and n39 are defined in the table below.

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz | FDD |
| n39 | 1880 MHz – 1920 MHz | 1880 MHz – 1920 MHz | TDD |

The CA_n3-n39 is an FDD and TDD inter-band combination, specifically, the upper bound of the n3 DL is next to the lower bound of the n39 without any gap in between. Therefore, the actual deployment scenarios/configuration would impact the potential interference. There was a discussion during the #110 meeting and a WF was reached as shown below [4]:

<Way forward 1>:

Single UL only in band n3 for DL CA_n3A-n39A is assumed.

Essentially, for the inter-band CA, PCell can operate either in FDD band or TDD band. To mitigate the potential interference, if only one UL transmission is assumed in band n3 for DL CA_n3-n39, then FDD band n3 could serve as the PCell and TDD band n39 serve as SCell while n39 is configured as DL to avoid interference with the n3 DL. It would be easier to adopt the following proposal to mitigate the potential interference:

- FDD band n3 serves as PCell and TDD band n39 could serve as SCell while n39 is configured as DL to avoid interference with the n3 DL.

However, it is worth noting that the above discussion was based on the handheld UE architecture. ATG UE is a different type of UE with a larger form factor. Therefore, the isolation is larger than that of handheld UE, which could make the n39 UL feasible for CA_n3A-n39A. This approach needs to be further verified by thoroughly studying the ATG UE architecture.

- The n39 UL may be feasible for CA_n3A-n39A due to its larger form factor and isolation than handheld UE.

- If the feasibility is confirmed, then the following table could be used for the CA_n3-n39 band combination.

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | n3, n39 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

# Conclusion

In the previous sections, we made the following observations:

Observation 1	The n39 UL may be feasible for CA_n3A-n39A due to its larger form factor and isolation than handheld UE.

Based on the discussion in the previous sections we propose the following:

Proposal 1	FDD band n3 serves as PCell and TDD band n39 could serve as SCell while n39 is configured as DL to avoid interference with the n3 DL.

Proposal 2	If the feasibility is confirmed, then the following table could be used for the CA_n3-n39 band combination.

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n39A | n3, n39 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n39 | 5, 10, 15, 20, 25, 30, 35, 40 |  |

# References

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2417121, WF on Rel-19 ATG UE RF requirements

- TS 38.101-1, V18.5.0

- R4-2403719, WF on CA_n3A-n39A MSD and architecture.