3GPP TSG-RAN WG4 Meeting #113									                        R4- 2419550

Orlando, USA, Nov. 18 – 22, 2024

Agenda item:			7.1.1.3.2

Source:	Ericsson

Title:	Discussion on the MIMO layers for 6Rx UE

Document for:	Approval

# Introduction

With advancements in hardware and design, modern smartphones increasingly integrate multiple Rx antennas to enhance DL performance. For instance, certain foldable mobile phones have adopted 6Rx antennas. Incorporating 6Rx facilitates a higher peak data rate by supporting more DL MIMO layers compared to 4Rx configurations. On the other hand, due to the higher diversity, it can also help increase the downlink coverage by improving the receiver sensitivity.

In 3GPP Release 18, RAN4 finalized the standardization efforts for enabling 8Rx for CPE/FWA/vehicle/industrial type of UEs. Although 6Rx was proposed and deliberated upon, it was given lower priority during the Rel-18 discussion. To enhance DL coverage and throughput for handheld UEs with larger sizes, RAN4 has approved a new WI to specify requirements for both handheld and FWA UEs [1].

6Rx for handheld and FWA UE

- Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

The following sections will discuss the objectives specified in the WI, focusing on the 6Rx MIMO layers.

# Support of MIMO layers

The last meeting reached a WF on the MIMO layers for 6Rx, and it mainly clarified the objectives that are not in the scope of the WI [2]:

- Sub-topic 3-1:	6-layer Support

Issue 3-1-1: 6-layer Support

Way Forward: RAN4 to further discuss the following options (options are not necessarily mutually exclusive).

-	Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios.

-	Option 2: 6-layer support is feasible for FWA devices.

-	Option 3: Companies to further discuss and align on the feasibility criteria and simulation assumptions by RAN4#113 to evaluate 6-Layer feasibility for handhelds using the following as a starting point.

-	Feasibility criteria

-	Consider 4-layer vs 6-layer throughput performance.

-	Simulation assumptions

-	Only consider link-level simulation for performance evaluations.

-	Updated simulation assumptions are provided in Annex A for discussions on alignment at RAN4#113 and can be used by companies to provide simulation results at RAN4#113 for discussion. Companies to clarify and justify (e.g., where the number comes from) the settings used in the simulation assumptions for BS correlation matrix and Tx EVM.

-	Option 4: 6-layer support is not feasible for handheld UEs.

-	Option 5: Companies to further discuss the following aspect of 6-layer support.

-	If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements.

-	If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only.

Issue 3-1-2: 6-layer Support as optional feature

Way Forward: RAN4 to further discuss if 6-Layer support should be considered an optional feature per the following.

-	Introduce 6 MIMO layers support as an optional feature, if 6-layer performance requirements are introduced for 6Rx.

Issue 3-1-3: Tx EVM for feasibility study

Way Forward: FFS on Tx EVM values to be used for feasibility evaluation. RAN4 to further discuss the following:

-	Option 1: Use Tx EVM = 1%

-	Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation

-	Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM

-	Option 4: use the existing BS Tx EVM core requirement

-	Option 5: The representative BS Tx EVM values provided by the BS vendor.

As we have pointed out in the introduction, modern smartphones increasingly integrate multiple Rx antennas to enhance DL performance. Numerous simulation results show that supporting a higher number of DL MIMO layers can improve the DL peak throughput.

The radio channel and several other factors impact the number of layers that can be supported, and these should be thoroughly studied and analyzed.

One factor is the receive antenna correlation. When antenna elements are closely spaced, the correlation between them increases.  In the previous 8Rx discussions, the objective is to define the requirement for FWA and CPE UEs. Given their larger form factor compared to the handheld UE, the demodulation performance study assumed a low antenna correlation [3]. In the 6Rx WI, the objective is to specify FWA and handheld UE requirements. Although the example bands specified in WI are mid-bands and the antenna size can be relatively compact, we understand the difficulty of integrating 6 antenna elements into the handheld deceive without any correlation.

We have accomplished some simulations based on the parameter assumptions listed in the WF. The simulation assumption is shown in Table 1, PDSCH performance with fixed rank is shown in Figure 1, and PDSCH performance with adaptive rank is shown in Figure 2.

Table 1 Simulation parameter assumption

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30 |
| Antenna configuration |  | 8Tx, 6Rx |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed rank [4,6] |
| MCS |  | Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx SVD based precoding for 32Tx; companies are encouraged to provide details on the SVD update periodicity |
| PDSCH DMRS configuration | DMRS Type | Type 1 [Rel-15] and eType-1 [Rel-18] |
|  | DMRS Configurations | Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) Option 2: Rel-15 1+1(4L), Rel-15 2+0(6L) Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) Option 4: Rel-15 2+2(4L), Rel-15 2+2(6L) |
|  | Precoding | Random (PMI Rel-15 Type I codebook) for 8Tx |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Other options omitted. |

Figure 1 PDSCH performance with fixed rank of 4 and 6 (UE correlation option 1)

From Figure 1, we can observe that with adaptive MCS but fixed rank configuration, the performance of PDSCH with rank 6 has gain over the performance with rank 4 when SNR is within higher range.

- Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 1 when SNR is within a higher range

Figure 2 PDSCH performance with fixed rank (UE correlation option 2)

Figure 2 also showed that with UE correlation option 2 and adaptive MCS but fixed rank configuration, the performance of PDSCH with rank 6 has gain over the performance with rank 4 when SNR is within higher range.

- Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 2 when SNR is within a higher range

The maximum EVM per radio for the DL is specified in 38.104. This is a minimum requirement at the TAB connector, and so basestations may achieve lower EVM to some degree, depending on load level. Also, and importantly for AAS basestations with large arrays, the signal experienced by the UE is an OTA combination of the ensemble of the outputs of the individual transmitters. Since the wanted signal is combined coherently and EVM components that are not coherent between different transmitters will not combine, the EVM experienced at the UE may exceed the per-transmitter EVM. Taking this into account, results are presented using 1.25% OTA EVM at the UE as an example, and the actual OTA EVM will depend on the AAS type, size, frequency, etc.

We have also evaluated the PDSCH throughput performance with fixed MCS/rank conditions and the correlation captured in the 3GPP spec., as shown in 2, where we reuse the simulation assumption discussed in PDSCH demodulation requirements as much as possible. Figure 3 (a) and Figure 4 (a) showed the absolute PDSCH throughput with low and medium antenna correlation conditions, respectively. We have also shown the relative throughputs in Figure 1 (b) and Figure 2 (b) to evaluate the achieved peak throughput. We have also selected TDLA30-10, usually configured for PDSCH demodulation requirements with higher layers and/or MCS.

Table 2	Simulation parameters for evaluation of MIMO layers in 6Rx UE.

| Parameters | Values |
| --- | --- |
| Propagation channel | TDLA30-10 (TDL-A, delay spread 30ns, Doppler frequency 10Hz) |
| Antenna configuration | 8Tx, 6Rx |
| Antenna correlation | low, medium (See TS 38.101-4 Table B.2.3.2.2-1) |
| MCS | QPSK 1/3 (MCS4) 16QAM 1/2 (MCS13) 64QAM 1/2 (MCS17) |
| MIMO layers | 4 and 6 |
| Precoding | Follow PMI with Rel-15 Type-I codebook |
| PDSCH DMRS configuration | Type 1, 1+1 Single symbol for 4 layers Double symbol for 6 layers |
| Receiver type | MMSE-IRC |

| (a) Absolute throughput | (b) Relative throughput |
| --- | --- |

Figure 3	PDSCH fixed MCS/rank link-simulation results with low correlation.

| (a) Absolute throughput | (b) Relative throughput |
| --- | --- |

Figure 4	PDSCH fixed MCS/rank link-simulation results with medium correlation.

With the low antenna correlation condition, as illustrated in Figure 1, the performance with 6 layers achieves higher throughput compared to 4 layers for QPSK, 16QAM, and 64QAM MCS. This demonstrates a performance gain observed in the given scenario.

- With the TDLA30-10 low antenna correlation condition, the performance gain with 6 layers over 4 layers for QPSK, 16QAM and 64QAM is observed.

Under medium antenna correlation conditions, the performance with 6 layers using QPSK can achieve higher throughput, demonstrating a performance gain. For MCS13 and MCS17, no obvious performance gain has been observed.

- With TDLA30-10 medium correlation, the performance gain with 6 layers over 4 layers for low and medium MCS is observed.

The link simulation results demonstrate a performance gain with 6 layers for QPSK, 16QAM, and 64QAM under lower antenna correlation conditions, making it applicable for FWA/CPE type devices.

In the meantime, a performance improvement with 6 layers over 4 layers was also observed under medium antenna correlation for low and medium MCS levels, which could be beneficial for handheld devices.

Another factor is the likelihood of getting a rank 6 channel in a practical deployment scenario. Rank distributions depend on SINR distribution and propagation conditions. For example, the likelihood of getting a higher rank is low in a deployment scenario with a larger ISD and rural or suburban areas. With a relatively small ISD and dense urban area, the probability of getting a higher rank is probably high.

This WI considers both the handheld and FWA/CPE devices, and considering their differences in the form factor, the correlation concerning the real antenna arrangement in the device varies. From the analysis provided above, we could observe that, with low antenna correlation conditions, MIMO 6 layers could yield performance gain compared to 4 layers. Furthermore, support of 6 layers also depends on the likelihood of getting a rank 6 channel in the practical deployment scenarios with different ISDs. Therefore, we made the following proposal.

- Support of MIMO 6 layers should be evaluated considering the impact of antenna correlation and the feasibility and probability of achieving a rank 6 channel in the deployment scenario.

- Since MIMO 8 layers are already supported for FWA, it is proposed that MIMO 6 layers should also be supported, at least for FWA devices.

- The support of MIMO 6 layers could be considered an optional feature.

According to the current RRC signalling, the number of MIMO layers can be configured as 1, 2, 3, 4, 5, 6, 7, 8. However, the existing UE capability signalling only allows a UE to report to 2, 4, and 8-layer DL MIMO [4]. If agreed to be introduced, one potential solution would be to introduce/add a new candidate value, i.e. 6-layer.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 1 when SNR is within a higher range

Observation 2	Gain has been observed from PDSCH performance with rank 6 to rank 4 configured with UE correlation option 2 when SNR is within a higher range

Observation 3	With the TDLA30-10 low antenna correlation condition, the performance gain with 6 layers over 4 layers for QPSK, 16QAM and 64QAM is observed.

Observation 4	With TDLA30-10 medium correlation, the performance gain with 6 layers over 4 layers for low and medium MCS is observed.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	Support of MIMO 6 layers should be evaluated considering the impact of antenna correlation and the feasibility and probability of achieving a rank 6 channel in the deployment scenario.

Proposal 2	Since MIMO 8 layers are already supported for FWA, it is proposed that MIMO 6 layers should also be supported, at least for FWA devices.

Proposal 3	The support of MIMO 6 layers could be considered an optional feature.

# References

- RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4.

- R4-2417209, WF on requirements for 6Rx, AT&T

- R4-2405154, Draft BigCR on 38.101-4 for introduction of 8Rx performance requirements

- TS 38.331, V18.1.0