3GPP TSG-RAN WG4 Meeting #113									                        R4-2419584

Orlando, US, November 18 – 22, 2024

Agenda item:			7.8.3.1

Source:	Ericsson

Title:	On system parameters for less than 5 MHz CBW for FR1-NTN

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify less than 5 MHz for NTN with the following objectives:

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands

  - Consider the NR-NTN bands n255, n256, and n254

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

  - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth

    - Leverage the work on less than 5MHz for TN

  - Note: no RAN1 impact is expected

- Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands

- Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

  - Focus on power class 3 (PC3) for Tx RF requirements

- Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands

- Specify the necessary signaling to support the above objectives

In this contribution, we focus on the system parameters for channel bandwidths of less than 5 MHz for NR-NTN in FR1-NTN bands and we provide our view for some of the remaining open issues from the approved WF [2].

# Discussion

In the approved WF [2], the two following issues are FFS:

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

- Way forward:

  - Option 1: Mandatory

  - Option 2: Optional

In TN, 3MHz channel bandwidth is optional for some (n26, n28, n85 and n100) and mandatory for other (n31, n72 and n106) bands, where all bands which support it are <1GHz FDD bands. n31, n72 and n106 are all narrow bands with only 5 MHz available bandwidth on DL/UL, which is not the case for FR1-NTN bands. Since the usual practice is to make the support for a new channel bandwidth optional in the Release where it is introduced unless there is a specific request from an operator, we propose to proceed in the same way for 3 MHz CBW for FR1-NTN bands:

Proposal 1: 3MHz channel bandwidth for NR-NTN in FR1-NTN bands should be optional in this Release unless there is a specific request from an operator for some band.

Issue 1-2: Clarification on maximum transmission bandwidth configuration

- Agreement:

  - Follow the previous agreement in WF R4-2413528 as:

Issue 1-1-3: Maximum transmission bandwidth configuration(NRB) - Agreement - Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN.

Maximum transmission bandwidth configuration of 15 PRBs does not prevent smaller transmission bandwidth configurations, e.g. 12 PRBs. SSB for 3MHz CBW should follow the same puncturing as defined in RAN1 for TN, i.e. the SSB of 12 PRBs (punctured from 20 PRBs). For TN 3 MHz channel bandwidth, CORESET#0 size is defined to be 15 PRBs, which was derived from puncturing the original CORESET#0 consisting of 24 PRBs. 12 PRB CORESET#0 was defined for one specific sync raster frequency for n100, and in this case a new CORESET#0 size consisting of 12 PRBs was added (i.e., no puncturing in this case, but new size). Its usage was not precluded for the main case of 3MHz CBW where transmissions can span up to the maximum transmission bandwidth, but if in such a case transmissions can span up to 15 PRBs then is preferred to use a 15 PRBs CORESET#0 (due to a link budget).

Observation 1: 3MHz CBW for NTN shall follow the same design as TN for a general case of 3 MHz CBW:

- Maximum transmission bandwidth configuration of 15 PRBs.

- 12 PRB SSB obtained by puncturing a legacy 20 PRBs SSB.

- 15 PRB CORESET#0 obtained by puncturing a legacy 24 PRBs CORESET#0.

Issue 1-3: Signalling of SS raster

- Way forward:

  - Option 1: The exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations.

  - Option 2: Defer the UE capability discussion for 3MHz in FR1-NTN after all the requirements are clear.

  - Option 3: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN. For both TN and NTN in NR, synchronization raster is never being signaled, and the same shall apply for 3 MHz CBW in NTN.

For both TN and NTN in NR, synchronization raster is never being signaled, and the same shall apply for 3 MHz CBW in NTN.

Proposal 2: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN.

When it comes to the UE capability discussion, we don’t see any issue if the same capabilities were used as for FR1-NTN bands: support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18. However, their description in TS38.306 should be modified – other than TS38.101-1, TS38.101-5 should also be referenced, and in case of support3MHz-ChannelBW-Symmetric-r18 capability the correct table reference should be used for “Applicable SS raster entries per operating band for 3 MHz channel bandwidth” table.

Proposal 3: The exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 should be reused for NTN operations, under certain modifications.

# Conclusion

Based on the discussion in the previous sections we have made the following observation:

Observation 1: 3MHz CBW for NTN shall follow the same design as TN for a general case of 3 MHz CBW:

- Maximum transmission bandwidth configuration of 15 PRBs.

- 12 PRB SSB obtained by puncturing a legacy 20 PRBs SSB.

- 15 PRB CORESET#0 obtained by puncturing a legacy 24 PRBs CORESET#0.

Based on the discussion in the previous sections we propose the following:

Proposal 1: 3MHz channel bandwidth for NR-NTN in FR1-NTN bands should be optional in this Release unless there is a specific request from an operator for some band.

Proposal 2: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN.

Proposal 3: The exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 should be reused for NTN operations, under certain modifications.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2416587, “Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF”, Xiaomi, RAN4#112-bis.