3GPP TSG-RAN WG4 Meeting#113							         		R4-2419618

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	Fragmented carrier in-gap and adjacent assumptions to assess PSD imbalance

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.6.3 Impacts on UE RF requirements and DL performance [FS_NR_FR1_DL_Frag_Carrier]

Document for:	Approval

# 1	Background

In RAN#112bis, we discussed the potential issues related to in-gap and adjacent blockers in [1] based on the current ACS and IIB specification. During the meeting it was clarified that the in-gap and adjacent signals are assumed to be from co-located operators that may not share their radio units. This was captured in way forward [2] as an assumption to study the potential PSD imbalance between the adjacent and in-gap interfering carriers and the wanted channel with the lowest PSD. In this contribution, we discuss the assumptions on the signal bandwidth that can be expected for both adjacent and in-gap regions.

# Discussion

## 2.1 Way forward

The way forward [2] captured agreements on the acceptable imbalance between the adjacent and in-gap interfering carriers and the wanted channel with the lowest PSD and is copied in italics below.

#### Issue 2-2: Study on power spectral density difference between carriers of co-located adjacent channel operators

Agreement: To evaluate acceptable [PSD] difference from UE side between wanted signal and un-wanted signal based on the deployment where BSs from different operators are collocated but their radio units (RUs) are not shared.

- The [PSD] difference means the difference between unwanted signal and the wanted carrier with the lower PSD

- Start evaluation with wanted signal level currently existing ACS requirements as baseline and to see the performance difference.

The above agreement approaches the problem from a UE capability angle rather than the deployment scenario. In this contribution, we will further discuss what may happen in a real deployment.

## 2.1 Interference from co-located separate RU adjacent and in-gap channels.

If we agree that starting from the UE approach based on existing scenarios in the specification as with ACS is the default unless we have a clear statement from operators and/or NW vendors of the worst case/typical PSD imbalance that can be expected from a co-located but separate RU, we are not sure the ACS scenario is completely relevant.

In the real deployment, all the in-gap or outer-adjacent channels will be present simultaneously as interference and their PSD would be in the range between the wanted signal with the lowest PSD and the worst case co-located and separate RU PSD.

In the worst case, the two wanted signals are at the band edge and all the in-gap bandwidth is occupied with co-located/separate RU channels where there would be no protection from the analog BB filter.

Even if the wanted channels are not on the edge of the band, the same in-gap issue would arise and the outer-adjacent channels would also see less filtering than in the 1CC per path case.

In our view, the ACS test may not be representative enough of the co-located/separate RU scenario as:

- The -25dBm power is the maximum received input power and thus would mean that fragmented carrier feature would only work in close proximity of the BS antennas.

- Only one in-gap or adjacent channel interference is present.

- The interfering channel BW is 5MHz.

Proposal: Beyond the ACS scenario RAN4 need to consider scenarios where:

- The interfering channel occupies the entire cap between the two wanted channels.

- The outer-adjacent channel may still use the 5MHz channel, but interfering channels of the same BW than the adjacent wanted channel may also be relevant.

- The wanted channel with the lowest PSD is closer to REFSENS to enable the fragmented carrier feature to work in a large portion of the cell. In that respect, the wanted signal level in the IBB scenario may be more relevant.

To illustrate the possible (symmetrical only) cases for the wanted and interfering channels, the figure presented in [1] is copied here.

The only change would be:

- To use the REFSENS+X level for the wanted signal with the lowest PSD from the IBB case

- Find the acceptable interferer power when an interfering channel occupies the entire gap by checking the throughput loss on the two wanted channels.

- Do the same when an outer adjacent channel of 5MHz or the same BW than the wanted channel is used.

Figure 1: In-band blocking and ACS case 1 and 2 scenarios for <2.7GHz bands

# Conclusion

In this contribution, based on our previous work on the topic and comparing ACS and in-band blocking scenarios with the collocated/separate RU interference, we formulate the following proposal.

Proposal: Beyond the ACS scenario RAN4 need to consider scenarios where:

- The interfering channel occupies the entire cap between the two wanted channels.

- The outer-adjacent channel may still use the 5MHz channel, but interfering channels of the same BW than the adjacent wanted channel may also be relevant.

- The wanted channel with the lowest PSD is closer to REFSENS to enable the fragmented carrier feature to work in a large portion of the cell. In that respect, the wanted signal level in the IBB scenario may be more relevant.

# References

[1] R4-2415533 On in-gap and adjacent blockers level for fragmented spectrum reception in a single Rx path, Skyworks Solutions Inc., RAN4#112bis

[2] R4-2417201 WF on fragmented, DL Mediatek, RAN4#112bis