# 3GPP TSG-RAN WG4 Meeting #113	R4-2419621

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.20.3.1

Source: 	Samsung

Title: 	Views on L1 CLI measurement requirements

Document for:	Discussion

- Introduction

In Rel-19 Duplex evoluation WI, L1 CLI measurement is introduced in RAN1 to handle co-channel UE interference due to SBFD opeation.

RAN4 initiated discussion on L1 CLI measurement requirements in previous meeting with latest agreement captured in WF [2]:

### Sub-topic 1-1: RRM impacts of UE-to-UE CLI handling #### Issue 1-1-1: Measurement quantities - Agreements (from online session) - RAN4 to define requirements for L1-SRS-RSRP and L1-CLI-RSSI. - Requirements for other measurement quantities can be considered based on further RAN1 agreement. #### Issue 1-1-6: Side condition - Agreements (from online session) - Send LS to RAN1: in addition to intra-cell scenario, whether inter-cell scenario is considered in this WI. Further discuss the details in the LS. - For SRS configuration, re-use Rel-16 side condition as baseline. #### Issue 1-1-14: Measurement timing for L1-SRS-RSRP - Agreements (from online session) - As baseline, for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. - This can be revisited if it proves to be conflicting with further RAN1 agreements.

In this contribution, further analysis is provided on remaining open issues for L1 CLI measurement.

- Discussion

#### Issue 1-1-3: Measurement methods

RAN1 made following agreements on measurement methods:

RAN1#118 Agreement For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

With above agreements from RAN1 , RAN4 can focus on method 1 for CLI-RSRP measurement and method 2 for CLI_RSSI measurement.

Proposal 1: RAN4 shall specify L1 CLI measurement requirements at least for below methods

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Note: Other methods not precluded if RAN1 agree to introduce.

#### Issue 1-1-4: Rx beam

For Rx beam assumption, RAN1 agreed that qcl-Type set to 'typeD' is optionally configured (either dedicated list of TCI states or unifiedTCI-StateType). If not configured, then legacy R16 L3-CLI measurement assumption used.

Proposal 2:  For Rx beam assumption, follow RAN 1 agreements as following：

- In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo

  - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic

- If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource:

  - If unifiedTCI-StateType is not configured

    - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET”

  - If unifiedTCI-StateType is configured

    - “Indicated” DL only/joint TCI state as specified in TS 38.214

  - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources

#### Issue 1-1-5: Measurement resources /Issue 1-1-8 measurement reporting

On measurement resource type, RAN1 agreed to reuse legacy L3 CLI measurement configuration with semi-static, aperiodic and periodic resources in CSI-ResourceConfig.

RAN1#118bis Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications. - Note: No new usage needs to be defined for SRS resource sets

Proposal 3: For measurement resource type, RAN4 shall cover semi-static, aperiodic and periodic resource types following the configuration in CSI-ResourceConfig.

On measurement resource allocation and reporting, two CLI-RSRI measurement resource types are specified in RAN1. And wideband CLI reporting is agreed in RAN1.

RAN1#118 Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting FFS: Subband CLI-RSSI reporting RAN1#118bis Agreement For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported. - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

Proposal 4: For CLI-RSRP reporting, focus on wideband CLI-RSRP reporting.

Proposal 5: For CLI-RSSI reporting, focus on wideband CLI-RSRI reporting

- Under method 1 (CLI-RSSI within DL sub-band(s)), CLI-RSRI measurement is derived based on DL usable PRBs within DL sub-band(s) with two measurement resource type:

  - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband

  - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands

Unlike R16 L3 CLI measurement, Rel-19 L1 CLI measurement use CSI reporting framework and at least A-P reporting supported.

Proposal 6: Focus on aperiodic CSI reporting mode for Rel-19 CLI measurement reporting

- For other reporting modes hold on the discussion pending on further progress in RAN1.

Issue 1-1-6: Side condition

During R16 L3 CLI measurement discussion, timing offset is critical issue due to timing difference between victim UE DL timing and SRS arrival timing from victim UE side as show in below figure:

Figure 1: Timing difference between SRS arrival timing from aggressor UE and DL timing from serving cell

In rel-16 CLI measurement, due to larger residual time error which even over than CP for some of SCSs; different L3-SRS-RSRP measurement accuracy was specified based on SCSs as in section 10 of TS 38.133.

In Rel-16 CLI following agreement reached in the end:

- R16 CLI value: TCLI = Tc*NTA_offset + Tother

  - Tother = Tpropagation_delay + Tcell_phase_synchronization

  - Tother = 4.67usec for FR1 and 3.67usec for FR2 with (ISD =500m for FR1 and 200m for FR2 assumed)

For L1 CLI measurement, this time offset and residual time error maybe optimized with following reason:

- Rel-16 L3 CLI introduced for SRS transmitted from neighbor cell for dynamic TDD scenario (different gNBs have different DL-UL TDD pattern)

- Rel-19 SBFD operation with both UL sub-band and DL sub-band in DL BWP from SBFD gNB side, L1 CLI SRS-RSRP is target for intra-cell scenario which no cell phase error needs to be considered

- If inter-cell L1 CLI measurement considered in Rel-19, cell phase error between cells/gNBs can be optimized under co-location and well synced scenario with info exchange between gNBs.

In last RAN4 meeting, RAN4 sent a LS to RAN1 for clarification of L1-CLI measurement target scenarios (intra-cell only or both intra-cell and inter-cell cases).

| Residual timing error (time offset between measurement timing at UE0 and arrival timing of SRR from UE 1) Note 2 | - Case 1: 4.67us for FR1, 3.67us for FR2 (inter-cell case) - Case 2: 1.67us for FR1, 0.67us for FR2 (intra-cell case) |
| --- | --- |

Based on the initial simulation results from companion t-doc [x], we observed SRS-RSRP measurement accuracy is sensitive to residual timing error especially under case 1. And under case 2, the performance improved a lot.

In our understanding, L1-CLI measurement was introduced major target for intra-cell inter-UE CLI with SBFD operation which is different compared L3 CLI measurement.

Pending on RAN1 feedback, several options can be considered for how to treat side condition assumption of residual timing error:

- Option 1: RAN4 specify two sets of requirements based on different residual timing error conditions

    - Note: In reality, UE may not aware of residual timing error condition; UE behavior is same meanwhile the expected the performance is different.

- Option 2: RAN4 specify two sets of requirements based on different residual timing error conditions with NW indication

    - E.g., if “timeRestrictionForChannelMeasurement” is configured than single shot measurement with residual timing error condition as case 2; otherwise as legacy L3 CLI measurement 3 samples with case 1 residual timing error condition or new dedicated signaling

- Option 3: RAN4 specify one set requirement based on case 2 for residual timing error condition

    - Note: It’s not precluded intra-cell or inter-cell measurement, RAN4 specify requirement based on major target scenario and typical residual timing error (in reality even inter-cell can maintain good performance on cell-phase sync.)  RAN4 existing L3-CLI measurement already provide performance reference on another side condition.

Proposal 7: Further discuss residual timing error side condition pending  on RAN1 feedback with following candidate options:

- Option 1: RAN4 specify two sets of requirements based on different residual timing error conditions

- Option 2: RAN4 specify two sets of requirements based on different residual timing error conditions with NW indication

- Option 3: RAN4 specify one set requirement based on case 2 for residual timing error condition

Regarding other side conditions, we can take Rel-16 CLI measurement assumption as starting point as agreed in the simulation assumption which also pending on the measurement evaluation.

Proposal 8: Reusing Rel-16 L3 CLI SRS-RSRP measurement assumption as starting point for following side conditions:

- SRS port: 1

- SRS bandwidth: 48 RBs

- SRS symbol: 1

- No SRS repetitions

- Frequency hopping, sequence group hopping or sequence hopping is disabled

- SNR side condition: 1 dB

#### Issue 1-1-7: Measurement period /1-1-9: Measurement accuracy

The measurement period and measurement accuracy are pending on side condition and simulation evaluations. Based on the initial simulation results from companion t-doc [x], the observations reproduced here for reference:

Observation 1: SRS-RSRP measurement accuracy is sensitive to residual timing error

- With residual timing error case 1 (R16 L3 CLI assumption), measurement accuracy will be degraded with numerologies (SCS) increased as ~1dB for 15kHz, ~2dB for 30kHz, ~3.5dB for FR1 60kHz, ~2.6 dB for FR1 60khz and ~6.0 dB for FR2 120kHz (with 3 samples).

- With residual timing error case 2 (w/o cell phase sync error), measurement accuracy improved a lot compared to case 1 especially under high numerologies as   0.7 dB for 15kHz, 1.0 dB for 30kHz, 1.5 dB for FR1-60kHz, 0.9dB for FR2-60kHz and 1.3 dB for FR2-120kHz.

Observation 2: With number of samples for measurement increased, the measurement accuracy improvement is limited with ~0.5dB improvement with 3 samples compared to 1 sample measurement and ~0.1dB improvement with 3 samples compared to 2 samples.

Figure 2: Absolute RSRP accuracy [dB] under AWGN with SNR =1dB, 48 PRB SRS BW

Proposal 9: With residual timing error assumption case 2, single shot measurement can be specified with unified L1-SRS-RSRP accuracy for all SCSs i.e., 3dB with RF margin.

#### Issue 1-1-10: Scheduling and measurement restriction

In Rel-16 L3 CLI,  FR2 QCL assumption leave to UE implementation with latest received PDSCH/coreset QCL.

R16 L3 CLI measurement:  Depending on the UE capability, UE is not expected to receive PDCCH/PDSCH on SRS-RSRP/CLI-RSSI measured OFDM symbols, and on Y/X data symbol before SRS-RSRP measured symbol in intra-band CA.

- Y = 1 : 15kHz/30kHz SCS for FR1 and 60kHz SCS for FR2

- Y = 2 : 60kHz SCS for FR1 and 120kHz SCS for FR2

For Rel-19 CLI, configuration/determination of ‘typeD’ QCL assumptions will be specified for the CLI measurement resource.

Regarding measurement restriction, this is majorly decided by time difference between SRS receiving measurement timing and serving cell DL timing. And it’s dominated by NTA offset.

Proposal 10: Further discuss scheduling availability restrictions for L1 CLI measurement and reusing Rel-16 CLI rules and values as starting point with necessary update by taking following changes into account:

- FR2 ‘typeD’ QCI assumption for L1 CLI measurement

- Timing difference assumption between SRS arriving timing from aggressor UE and DL timing of serving cell

#### Issue 1-1-11: Measurement capability

We agree with other companies’ view from last RAN4 meeting. Measurement capability shall belong to RAN1 scope, RAN4 can wait further progress in other WGs and furthe evaluate it in later stage.

Propsoal 11: Postpone the dicussion on measurement capability pending the progress and agreements from othe WGs.

#### Issue 1-1-12: Report mapping

For L1-RSRP and L1-SINR reporting, the reporting range with step size is specified in RAN1 specification TS 38.214 5.2.1.4.3 and 5.2.1.4.4. And RAN4 TS 38.133 specified the mapping between reported value and measurement quantity followed RAN1 specification.

##### 5.2.1.4.3 L1-RSRP Reporting For L1-RSRP computation - the UE may be configured with CSI-RS resources, SS/PBCH Block resources or both CSI-RS and SS/PBCH block resources, when resource-wise quasi co-located with 'type C' and 'typeD' when applicable. - the UE may be configured with CSI-RS resource setting up to 16 CSI-RS resource sets having up to 64 resources within each set. The total number of different CSI-RS resources over all resource sets is no more than 128. For L1-RSRP reporting, if the higher layer parameter nrofReportedRS in CSI-ReportConfig is configured to be one, or if the higher layer parameters nrOfReportedCells and nrOfReportedRS-PerCell are both configured to be one, the reported L1-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedRS is configured to be larger than one, or if the higher layer parameter groupBasedBeamReporting is configured as 'enabled', or if the higher layer parameter groupBasedBeamReporting-r17 is configured, or if any of the higher layer parameters nrOfReportedCells and nrOfReportedRS-PerCell is configured to be larger than one, the UE shall use differential L1-RSRP based reporting, where the largest measured value of L1-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-RSRP is quantized to a 4-bit value. The differential L1-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-RSRP value which is part of the same L1-RSRP reporting instance. The mapping between the reported L1-RSRP value and the measured quantity is described in [11, TS 38.133].

For L1-CLI measurement reporting mapping, RAN4 shall wait for the progress from RAN1 to avoid duplicated work and any misalignment between RAN1 and RAN4.

Proposal 12: Postpone the discussion on report mapping for L1-CLI measurement and wait for the agreement from RAN1.

#### Issue X: Timing assumption for L1-RSSI measurement

RAN1#118bis Agreement For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband).

Proposal 13: For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband).

- Conclusion

In this contribution, further analysis is provided on remaining open issues for L1 CLI measurement.

#### Issue 1-1-3: Measurement methods

Proposal 1: RAN4 shall specify L1 CLI measurement requirements at least for below methods

- Method#1: UE measures RSSI within DL subband

- Method#2: UE measures RSRP of aggressor UE within UL subband

- Note: Other methods not precluded if RAN1 agree to introduce.

#### Issue 1-1-4: Rx beam

Proposal 2:  For Rx beam assumption, follow RAN 1 agreements as following：

- In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo

  - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic

- If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource:

  - If unifiedTCI-StateType is not configured

    - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET”

  - If unifiedTCI-StateType is configured

    - “Indicated” DL only/joint TCI state as specified in TS 38.214

  - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources

#### Issue 1-1-5: Measurement resources /Issue 1-1-8 measurement reporting

Proposal 3: For measurement resource type, RAN4 shall cover semi-static, aperiodic and periodic resource types following the configuration in CSI-ResourceConfig.

Proposal 4: For CLI-RSRP reporting, focus on wideband CLI-RSRP reporting.

Proposal 5: For CLI-RSSI reporting, focus on wideband CLI-RSRI reporting

- Under method 1 (CLI-RSSI within DL sub-band(s)), CLI-RSRI measurement is derived based on DL usable PRBs within DL sub-band(s) with two measurement resource type:

  - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband

  - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands

Proposal 6: Focus on aperiodic CSI reporting mode for Rel-19 CLI measurement reporting

- For other reporting modes hold on the discussion pending on further progress in RAN1.

Issue 1-1-6: Side condition

Proposal 7: Further discuss residual timing error side condition pending on RAN1 feedback with following candidate options:

- Option 1: RAN4 specify two sets of requirements based on different residual timing error conditions

- Option 2: RAN4 specify two sets of requirements based on different residual timing error conditions with NW indication

- Option 3: RAN4 specify one set requirement based on case 2 for residual timing error condition

Proposal 8: Reusing Rel-16 L3 CLI SRS-RSRP measurement assumption as starting point for following side conditions:

- SRS port: 1

- SRS bandwidth: 48 RBs

- SRS symbol: 1

- No SRS repetitions

- Frequency hopping, sequence group hopping or sequence hopping is disabled

- SNR side condition: 1 dB

#### Issue 1-1-7: Measurement period /1-1-9: Measurement accuracy

Observation 1: SRS-RSRP measurement accuracy is sensitive to residual timing error

- With residual timing error case 1 (R16 L3 CLI assumption), measurement accuracy will be degraded with numerologies (SCS) increased as ~1dB for 15kHz, ~2dB for 30kHz, ~3.5dB for FR1 60kHz, ~2.6 dB for FR1 60khz and ~6.0 dB for FR2 120kHz (with 3 samples).

- With residual timing error case 2 (w/o cell phase sync error), measurement accuracy improved a lot compared to case 1 especially under high numerologies as   0.7 dB for 15kHz, 1.0 dB for 30kHz, 1.5 dB for FR1-60kHz, 0.9dB for FR2-60kHz and 1.3 dB for FR2-120kHz.

Observation 2: With number of samples for measurement increased, the measurement accuracy improvement is limited with ~0.5dB improvement with 3 samples compared to 1 sample measurement and ~0.1dB improvement with 3 samples compared to 2 samples.

Figure 2: Absolute RSRP accuracy [dB] under AWGN with SNR =1dB, 48 PRB SRS BW

Proposal 9: With residual timing error assumption case 2, single shot measurement can be specified with unified L1-SRS-RSRP accuracy for all SCSs i.e., 3dB with RF margin.

#### Issue 1-1-10: Scheduling and measurement restriction

Proposal 10: Further discuss scheduling availability restrictions for L1 CLI measurement and reusing Rel-16 CLI rules and values as starting point with necessary update by taking following changes into account:

- FR2 ‘typeD’ QCI assumption for L1 CLI measurement

- Timing difference assumption between SRS arriving timing from aggressor UE and DL timing of serving cell

#### Issue 1-1-11: Measurement capability

Propsoal 11: Postpone the dicussion on measurement capability pending the progress and agreements from othe WGs.

#### Issue 1-1-12: Report mapping

Proposal 12: Postpone the discussion on report mapping for L1-CLI measurement and wait for the agreement from RAN1.

#### Issue X: Timing assumption for L1-RSSI measurement

Proposal 13: For L1 CLI-RSSI measurement, DL timing is used for Method #1 (UE measures RSSI within DL subband).

Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei, HiSilicon

[2] R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

[3] R4-2419620, Initial simulation results for L1 CLI measurement, Samsung