# 3GPP TSG-RAN WG4 Meeting #113	R4-2419623

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.8.3.2

Source: 	Samsung

Title: 	UE RF requirements for Less than 5MHz supporting in NTN FR1 bands

Document for:	Discussion

1		Introduction

For Rel-19 Enhanced requirements and conductive test methodology for NR NTN and IoT NTN WI, supporting 3MHz CHBW for NTN operation is one of major objective with detailed WID as following:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signalling to support the above objectives

In this contribution, we analyse remaining open issues for UE RF requirements.

2		Discussion

- UE Tx requirements

In below table we summarize the expected changes/update on TS 38.101-5 section 6 Tx RF requirements due to 3MHz CHBW introduction.

| Section | Expected changes/update |
| --- | --- |
| 6.2 Transmitted power 6.2.1 UE maximum output power 6.2.2 UE maximum output power reduction 6.2.3 UE additional maximum output power 6.2.4 Configured transmitted power | According to WID, target PC is PC3. No update for MPR, refer to TN specification For regulation requirements and A-MPR (table), further work required for below NS values: - Band n254: NS_03N, NS_04N and NS_05N - Band n256: NS_24 - Band n255: NS_02N (no A-MPR required) |
| 6.3 Output power dynamics 6.3.1 Minimum output power 6.3.2 Transmit OFF power 6.3.3 Transmit ON/OFF time mask 6.3.4 Power control | 3MHz CHBW need to be added into section 6.3.1 and 6.3.2 without change on the requirements. |
| 6.4 Transmit signal quality | No update expected |
| 6.5 Output RF spectrum emission 6.5.1 Occupied bandwidth 6.5.2 Out of band emission 6.5.3 Spurious emission 6.5.4 Transmit intermodulation | For 6.5.1 Occupied bandwidth: - 3MHz CHBW need to added into table without change on requirement. For 6.5.2 out of band emission: - 3MHz CHBW need to be added into ACLR table without change on requirements - General emission mask needs to be update with 3MHz CHBW as similar changes for breaking point from TN TS 38.101-1 For 6.5.3 Spurious emission - FOOB need to be updated to add 3MHz CHBW with 6MHz boundary No update on 6.5.4 transmit intermodulation |

Issue 2-1-3: A-MPR

- Way forward - RAN4 to evaluate A-MPR for 3MHz channel BW for NS_02N, NS_03N, NS_04N, NS_05N and NS_24;

Regarding A-MPR evaluation, with further checking, at least no A-MPR required for NS_02N. For other cases, further evaluation still ongoing.

Proposal 1: For A-MPR, no A-MPR required for NS_02N.

Issue 2-1-9: General spurious emission

- Way forward - FFS

Proposal 2: For 6.5.3 Spurious emission: FOOB need to be updated to add 3MHz CHBW with 6MHz boundary

Issue 2-1-10: Additional spurious emissions

- Way forward - Option 1: New requirements need define for NS_02N, 03N, 04N, 05N - Option 2: For the additional spurious emissions requirements, add 3 MHz CBW into - Table 6.5.3.3.2-1: Additional requirements for "NS_02N"; - Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N"; - Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N". - Add Table 6.5.3.3.5-1 Additional out-of-band requirements for "NS_24" - Option 3: Introduce 3 MHz channel bandwidth for all network signalling values

In previous meeting, we have agreements that all existing NR bands need to support 3MH CHBW including band n256, n255 and n254. And these additional spurious emissions are regulation requirement which irrespective of channel bandwidth.

Above options are not contradicted with each other.  Only one issue that existing NS_24 requirements are specified in TS 38.101-1 and TS 38.101-5 refer to TN 38.101-1 meanwhile TN band n65 has no request to support 3MHz CHBW. Then we need to update NTN TS 38.101-5 to make NS 24 regulation emission as self-contained instead of referring to TS 38.101-1.

Proposal 3:  For the additional spurious emissions requirements, add 3 MHz CHBW for NS_02N, 03N,04N, 05N and NS_24 with following detailed update to TS 38.101-5

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N".

- New table for Additional out-of-band requirements for "NS_24"  as self-contained instead of referring to TS 38.101-1

- UE Rx requirements

In below table we summarize the expected changes/update on TS 38.101-5 section 7 Rx RF requirements due to 3MHz CHBW introduction.

| Section | Expected changes/update |
| --- | --- |
| 7.3 Reference sensitivity | New requirements for 3MHz need be specified for reference sensitivity on band n256, n255 and n254. |
| 7.4 Maximum input level | 3MHz CHBW need to be added into table without change on the requirements. |
| 7.5 Adjacent channel selectivity | 3MHz CHBW need to be added into table without change on the requirements. - The interference signal configuration also need to be updated with CHBW 3MHz and offset. |
| 7.6 Blocking characteristics 7.6.2 In-band blocking 7.6.3 Out of band blocking 7.6.4 Narrow band blocking | For in-band blocking, similar change as TN specification, add 3MHz CHBW into table and interference frequency offset need to be updated. For out of band blocking, 3MHz CHBW need to be added into table without change on the requirements. For narrow band blocking, similar update as TN specification, 3MHz CHBW added into table with Pw as 18dBw. |
| 7.7 Spurious response | 3MHz CHBW need to be added into table without change on the requirements. |
| 7.8 Intermodulation characteristics | No update on NTN specification expected. |
| 7.9 Spurious emissions | No update |
| FRC | No additional update on FRC |

Issue 2-2-1: Reference sensitivity

- Way forward - Option 1: 2.2dB improvement compared with 5 MHz CBW; - Option 2: 1.7dB improvement compared with 5 MHz CBW;

In previous meeting, we proposed with 2.2 dB lower value compared to 5MHz CHBW with scaling the channel BW. After further checking, we think option 2 is more reasonable considering IP2 impact and lower input level. Option 2 is aligned with TN 3MHz approach.

Proposal 4: For Reference sensitivity, option 2 with 1.7dB difference compared to 5MHz CHBW.

Issue 2-2-2: Maximum input level

- Way forward - FFS on the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm)

We didn’t see the reason why existing requirements from TS 38.101-5 for 5MHz can’t be applied for 3MHz CHBW.  And the maximum input level from NTN specification is much lower than the value specified for TN due to propagation distance difference, we didn’t see any issue from UE implementation to support -40dBm.

Proposal 5: For Maximum input level, reuse existing value in TS 38.101-5 as -40dBm.

Issue 2-2-3: ACS

- Way forward - Option 1: Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE. - Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

Option 1 and option 2 list above are not contradicted with each other and both of them can be agreed as package.

Proposal 6: For ACS:  Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE with following changes to TS 38.101-5

- 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

3		Conclusion

In this contribution, brief summary on UE RF specification impact of introducing 3MHz CHBW was provided.

Proposal 1: For A-MPR, no A-MPR required for NS_02N.

Proposal 2: For 6.5.3 Spurious emission: FOOB need to be updated to add 3MHz CHBW with 6MHz boundary

Proposal 3:  For the additional spurious emissions requirements, add 3 MHz CHBW for NS_02N, 03N,04N, 05N and NS_24 with following detailed update to TS 38.101-5

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N".

- New table for Additional out-of-band requirements for "NS_24"  as self-contained instead of referring to TS 38.101-1

Proposal 4: For Reference sensitivity, option 2 with 1.7dB difference compared to 5MHz CHBW.

Proposal 5: For Maximum input level, reuse existing value in TS 38.101-5 as -40dBm.

Proposal 6: For ACS:  Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE with following changes to TS 38.101-5

- 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

Reference

[1] RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[2] R4-2416587, Way Forward for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi