# 3GPP TSG-RAN WG4 Meeting #113	R4-2419625

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.9.3

Source: 	Samsung

Title: 	System parameters for Ku band

Document for:	Discussion

1		Introduction

For Rel-19 Enhanced requirements and conductive test methodology for NR NTN and IoT NTN WI, the suitable system parameters e.g. FR1 like or FR2 like is key issue still on discussion

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

In this contribution, views on system parameters were provided.

2		Discussion

In previous RAN4 meeting, WF [2] list several open issues to check the numerologies on Ku band FR1 like vs FR2 like:

- Issue 3-2: Numerology - Decision criteria The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology - UL timing synchronization - Phase noise - Efficient channel utilization - Beam hopping granularity - Link budget UL/DL – as a function of SCS and channel bandwidth

UL timing synchronization

In general, with SCS increased, CP length will be reduced and the tolerated timing error capability will be decreased accordingly. In RRM session, different UL timing requirements are specified for NTN FR1 and NTN FR2 operation with principle that timing error within 1/2 CP. It’s more challenge to balance the system performance and reasonable UL timing requirements on UE implementation of the estimation of satellite position and UE position.  Especially with 120kHz SCS, Mobile VSAT for NGSO maybe introduced in the future that will bring challenge to UL timing synchronization in terms of UE position capability and system performance impact.  Separate numerologies can be applied pending for DL and UL e.g., 60kHz kHz for UL and 120 kHz for DL.

Observation 1: Lower SCSs are more friendly for UL timing synchronization error.

Observation 2: No issue identified for SCSs up to 60kHz on UL timing synchronization, if FR2 like numerologies considered, separate numerologies can be applied for DL and UL e.g., 60kHz/120kHz for DL and 60kHz for UL.

Phase noise

With operating frequency increased, system performance is more sensitive on phase noise. And large SCSs is more robust on PN impact. Currently 15kHz, 30kHz and 60kHz are specified for FR1 bands (400MHz ~ 7 GHz), 60kHz and 120kHz are specified for FR1-1 (24 ~56 GHz) and 120kHz, 240kHz are specified for FR2-2 (above 52.6 GHz).  For operation under 10 ~15 GHz, 60kHz and 120kHz SCS are more robust to phase noise impact. With our internal checking, 30kHz above is feasible if not consider mobile VSAT. If mobile VSAT for LEO taking into account, 60kHz above is more robust performance.

Observation 3: Larger SCSs are more robust for PN noise.

- 15kHz SCSs shall not be considered

- 60kHz or above is more robust with mobile VSAT for LEO scenario. Other scenarios, 30kHz above is feasible.

Efficient channel utilization

There is no big difference in terms of single channel bandwidth spectrum utilization. Meanwhile we need to consider the future compatibility to support aggregated bandwidth as 125 MHz for UL and 250MHz for DL.  For future supporting wider aggregated bandwidth i.e. 125MHz for UL and 250MHz for DL, FR2 has more advantage e.g. using single carrier with new channel bandwidths introduced in future release for 125MHz CHBW and 250MHz CHBW. For FR1, CA operation is required i.e., 3 carriers with 100MHz per carrier to support 250MHz aggregated channel bandwidth.  The basic assumption in NR for FFT size is 4K, with 250MHz under 30KHz and 60kHz, multiple FFT required which bring more complexity on UE side. CA approach to support wider bandwidth will bring scheduling complexity under both gNB and UE side, which also increase overhead on control channel and other reference signals e.g. SSB, CSI-RS to decrease spectrum utilization.

On other side, if considering less than 100MHz spectrum, then FR1 CHBW sets provide more flexibility on spectrum usage.

For forward compatibility to support wider bandwidth 125MHz in UL and 250MHz in DL:

- FR1 like approach: CA required with at least 2 CCs in uplink and 3 CCs in DL

- FR2 like approach: Possible to support 125MHz and 250MHz with new channel bandwidth introduced in FR2 without CA

Observation 4:  FR1 CHBW sets provide more flexibility of usage of smaller BW e.g. below 100MHz and with more cost to support wider BW under CA approach.

Observation 5:  FR2 CHBW sets  provide possibility to support 125MHz and 250MHz with new channel bandwidth introduced in FR2 without CA

Beam hopping granularity

With wider coverage of satellite service, beam hopping is required to efficient utilize the transmitted power from satellite side with better coverage.  With FR2 numerologies, more candidate SSB beam indexes (64 SSB indexes for FR2 and maximum 16 SSB indexes in FR1/30kHz) in time domain can be supported which can support finer beam hopping granularity.

Observation 6: FR2 SSB provide more finer Beam hopping granularity which is helpful for coverage enhancement.

Link budget UL/DL – as a function of SCS and channel bandwidth

The link budget is complexity with many factors to be considered i.e. operating frequency, transmission bandwidth, transmission power, hybrid beamforming and scheduling. It’s hard to make judgment which numerologies for friendly on link budget with many factors need to be considered.

Observation 7: No direct linkage between numerologies and link budget UL/DL.

Another issue which didn’t list in the factors for consideration is the work load in RAN4. The target terminal device types for Ku band is same as Ka band as VSAT.  Existing FR1 NTN bands requirements are specified based on target device type assumption as handheld devices.

The requirements specified for Ka band including system parameters, UE RF and RRM are specified based FR2 like approach with beamforming assumption. In order to leverage the work from Ku band as much as possible, it’s better to align with previous Ka band assumption with FR2 like otherwise all UE RF and RRM requirements which specified for FR2 Ka band need to be reviewed and checked whether still applicable to Ku band if different numerologies applied for Ku band and Ka band.

Observation 8: It’s beneficial to leverage the work from Ka band and control RAN4 workload if same numerologies applied for Ka band and Ku band.

To summary above analysis:

Observation 9: Both options (FR1 like or FR2 like) have pros and cons with different numerologies.

Proposal 1: Further discuss below options:

- Option 1: Introduce Ku band as FR2 like with 60kHz and 120kHz SCSs, separate numerologies can be applied for DL and UL (our preference)

- Option 2: Introduce Ku band as FR1 like with 30kHz and 60kHz numerologies

- Option 3: Introduce separate Ku bands with FR1 like approach and FR2 like approach separately.

3		Conclusion

In this contribution, views on system parameters for Ku band were provided.

Observation 1: Lower SCSs are more friendly for UL timing synchronization error.

Observation 2: No issue identified for SCSs up to 60kHz on UL timing synchronization, if FR2 like numerologies considered, separate numerologies can be applied for DL and UL e.g., 60kHz/120kHz for DL and 60kHz for UL.

Observation 3: Larger SCSs are more robust for PN noise.

- 15kHz SCSs shall not be considered

- 60kHz or above is more robust with mobile VSAT for LEO scenario. Other scenarios, 30kHz above is feasible.

Observation 4:  FR1 like approach provide more flexibility of usage of smaller BW e.g. below 100MHz and with more cost to support wider BW under CA approach.

Observation 5:  FR2 like approach provide possibility to support 125MHz and 250MHz with new channel bandwidth introduced in FR2 without CA

Observation 6: FR2 SSB provide more finer Beam hopping granularity which is helpful for coverage enhancement.

Observation 7: No direct linkage between numerologies and link budget UL/DL.

Observation 8: It’s beneficial to leverage the work from Ka band and control RAN4 workload if same numerologies applied for Ka band and Ku band.

To summary above analysis:

Observation 9: Both options (FR1 like or FR2 like) have pros and cons with different numerologies.

Proposal 1: Further discuss below options:

- Option 1: Introduce Ku band as FR2 like with 60kHz and 120kHz SCSs, separate numerologies can be applied for DL and UL (our preference)

- Option 2: Introduce Ku band as FR1 like with 30kHz and 60kHz numerologies

- Option 3: Introduce separate Ku bands with FR1 like approach and FR2 like approach separatly.

Reference

# [1] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

# [2] R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Eutelsat

# [3] R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Eutelsat