3GPP TSG-RAN WG4 Meeting #113	R4-2419630

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Measurement requirements for less than 5 MHz operation in NTN

Agenda item:	7.8.3.4

Document for:	Discussion

Introduction

In RAN #103, RAN plenary has approved the RAN 4 WI for NTN features [1]. The WI was then updated in RAN #104 [2]. One of the topics for this WI is the operation of NTN in less than 5 MHz whose objectives are presented below:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

Currently, RAN4 has agreed – in what concerns RRM requirements – to reutilize for the most part the proper modifications the pre-existing requirements utilized for operation with less than 5MHz in TN to the operation  with such bandwidths in NR-NTN. In the WF approved in the most recent meeting, very few open issues are presented, including, but not limited to:

- Issue 1-2-1-2 CHO delay - Agreements/ Way-forward: - for CHO without L3 measurement: - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - FFS on for CHO with L3 measurement: - Tmeasure which is same as the definition for Tidentify_intra_with_index and Tidentify_inter_with_index in issue 1-2-4-2 will be extended - T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [3 Trs] - Issue 1-2-4-2: How to define the new requirements for SSB time index detection - Agreements/Wayforward - The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively. - For X and Y, the values from TN less than 5MHz as starting point.

Discussion

We will focus our analysis on the two issues related above.

CHO delay

First of all it is important to highlight that although most of the previous requirements agreed for TN were reutilized/re-edited for the NTN operation in less than 5 MHz, there are some aspects related to CHO that are exclusive to NTN. For instance, NTN has introduced, in Rel-18, the concept of location-based and time-based CHO. This new CHO types define a handover once a trigger that is based on UE position (location-based) or in a instant in time (time-based) the network needs to perform the CHO to a given cell.

The difference of this approach is that for the first time the CHO is not reliant on a cell measurement and a HO condition might be triggered without prior detection of the target cell, the so called “blind CHO”. For such type of HO, the search is most of the times performed blindly, and the UE cannot pre-emptively acquire timing information of the target cell, therefore, there is good justification to increase the total search time - T∆ - for this case, based on the lack of previous information on the target cell.

However, for the case where there is L3 measurement performed by the UE, the UE might perform the HO in a more efficient manner. Upon receiving a CHO the UE knows that a HO is soon to be expected by Network, in special in the cases where a secondary trigger (time or location based) is configured.

Upon receiving a CHO command with secondary triggers, the UE can anticipate a HO is expected to happen.

- There is some discussion whether the measurement preparations, common for the traditional CHO,  needs a larger time requirement time in order to follow increases in Tidentify_intra/inter_with_index caused by the punctured SSB. If that happens, the total time is increased by some SMTC periodicities (representing the UE will have multiple times to read the SSB block of the target cell).

If the value of T∆ is also expanded, as the UE needs to acquire full timing information of the serving cell by a similar relaxation factor, the assumption is that the UE is not leveraging the opportunities from the increase in Tmeasure to acquire an early time tracking of the target cell.  Moreover, in several scenarios, the target cell is located in the same satellite, most likely with same timing properties as the current serving cell of the UE.

We propose that T∆ is only relaxed by one Trs such that the UE can leverage some information on the already previously extended value of  Tidentify_intra/inter_with_index.

The value of T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [2 Trs]

Index detection requirements

Another open issue is related to the Index detection requirements during neighbor cell measurements. For TN deployments in less than 5 MHz, the Index detection was increased in 4 (intra-frequency measurements) and 3 (inter-frequency measurements).

However, the number of samples was increased because companies, when specifying the Less than 5MHz operation in Rel-18 decided to maintain the side-conditions and adjust the number of samples. In a specification effort, this is not always the most efficient approach for measurement relaxation. In certain cases, the technical aspects of a given feature must strive for detectability levels, and in such cases this is the right approach.

However, in other scenarios, the technical aspects to be prioritized is the completion time of a certain operation. In such scenarios, it is first necessary to establish what is the maximum expected time within reasonable margins the UE can take to complete an operation and then find the side conditions that suits this requirement.

We believe that in this case, we are closer to the second type of scenarios. The main reason being that NTN has already significant measurement delays caused by the “split” of measurement opportunities across different satellites (i.e. with different Doppler conditions) and things will get more stringent when the “DL Coverage enhancement” feature is implemented in Rel-19. Currently, RAN1 has already agreed on SSB periods of 160 ms (with cell DTX) and is still discussing whether such values can be increased up to 320 ms.

By increasing the measurement delay in 4*160 (320) ms, the specification is adding 640ms (1.28s) in the measurement operations conducted by the UE at minimum. Numbers can get significantly worse when the UE is required to perform measurements across multiple satellites.

Even though we known that the DL coverage enhancement is not part of this RAN4-led WI, when both works are completed, by the end of Rel-19, their combined implementation will not be precluded, it will rather be one possible implementation scenario. If RAN4 can already envision an harmonic deployment between both scenarios, without any additional cost, a more efficient system will be available for all parts involved.

Maintain then values of samples related to Tidentify_xxx_with_index and adjust the side conditions accordingly.

Conclusion

The paper has analyzed the initial scope of the work for adopting the less than 5 MHz for the NTN UEs in FR1 and the following proposals were made.

Observation 1: Upon receiving a CHO command with secondary triggers, the UE can anticipate a HO is expected to happen.

There is some discussion whether the measurement preparations, common for the traditional CHO,  needs a larger time requirement time in order to follow increases in Tidentify_intra/inter_with_index caused by the punctured SSB. If that happens, the total time is increased by some SMTC periodicities (representing the UE will have multiple times to read the SSB block of the target cell).

Proposal 1: The value of T∆ in CHO requirement for NTN in exiting TS38.133 shall be extended to [2 Trs]

Proposal 2: Maintain then values of samples related to Tidentify_xxx_with_index and adjust the side conditions accordingly.

- References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 Chair (Huawei), RAN #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2413974, "WF for RRM of NTN with channel BW less than 5MHz", Moderator (Xiaomi), RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.

- R4-2413189, “Support of less than 5MHz for NR NTN”, Qualcomm, RAN4 #112, Maastricht, Netherlands, 19th – 23rd August, 2024

- R4-2413043, “Discussion on RRM requirements for NTN enhancement”, ZTE Corporation, Sanechips, RAN4 #112, Maastricht, Netherlands, 19th – 23rd August, 2024