3GPP TSG-RAN WG4 Meeting #113	R4-2419632

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	RRM requirements for Rel-19 DL Coverage Enhancements in NTN

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The most recent version of the WID can be found in [2]. It includes a handful of objectives such as:

- Downlink Coverage Enhancements

- Uplink Capacity/Coverage Enhancements

- Signaling of the intended service area of a broadcast service (MBS broadcast)

- Support of regenerative payload for NTN systems

- Support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems

The discussions of this WI in RAN4 were initiated in the previous meeting RAN4 #112. Discussions ensued in RAN4 #112bis and as the outcome of the discussion a way-forward (WF) document was approved which contains a set of agreements and other way-forward (for further discussion) guidance for currently identified open issues [3].

In this document we provide our views, focused on the first objective “Downlink Coverage Enhancements”.

Discussion

RAN1 has recently agreed that for the DL Coverage Enhancement features, the SSB periodicity will be extended at least up to 160 ms (while 320 ms is still under consideration).

This is a fundamental change compared to the previous assumption of 20 ms SSB periodicity (although 160 ms was already a possible configuration, there was the expectation that due to standalone deployments, the periodicity was limited to 20 ms for enabling efficient cell search). Moreover, RAN1 is considering “DTX” aspects on the DL frame, meaning that certain subframes will not be transmitted at all. This consideration is in-line with the main motivation of the present WI: enabling power sharing across multiple satellite footprints (NR Cells) in a power-limited scenario (i.e. considering the available instantaneous power available at the satellite is limited).

Certain aspects might be impacted by this modification. In this contribution we identify some of the impacted RRM requirements and propose a WF for them.

Impact on Measurements for Cell reselection of Intra-frequency NR cells

In TS 38.133, Clauses 4.2C.2.3 and 4.2C.2.4 define respectively measurements of intra and interfrequency cells for cell reselection in Idle Mode. And for the measurement evaluation of the serving cell, the following values are defined:

| DRX cycle length [s] | Tmeasure,NR_Intra [s] (number of DRX cycles) | Tmeasure,NR_Intra_enh [s] (number of DRX cycles) | Tmeasure,NR_Inter [s] (number of DRX cycles) | Tmeasure,NR_Inter_enh [s] (number of DRX cycles) |
| --- | --- | --- | --- | --- |
| 0.32 | 1.28 x N1 x M2 (4 x N1 x M2) | 0.32 x M3 (1 x M3) Note 1 | 1.28 x N1 x 1.5 (4 x N1 x 1.5) | [0.32 x M3 ([1] x M3)] Note 1 |
| 0.64 | 1.28 x N1 (2 x N1) | 0.64 (1) | 1.28 x N1 (2 x N1) | [0.64 (1)] |
| 1.28 | 1.28 x N1 (1 x N1) | 1.28 (1) | 1.28 x N1 (1 x N1) | 1.28 (1) |
| 2.56 | 2.56 x N1 (1 x N1) | 2.56 (1) | 2.56 x N1 (1 x N1) | 2.56 (1) |

Specification also states that: “The UE shall filter SS-RSRP or SS-RSRQ measurements of each measured higher, lower and equal priority inter-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Inter/2.”

Observe that for enhanced measurements, when the DRX Cycle is equal to 0.32 s, only two SSB periods (of 160 ms) can fit one DRX Cycle, and the UE shall perform measurements across both of these opportunities necessarily in order to achieve the requirements for Tmeasure,NR_xx_enh/2. This means that the UE has to successfully decode 100% of the SSB opportunities within a DRX Cycle, which is considerably more stringent than the previous requirement, where the UE would have at least 16 opportunities to collect at least 2 measurements.

Therefore, we propose:

For the SSB-periodicity of 160 ms, RAN4 to discuss whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s.

Paging Interruption

Paging interruption requirements state that:

#### 4.2C.2.5 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + K*Ttarget_cell_SMTC_period ms. Where, If the target cell belongs to the same satellite as the current one, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is GEO, and if the target cell is known, then K = 2. If the target cell belongs to a different satellite than the current one and the target cell’s satellite is non-GEO, then K = 5 if the target cell is known.

Observe that for non-GEO scenarios, if SSB periods of 160 ms are adopted, a total interruption time equal to 5*160=900ms + TSI-NR might be expected, which may exceed 1 second of interruption in a system with already very long natural interruption times due to cell DTX.

Therefore, we propose that:

Adjust the paging interruption requirements when SSB periods equal or larger 160 ms are used in NGSO scenarios.

Time interruption uncertainty for UL resources

In several mobility features, such as HO, the total interruption requirements are increased by TIU, where TIU  is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. In general, it is assumed that “TIU can be up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in the table 8.1-1 of TS 38.213 [3].”

However, the 10 ms in the equation above is added as an uncertainty of one radio frame for the UE to identify the proper SSB to PRACH occasion. However, due to DTX settings in the cell, the next radio frame might not be available in the current cell.

Consider extending TIU when the cell is in DTX mode due to the DL coverage enhancement feature.

Conclusion

In this paper we analyzed RAN4 impact of the NR_NTN_PH3 objectives considering the scope of this WI and/or the open issues already identified in the WF.

The following Observations and Proposals were made:

Proposal 1: For the SSB-periodicity of 160 ms, RAN4 to discuss whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s.

Proposal 2: Adjust the paging interruption requirements when SSB periods equal or larger 160 ms are used in NGSO scenarios.

Proposal 3: Consider extending TIU when the cell is in DTX mode due to the DL coverage enhancement feature.

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2416866, "The WF on RRM requirements for Rel-19 NR NTN phase3", Moderator (CATT), RAN4 #112bis, Hefei, Anhui, China, 14th – 18th October, 2024.

- R2-2407623, “LS on common TA in a regenerative payload scenario”, China Mobile, Maastricht, Netherlands, 19th – 23rd August 2024

- R1-2407538, “Draft Reply LS on DL coverage enhancements”, China Mobile, RAN1 #118, Maastricht, The Netherlands, 19th – 23rd August, 2024