3GPP TSG-RAN WG4 Meeting #113	R4- 2419638

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.21.3.2

Source: 	Huawei, HiSilicon

Title:	On the RF Requirements for Ambient IoT Devices

Document for:	Approval

# 1	Introduction

Following the Ambient IoT SID [1], the WF [2] from last meeting captures the progress on the study of impacts of A-IoT on RF requirements. In the sequel, we share our views on the open issues.

# 2	Discussion

### 2.1 Receiver

Agreement in RAN4#112bis

| A-IoT device- RX Requirement | Potentially needed or not |  |  |
| --- | --- | --- | --- |
|  | Device 1 | Device 2a | Device 2b |
| Reference sensitivity | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed | YES, as RAN4#112 agreed |
| Maximum input power | TBD | TBD | TBD |
| ACS | NO | NO | Depend on co-existence study |
| ACSC | NO | NO | NO |
| In-band blocking | NO | NO | TBD |
| Out-of-band blocking | NO | NO | TBD |
| Receiver intermodulation | NO | NO | TBD |
| Rx spurious emission | NO | NO | TBD |
| Spurious response | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed | NO, as RAN4#112 agreed |
| Interference rejection | TBD | TBD | TBD |

In a factory or a warehouse, the location of a device could be close to the reader installed on the ceiling. For a portable reader such as an intermediate UE, it could also get very close to a device. It’s important to ensure that the device can function properly at close distance, in other words, with large input power.

For UHF RFIDs, similar RF requirements are defined in ISO/IEC 18046-3 [3], such as: maximum operating power of tag, survival electromagnetic power of tag.

By definition, the blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels. For a NR receiver, it’s expected that the NR receiver has a BPF to attenuate the out-of-band blocker, and has multi-stage amplification, LPF and AGC to handle in-band blocker.

By contrast, an AIoT device has much simpler receiver. It might rely on the matching network to achieve coarse frequency selectivity, and the RF-ED would simply convert all received RF signals to the baseband. Apart from the simple LPF (and maybe RF BPF for device 2a), there’s not much the device can do to suppress the interference. As long as the blocking interference is temporary, it’s not a big issue for the AIoT system.

Moreover, it’s not expected that an AIoT system from another operator will be deployed on the adjacent channels. Hence, ACS requirement is not needed, either. However, it’s expected that the AIoT system will be deployed within an NR channel from the same operator. The AIoT device should be able to receive in the presence of a wideband NR interferer. Additionally, the device might also suffer from CW interference from a nearby reader/CW node, if it’s not synchronised with the serving reader.

By definition, the spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the antenna connector. For RF-ED based receivers, there’s no LO for receiving. An LNA might be present for device 2a/b, but the gain would be much smaller than that for an NR receiver. Hence, Rx spurious emission requirement is unnecessary.

In summary, we propose that:

Proposal 1: Considering the simple receiver architecture for the AIoT devices, we recommend:

| Maximum input power | potentially needed |
| --- | --- |
| ACS | not needed |
| In-band blocking |  |
| Out-of-band blocking |  |
| Receiver intermodulation |  |
| Rx spurious emission |  |

Proposal 2: Considering the potential deployment scenarios, we recommend to define a new requirement for

| Interference rejection | potentially needed. The interferer could be wide-band NR signal (e.g. in-band deployment) or CW signal (e.g. from unsynchronised reader/CW node). |
| --- | --- |

### 2.2 Transmitter

Agreement in RAN4#112bis:

| Transmit signal quality | Frequency error | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
| --- | --- | --- | --- | --- |
|  | Modulation quality requirements | TBD | TBD | TBD |
|  | In band emissions | NO, as RAN4#112 agreed | TBD | TBD. |
|  | Carrier leakage | NO, as RAN4#112 agreed | TBD | YES, as RAN4#112 agreed |
| Output RF spectrum emissions | Occupied bandwidth | YES | YES | YES |
|  | SEM | TBD | TBD | TBD |
|  | ACLR | TBD | TBD | TBD |
|  | Spurious emissions | TBD | TBD | TBD |
| Unwanted emissions |  | TBD | TBD | TBD |
| Transmit intermodulation |  | NO | NO | TBD |

- Frequency error for device 2a

For a device 2a that supports large frequency shift, some frequency error requirement might be needed.

- IBE for device 2a/2b

If UL FDM is not supported, IBE requirement will not be needed. Otherwise, whether IBE is needed depends on how the FDM-ed UL is processed by the BS receiver.

- Carrier leakage for device 2a

Since device 2a relies on backscattering for the UL, carrier leakage requirement is not needed. Even if frequency shift is applied, the BS receiver will filter out the residual carrier leakage.

- Modulation quality requirements

Some requirements on the RF envelope can be found in the RFID specs for Interrogator-to-Tag signal (similar to R2D), such modulation depth, pulse-width, rise/fall time, etc. However, there’re no such requirements for the Tag-to-Interrogator signal (i.e. D2R). Given that device 1 is going to have similar complexity/cost/power consumption as the RFID tag, it’s recommended not to define modulation quality requirements for the devices.

- SEM/ACLR/Spurious emission/Unwanted emissions

As the devices operate in licensed spectrum, SEM and spurious emission requirements are needed. Based on the coexistence study, the ACLR requirement is not needed for device 1/2a. The “unwanted emissions” can be covered by SEM and spurious emission requirements.

- Transmit intermodulation for device 2b

By definition, the transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

Such requirement can be considered for device 2b since its transmitter contains a power amplifier.

In summary, we propose that:

Proposal 3: For the Tx requirements, we recommend:

| Frequency error for device 2a | potentially needed for device 2a that supports large frequency shift |
| --- | --- |
| IBE for device 2a/2b | To be decided in WI phase |
| Carrier leakage for device 2a | not needed |
| Modulation quality | not needed |
| SEM | potentially needed |
| Spurious emissions | potentially needed |
| ACLR | not needed for device 1/2a, to be decided for device 2b in WI phase |
| Unwanted emissions | covered by SEM and spurious emission requirements |
| Transmit intermodulation for device 2b | potentially needed |

### 2.4 Testability

Agreement in RAN4#112bis:

- Take OTA test as the baseline for at least Devices 1 and 2a

  - FFS on the side conditions in the OTA test setup including CW signal and RF energy source, etc.

Different devices might utilise different sources of energy, such as RF signal, vibration, light, etc. It’s unlikely for 3GPP to define the energy harvesting requirements or the methodology to verify the performance.

The device vendors can declare the device’s energy harvesting capability, and the test house should ensure sufficient energy supply when verifying the device’s RF/RRM/Demod performances.

Proposal 4: For testability, the test house should ensure sufficient energy supply during tests based on vendor’s declaration of the device’s energy harvesting capability.

# 3	Conclusion

In this paper, we have shared further analysis on the potential RF requirements for A-IoT devices. The following observations and proposals are made:

Proposal 1: Considering the simple receiver architecture for the AIoT devices, we recommend:

| Maximum input power | potentially needed |
| --- | --- |
| ACS | not needed |
| In-band blocking |  |
| Out-of-band blocking |  |
| Receiver intermodulation |  |
| Rx spurious emission |  |

Proposal 2: Considering the potential deployment scenarios, we recommend to define a new requirement for

| Interference rejection | potentially needed. The interferer could be wide-band NR signal (e.g. in-band deployment) or CW signal (e.g. from unsynchronised reader/CW node). |
| --- | --- |

Proposal 3: For the Tx requirements, we recommend:

| Frequency error for device 2a | potentially needed for device 2a that supports large frequency shift |
| --- | --- |
| IBE for device 2a/2b | To be decided in WI phase |
| Carrier leakage for device 2a | not needed |
| Modulation quality | not needed |
| SEM | potentially needed |
| Spurious emissions | potentially needed |
| ACLR | not needed for device 1/2a, to be decided for device 2b in WI phase |
| Unwanted emissions | covered by SEM and spurious emission requirements |
| Transmit intermodulation for device 2b | potentially needed |

Proposal 4: For testability, the test house should ensure sufficient energy supply during tests based on vendor’s declaration of the device’s energy harvesting capability.

# References

- RP-240826, “Study on solutions for Ambient IoT (Internet of Things) in NR”, 3GPP TSG RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024

- R4-2417088 WF on impacts of A-IoT on RF requirements, Moderator (Huawei), RAN4#112bis

- ISO/IEC 18046-3 Information technology – Radio frequency identification device performance test methods – Part 3: Test methods for tag performance

- ETSI EN 302 208, Radio Frequency Identification Equipment operating in the band 865 MHz to 868 MHz with power levels up to 2 W and in the band 915 MHz to 921 MHz with power levels up to 4 W; Harmonised Standard for access to radio spectrum