# 3GPP TSG-RAN WG4 Meeting #113	R4-2419640

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.8.2.3.1

Source: 	Huawei, HiSilicon

Title:	Discussion on Tx requirements for IoT-NTN

Document for:	Approval

# 1	Introduction

In this paper, we discuss the open issues related to the Tx requirements for HPUE IoT NTN as captured in [1].

# 2	Discussion

Issue 1-1-2: How many Tx number should be considered for Iot-NTN (in the specification).

- Focus on 1Tx for PC2 [and PC1], further study [2Tx for PC1]

  - FFS on the applicability for handheld for PC1.

  - Continue work on 1Tx PC1 requirements.

Regarding the proposal of 2Tx PC1 architecture, it’s unclear what the power class of the component PA is. Certainly, two PC2 PA are not sufficient for PC1.

Proposal 1: Regarding the 2Tx PC1 candidate architecture, clarification on the applicable power class and ACLR for the component PAs are needed.

Issue 1-1-3: For NB-Iot based Iot-NTN, whether PC1 is feasible for handheld device.

- Proposals

  - Option 1: Yes, (vivo, Qualcomm)

    - Based on measurements for two different PAs. (vivo)

    - Handheld satellite phones which have very high output power exist.(Qualcomm)

  - Option 2: Others

- WF

  - Continue discussion in next meeting.

Some study results on using GSM PAs for PC1 were provided in [2]. Although the data show positive signs, the ACLR metric is still based on PC3. Furthermore, the OBW requirement is actually tighter than the GSM ACLR for NB-IoT. More explicitly, the effective ACLR is 23dB if 99% energy is inside the channel and 0.5% in either side of the adjacent channel.

Proposal 2: For NB-IoT based NTN, the feasibility of PC1 for handheld depends on the applicable ACLR as well as the effective ACLR from the OBW requirement.

Issue 1-1-4: For half duplex eMTC based Iot NTN, whether PC1 is feasible for handheld device.

- WF

  - FFS.

Issue 1-1-5: For full duplex eMTC based Iot NTN, whether PC1 is feasible for handheld device.

- WF

  - FFS.

Handheld devices are likely to integrate the IoT NTN transceivers together with other TN transceivers, except for dedicated satellite phones. The IL of the duplexer as well as the cost of PAs are the major limiting factors.

Proposal 3: For full duplex eMTC based NTN, stop working on PC1 for handheld devices.

Issue 1-2-2: ACLR

- Proposals

  - Proposal 1: Discuss whether to reuse IoT-NTN PC3 ACLR values for PC2, PC1.5, and PC1 ACLR because the NTN-TN coexistence evaluation [5] shows IoT-NTN ACLR values can be lower than 15dB for PC2 to PC1 HPUE. (MediaTek)

  - Proposal 2: Based on our co-ex study results, we suggest ACLR for NB IoT-NTN HPUE PC2 can be 24.5 dBc. And this can apply to both handheld and non-handheld devices. (Samsung)

- WF

  - Encourage more input next meeting.

As pointed out above, there’s no need to specify ACLR if it’s lower than 23dB, which is mandated by the OBW requirement.

Proposal 4: For IoT-NTN, the ACLR requirements for PC1 and PC2 should not be lower than that for PC3. If necessary, the PC3 ACLR requirement can be relaxed.

Issue 1-2-4: MPR

- Proposals

  - Proposal 1: If IoT-NTN PC3 ACLR value could be reused for IoT-NTN HPUE ACLR and relaxing SEM would be possible under HPUE operation, discuss whether reusing IoT-NTN PC3 MPR values for PC2 MPR would be one possible way

  - Proposal 2: Others

- Recommended WF

  - Encourage more input next meeting.

The PC3 MPR could be a starting point, but the PC2/PC1 MPR should be evaluated properly once the ACLR/SEM requirements are decided.

Proposal 5: Evaluate the HPUE MPR after the HPUE ACLR/SEM requirements are decided.

# 3	Conclusion

In this paper, we have discussed the open issues and made the following proposals:

Proposal 1: Regarding the 2Tx PC1 candidate architecture, clarification on the applicable power class and ACLR for the component PAs are needed.

Proposal 2: For NB-IoT based NTN, the feasibility of PC1 for handheld depends on the applicable ACLR as well as the effective ACLR from the OBW requirement.

Proposal 3: For full duplex eMTC based NTN, stop working on PC1 for handheld devices.

Proposal 4: For IoT-NTN, the ACLR requirements for PC1 and PC2 should not be lower than that for PC3. If necessary, the PC3 ACLR requirement can be relaxed.

Proposal 5: Evaluate the HPUE MPR after the HPUE ACLR/SEM requirements are decided.

# References

- R4-2417205 WF for HPUE for IoT NTN, RAN4#112bis

- R4-2415790Feasibility study and analysis for IoT NTN HPUE Tx Requirements, RAN4#112bis