3GPP TSG-RAN WG4 Meeting #113 	R4-2419705

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.17.1

Source: 	Ericsson

Title:	On general AI/ML aspects

Document for:	Discussion

# Introduction

The following was agreed [3] in RAN4#112-bis on general issues/post-deployment testing for AI/ML:

Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance. - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

# Discussion

## Post-deployment testing and verification

Two options for post-deployment testing were agreed in RAN4#112-bis [3].

- OPTION 1 (Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs) refers to testing of new/updated models or functionalities in a realistic way, considering the UE hardware that they will be deployed to, prior to use of the models in deployed UEs.

For this option, what is within the RAN4 scope is how to define RAN4 requirements related to this option, which could then be verified in test(s). Such requirements would then be related to the ability of the new model/functionality to:

  - allow the UE to maintain or enhance the UE performance, compared to the previous model/functionality and compared to the non-AI/ML operation mode,

  - correctly cooperate with other models/functionalities in the UE, e.g., to perform a timely fall back from the new model to the non-A/ML operation mode or (if supported) to another model/functionality.

These requirements should preferably be generic as much as possible, but making generic test configurations is more challenging, so the test configurations should reflect at least typical/critical scenarios. Since the UE with any (old or new) AI/ML model/functionality has to still meet the legacy requirements, the set of RAN4 requirements for Option 1 should include at least a subset of the relevant existing legacy requirements to be verified with the new AI/ML model/functionality ON. The subset depends on the use case and the type of the model/functionality, possibly with some margin (FFS if the margin is non-zero). Whether other KPIs are to be also verified is FFS and may depend on the use case.

According to option 1, the verification of a model update should be carried out before sending the model to UEs in the field. Questions arise on the means to validate the model (e.g. in a UE vendor lab, in a simulation containing the new model/model update and some appropriate input taking into account UE hardware etc.), the logistics and the level of “certification” of model updates (e.g. self-certification by the UE vendor or model provider).

  - Proposal 1: For Option 1, it is feasible to verify at least a subset of relevant existing legacy RAN4 requirements with the new AI/ML model/functionality ON.

    - The subset depends on the use case and the type of the model/functionality.

    - FFS if a non-zero margin should be added to the existing requirements target values. As a baseline assumption , the margin can be zero. A non-zero margin may be a capability, to enable the support of more advanced UEs.

    - FFS if new KPIs are to be also developed.

  - Proposal 2: The RAN4 requirements for Option 1 can be selected/defined to enable verification of the ability of the new model/functionality to:

    - allow the UE to maintain or enhance the UE performance, compared to the previous model/functionality and compared to the non-AI/ML operation mode,

    - correctly cooperate with other models/functionalities in the UE, e.g., to perform a timely fall back from the new model to the non-A/ML operation mode or (if supported) to another model/functionality.

RAN5 has commenced a discussion on AI/ML testing and can both formulate questions and provide useful input.

  - Proposal 3: For Option 1, RAN5 should discuss means of test and logistics. RAN4 should focus on the issues in proposals 1 and 2. RAN4 could request RAN5 to consider the following issues, by LS or other means:

    - What kinds of test of model update may be feasible prior to transfer to UEs in the field?

    - How and where the testing would be carried out?

    - How documentation or “certification” of models would work?

    - Relation to existing procedures for e.g. UE software updates.

- OPTION 2 (Using performance monitoring and LCM procedures)

This option suggests verifying the new model/functionality’s ability to maintain/enhance the UE performance and correctly cooperate with other models/functionalities in the UE, but during the performance monitoring and by LCM procedures. A challenge here is to how reliably separate the AI/ML model/functionality behaviour from the complexity of other factors that could impact UE performance in the real world (e.g., changing channel, traffic/scheduling, interference, RF behaviours, other AI functionality in the UE or network, etc.) and how to compare with the old model/functionality.

  - Proposal 4: For Option 2, one way to compare the old and new model/functionality during the performance monitoring is to:

    - monitor the output from both (from the old and from the new ones), based on the same real-time input, while still using the old model/functionality and only observing the new one and comparing it to the old one during the same observation period,

    - allow the using of the new model/functionality only in case of no issues during the observation period.

  - Proposal 5: For Option 2, observing the new model/functionality in parallel to using the old one can be resource demanding and the observation period must be sufficiently long, so new requirements may be needed to ensure the correct and reliable observation.

  - Proposal 6: For Option 2, another way may be that UE reports the measured KPI, uncertainty, confidence level, and other indirect performance metric upon the need (e.g. SGCS for CSI related models). For this approach, RAN4 should define requirements ensuring  the correct relation between UE reports and actual model performance/confidence, etc.

  - Proposal 7: For Option 2, the monitoring solution and requirements should ensure that the specific AI model performance can be identified specifically, without being mixed up with surrounding processes (e.g., changing channel, traffic/scheduling, interference, RF behaviours, other AI functionality in the UE or network, etc.).

## LCM RRM requirements for AI/ML

In RAN4#111, it was agreed that RAN4 will consider defining RRM requirements for selected LCM procedures, one of which is the fallback procedure to non-AI operation.

It is generally not expected that the performance will be worse with AI/ML than the legacy operation, since the UE will have to meet at least the legacy requirements even under AI/ML operation. However, a UE capable of AI/ML operation, may still need to go to non-AI/ML operation mode. Some of the critical reasons for a fallback to non-AI/ML operation can be, e.g.,

- potential performance degradation below the expected legacy performance,

- due to an error, from which it cannot reliably recover in AI/ML mode, etc.

Hence it is important to have a requirement on how efficient the UE can go back to the legacy non-AI/ML operation and what are the requirements for transitions between AI/ML and non-AI/ML modes.

- Proposal 8: RAN4 to study the performance expectations and potential requirements for the following UE-initiated transitions:

  - AI/ML mode → non-AI/ML mode,

  - Non-AI/ML mode → AI/ML mode.

- Proposal 9: RAN4 will study the desired performance gap between AI/ML and non-AI/ML operation modes and how it can be evaluated and achieved (which may require the same KPI(s) applicable for both AI/ML and non-AI/ML modes). This would be needed at least for UE-triggered fallback.

- Proposal 10: RAN4 will study a maximum delay requirement for switching between AI/ML and non-AI/ML operation modes, at least in the fallback direction (AI/ML →non-AI/ML).

- Proposal 11: The maximum fallback delay may include one or more components, e.g.,

  - obtaining and starting to use a fallback tx/rx configuration,

  - performing the necessary transmission and/or detection, identification, or measurement procedures,

  - informing the network about the fallback decision (e.g., if the UE decides this autonomously) or about completion of the fallback, etc.

# Summary

The following have been proposed in the current contribution:

  - Proposal 1: For Option 1, it is feasible to verify at least a subset of relevant existing legacy RAN4 requirements with the new AI/ML model/functionality ON.

    - The subset depends on the use case and the type of the model/functionality.

    - FFS if a non-zero margin should be added to the existing requirements target values. As a baseline assumption , the margin can be zero. A non-zero margin may be a capability, to enable the support of more advanced UEs.

    - FFS if new KPIs are to be also developed.

  - Proposal 2: The RAN4 requirements for Option 1 can be selected/defined to enable verification of the ability of the new model/functionality to:

    - allow the UE to maintain or enhance the UE performance, compared to the previous model/functionality and compared to the non-AI/ML operation mode,

    - correctly cooperate with other models/functionalities in the UE, e.g., to perform a timely fall back from the new model to the non-A/ML operation mode or (if supported) to another model/functionality.

  - Proposal 3: For Option 1, RAN5 should discuss means of test and logistics. RAN4 should focus on the issues in proposals 1 and 2. RAN4 could request RAN5 to consider the following issues, by LS or other means:

    - What kinds of test of model update may be feasible prior to transfer to UEs in the field?

    - How and where the testing would be carried out?

    - How documentation or “certification” of models would work?

    - Relation to existing procedures for e.g. UE software updates.

  - Proposal 4: For Option 2, one way to compare the old and new model/functionality during the performance monitoring is to:

    - monitor the output from both (from the old and from the new ones), based on the same real-time input, while still using the old model/functionality and only observing the new one and comparing it to the old one during the same observation period,

    - allow the using of the new model/functionality only in case of no issues during the observation period.

  - Proposal 5: For Option 2, observing the new model/functionality in parallel to using the old one can be resource demanding and the observation period must be sufficiently long, so new requirements may be needed to ensure the correct and reliable observation.

  - Proposal 6: For Option 2, another way may be that UE reports the measured KPI, uncertainty, confidence level, and other indirect performance metric upon the need (e.g. SGCS for CSI related models). For this approach, RAN4 should define requirements ensuring  the correct relation between UE reports and actual model performance/confidence, etc.

  - Proposal 7: For Option 2, the monitoring solution and requirements should ensure that the specific AI model performance can be identified specifically, without being mixed up with surrounding processes (e.g., changing channel, traffic/scheduling, interference, RF behaviours, other AI functionality in the UE or network, etc.).

- Proposal 8: RAN4 to study the performance expectations and potential requirements for the following UE-initiated transitions:

  - AI/ML mode → non-AI/ML mode,

  - Non-AI/ML mode → AI/ML mode.

- Proposal 9: RAN4 will study the desired performance gap between AI/ML and non-AI/ML operation modes and how it can be evaluated and achieved (which may require the same KPI(s) applicable for both AI/ML and non-AI/ML modes). This would be needed at least for UE-triggered fallback.

- Proposal 10: RAN4 will study a maximum delay requirement for switching between AI/ML and non-AI/ML operation modes, at least in the fallback direction (AI/ML →non-AI/ML).

- Proposal 11: The maximum fallback delay may include one or more components, e.g.,

  - obtaining and starting to use a fallback tx/rx configuration,

  - performing the necessary transmission and/or detection, identification, or measurement procedures,

  - informing the network about the fallback decision (e.g., if the UE decides this autonomously) or about completion of the fallback, etc.

# References

[1]	RAN4#112 Chairman’s notes, main session, Aug. 2024.

[2] R4-2414323, WF on AI/ML, Qualcomm Inc., Aug. 2024.

[3] R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Inc. Oct. 2024.