# 3GPP TSG-RAN WG4 Meeting # 113                                                                R4-2419712

# Orlando, USA, 18 – 22 November 2024

# Title:	1UL HPUE NR-CA MSD Observations

# Agenda Item:	7.1.1.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Discussion

# Introduction

This document presents observations resulting from the parsing of all one uplink (1UL) High Power UE (HPUE) MSD test points from TS 38.101-1 Rel-18.7.0. Based on these observations, we present, in a separate contribution [1], our views on simplifying the Rel-19 1UL HPUE MSD requirements.

# Discussion

## Background on previous 1UL HPUE MSD simplification proposals.

## The following summarizes the timeline of events that led to the initial 1UL HPUE MSD simplification proposals:

## In Rel-18, new 1UL power class 2 (PC2) configurations in the FDD bands were requested for downlink carrier aggregation (DL CA). Reaching agreement on how to calculate the DL band MSD due to 1UL PC2 1Tx / 2Tx MSD was complicated, due to varying assumptions.

- At meeting #110, several 1UL HPUE MSDs were proposed. However, due to an inconsistent approach, a WF [2] was agreed upon. This involves inviting companies to propose guidelines on how to simplify the PC2 2Tx MSD analyses.

- At meeting #110bis, we presented a “step-by-step” approach to evaluate the 1UL PC2 1Tx / 2Tx MSD [3]. An example of how to calculate the single-band PC2 reference sensitivity degradation (RSD) was provided. The proposals in [3] require re-evaluating the 1Tx PC3 interference levels that affect the Main and the Diversity (Div) antenna ports. The PC3 (1Tx) Main/Div interference imbalance is essential to calculating the PC2 2Tx MSD. This is shown in the Figure 1 example of Band n77 MSD due to Band n25 PC2 1Tx/2Tx operation in band.

Figure 1: Band n77 UL2/DL1 MSD due to Band n25 1UL PC2 1Tx and PC2 2Tx operation [3].

It was recognized that this approach may be cumbersome for some proponents. Thus, a subsequent WF [4] was agreed upon to invite companies to propose useful PC2 1Tx/2Tx MSD classes, wherein, for example, the PC2 1Tx MSD could be simplified as PC2 1Tx MSD = PC3 1Tx MSD + [3]dB.

- At the start of meeting #111:

- We proposed to simplify the 1UL HPUE MSD by adopting look-up tables (LUTs) that map the calculated PC2 1Tx/2Tx MSD from the agreed-upon PC3 MSD. This approach saves the HPUE MSD calculation, and delivers accurate and consistent HPUE MSD requirements [5]. The LUTs were proposed with the parameter “PC2 1Tx /2Tx”, where:

      - PC2 1Tx = PC2 1Tx MSD – PC3 MSD

      - PC2 2Tx = PC2 2Tx MSD – PC3 MSD

- LUTs were provided for 1UL HPUE MSD, due to UL harmonic and Rx harmonic mixing, with and without the effect of reverse-IMD, and:

- LUT for PC2 1Tx was proposed for the Main/Div 1Tx imbalance of 0 dB and 9 dB. Note that 1Tx Main/Div imbalance has almost no impact on the accuracy of PC2 1Tx MSD, using the proposed LUT.

- LUT for PC2 2Tx was presented for the Main/Div 1Tx imbalance of 0 dB, 6 dB, and 9 dB.

Figure 2 and in Table 1 show the proposals from [5], and reiterate that the value of the “PC2 2Tx saturation” increases with the 1Tx imbalance assumption:

  - For 0 dB imbalance, the PC2 2Tx plateau is at 3 dB.

  - For 6 dB imbalance, the PC2 2Tx plateau is at ~ 5.4 – 5.8 dB.

  - For 9 dB imbalance, the PC2 2Tx plateau is at ~ 8 – 8.4 dB.

Figure 2: Calculates PC2 1Tx (blue lines) versus specified PC2 1Tx (dots) for a 0 dB imbalance (plain blue line), versus a 9 dB imbalance (dotted blue line). PC2 2Tx for 0 dB, 6 dB, and 9 dB Main/Div imbalance, with Rev-IMD (dotted green lines), and without Rev-IMD (plain green lines) [5].

Table 1: Example of PC2 1Tx LUT (left) and PC2 2Tx LUT (right) with 0.5 dB accuracy, no reverse-IMD and 9 dB imbalance [5,18].

| Agreed PC3 MSD | PC2_1Tx LUT | Agreed PC3 MSD | PC2_2Tx LUT |
| --- | --- | --- | --- |
| PC3 MSD ≥ 10dB | 3.0dB | PC3 MSD ≥ 15dB | 8.0 |
| 5.0dB ≤ PC3 MSD < 10dB | 2.5dB | 8.5dB ≤ PC3 MSD < 15dB | 7.0 |
| 3.0dB ≤ PC3 MSD < 5.0dB | 2.0dB | 6.5dB ≤ PC3 MSD < 8.5dB | 6.0 |
| 1.8dB ≤ PC3 MSD < 3.0dB | 1.5dB | 4.5dB ≤ PC3 MSD < 6.5dB | 5.0 |
| 1.0dB ≤ PC3 MSD < 1.8dB | 1.0dB | 3.5dB ≤ PC3 MSD < 4.5dB | 4.0 |
| PC3 MSD < 1.0dB | Analysis case by case | 2.5dB ≤ PC3 MSD < 3.5dB | 3.0 |
|  |  | 1.0dB ≤ PC3 MSD < 3.5dB | 2.0 |
|  |  | PC3 MSD < 1.0dB | [FFS] |

Observation 1: PC2 2Tx increases with the 1Tx Main/Div imbalance assumption [5]:

- For a 0 dB imbalance, the PC2 2Tx plateau is at 3 dB

- For a 6 dB imbalance, the PC2 2Tx plateau is at ~ 5.4 - 5.8 dB

- For a 9 dB imbalance, the PC2 2Tx plateau is at ~ 8 - 8.4 dB

- The validity of the PC2 approach was proven by comparing the calculated PC2 MSD versus the tabulated PC2 MSD, using PC2 LUTs. The example for CA_n25-n77 [6] is summarized in Table 1.

Table 1: PC2 1Tx/2Tx calculated MSD versus LUT-based MSD for CA_n25-n77.

- In Rel 18.5.0 (meeting #111), CA_n3-n78 was the only specified 1UL MSD test point, due to Band n3 PC2 1Tx/2Tx. The agreed-upon MSD reproduced in Table 2 was the result of averaging amongst three companies, with a wide range of 1Tx interference imbalance, from 2.4 dB to 16 dB. The agreed-upon test points correspond to a PC2 1Tx of 3.2 dB and a PC2 2Tx of 8.4 dB.

Table 2: Agreed-upon Band n78 PC3, PC2 UL harmonic MSD requirements for CA_n3-n78 (Rel-18.5.0) and 1Tx Main/Div imbalance assumptions.

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | PC3 MSD | PC2 1Tx MSD | PC 2Tx MSD | PC21Tx | PC22Tx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) | (dB) | (dB) | (dB) |
| n3 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | 27.1 | 32.3 | 3.2 | 8.4 |
| n3 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | 16.6 | N/A | 2.8 | N/A |
| 1Tx interference Main/Div imbalance (dB) |  |  | Reference |
| 16 |  |  | [7] |
| 6.3 |  |  | [8] |
| 2.4 |  |  | [9] |

  - At the conclusion of meeting #111, several band combinations were completed using our LUT tables, as captured in TR 38.850 [10]. For PC2 2Tx, the LUT corresponding to a 9 dB imbalance was used for CA_n25-n77, and CA_n25-n71, and so forth. The adoption of a 9 dB imbalance was driven by:

      - An assumption used in most TPs for TRs, as as result of early Rel-15 studies.

      - Figure 2 shows that this assumption is the worst-case scenario. This was also the most acceptable compromise established during the CA_n3-n78 MSD discussions.

Observation 2: Several PC2 2Tx MSD test points that include 1UL HPUE harmonic interference have already been agreed upon and are specified in Rel-18. These TPs use the PC2 2Tx simplification approach, with a 9 dB 1Tx imbalance [10].

## TS 38.101-1 Rel 18.7.0: 1UL HPUE Harmonic MSD status

At meeting #112, several change requests (CRs) were agreed upon to correct the 1UL MSD test points due to UL harmonic and Rx harmonic mixing [11..17].  Figure 3 compares PC2 1Tx and PC2 2Tx calculated versus specified, for UL harmonic and Rx harmonic mixing.

Figure 3: Calculated PC2 1Tx (blue lines) versus specified PC2 1Tx (blue dots for UL harmonic and blue crosses for Rx hamonic mixing). Calculated PC2 2Tx for 9 dB Main/Div imbalance versus specified PC2 2Tx (green dots and green crosses) – Rel 18.7.0.

Figure 3 shows that the PC2 1Tx and PC2 2Tx LUTs of Table 1 [5,18] could be replaced, with reliable accuracy for all of these 1UL HPUE MSD test points.

Observation 3: The PC2 1Tx and PC2 2Tx LUTs of Table 1 [5,18] match, with reliable accuracy, the Rel 18.7.0 1UL HPUE MSD test points due to UL harmonic and Rx harmonic mixing interference.

## TS 38.101-1 Rel 18.7.0: Remaining 1UL HPUE MSD status

Figure 4 shows the statistics of the remaining 1UL HPUE MSD test points. This includes HPUE cross-band (Xband) isolation and PC1.5 test points for both Rx harmonic mixing and Xband isolation.

Figure 4: Remaining 1UL HPUE-specified MSD test points: cross-band isolation (diamonds), Rx harmonic mixing (crosses). Calculated PC1.5 with a 0 dB imbalance (red lines), PC1.5 with a 9 dB imbalance (black lines), PC2 1Tx (blue lines), and PC2 2Tx (green lines) – Rel 18.7.0.

The promising news is that, despite the lack of analysis for Xband MSD:

- Most Xband isolation test points (blue diamonds) are aligned with the calculated PC2 1Tx blue curves.

- The several agreed-upon PC2 2Tx Xband isolation test points (green diamonds) appear to align with PC2 1Tx with a 9 dB imbalance.

There are, however, a several “oddities” from the legacy Rel-17 PC1.5 MSD test points:

- Most harmonic mixing (red crosses) and Xband isolation test points (orange diamonds) are aligned with the PC1.5 calculated, assuming a 0 dB imbalance (red curves).

- The CA_n40-n41 Xband isolation test points appear to align with PC1.5 calculated, assuming a 9 dB imbalance (black curves). This also appears to be the case for CA_n78-n79.

- For PC2 1Tx, Rel-17 did not evaluate the impact of 2Tx architecture on inter-band CA MSD. Only one set of PC2 MSD requirements are specified, and these correspond to the 0 dB imbalance assumption.

## Case of CA_n78-n79 PC1.5 MSD

The HPUE MSD for CA_n78-n79 were specified with the following proposals [19]:

- “The MSD value of the PC2 case will be 3 dB higher than the PC3 case.”

- “The MSD value of the PC1.5 case will be 6 dB higher than the PC3 case.”

This means that the HPUE MSD was proposed using a fixed 3 dB PC2, and a fixed 6 dB PC1.5 value, independently of the agreed-upon PC3 level. The MSD analysis in Figure 5 is based on method [3], and demonstrates that these test points may remain aligned with the 9 dB imbalance set of curves, with the following minimal corrections:

- PC2 1Tx would need to be corrected to ~ 1.1 dB.

- PC1.5 would need to be corrected to ~ 4.3 dB.

Figure 5: Band n79 cross-band isolation MSD evaluation due to Band n78 1UL PC2 1Tx, PC2 2Tx, and PC1.5

## Case of CA_n40-n41 PC1.5 MSD

The HPUE MSD for CA_n40-n41 were agreed upon in [20] with a PC1.5 of 9.8 dB. This is the first band combination that appears to be aligned with a calculated PC1.5 for a 9 dB imbalance, since all of the other PC1.5 MSD test points are aligned with a 0 dB imbalance assumption. In Figure 6 below, we cross-check the PC2 and PC1.5 MSD for a 9 dB imbalance, according to method [3]. We also find that PC1.5 should be close to ~11 dB (without Rev-IMD). This confirms that the agreed-upon MSD for CA_n40-n41 can be justified.

Figure 6: Band n41 cross-band isolation MSD evaluation due to Band n40 1UL PC2 1Tx, PC2 2Tx, and PC1.5

Observation 4:

- Rel-18 inherited some Rel-17 PC1.5 MSD test points, which appear to assume a 0 dB interference imbalance for 1Tx Main/Div.

- In Rel-17, the impact of PC2 2Tx architecture on MSD was not evaluated. There is only one set of PC2 1UL MSD requirements, and these agreements match the PC2 1Tx calculations.

- In Rel-18, there are several agreed-upon 1UL PC1.5 MSDs that match the PC1.5, calculated with a 9 dB imbalance. This creates inconsistencies within the Rel-18 1UL PC1.5 MSD requirements.

## 1UL PC1.5 and PC2 2Tx Discrepancies and Way Forward.

Figure 7 concludes the Rel-18 MSD parsing results, and shows the following discrepancy:

- The PC2 2Tx plateau is ~ 8 dB.

- The PC1.5 plateau is ~ 6 dB. These test points are, with minimal exceptions, inherited from Rel-17.

This means that, potentially, the PC2 MSD may be higher than PC1.5. Even if this condition can be explained using different assumptions, these discrepancies must be addressed for Rel-19.

Figure 7: Discrepancy between Rel-18 PC2 2Tx and Rel-17 PC1.5 MSD requirements.

We see two options to solve this issue, as summarized in Figure 8:

- Option 1 (Fig.8-left): For Rel-19 new band combinations, we assume a 0 dB imbalance for PC2 2Tx. Thus:

- PC2 1Tx plateau at 3 dB

- PC2 2Tx plateau at 3 to 3.5 dB for 0.5 dB Rev-IMD

- PC1.5 plateau at ~ 6 to 6.5 dB for 0.5 dB Rev-IMD

- Compared to Rel-18, Option 1 would tighten the PC2 2Tx MSD, and the PC1.5 MSD for CA_n40-n41.

- Option 2 (Fig.8-right): For Rel-19 new band combinations, we assume a 9 dB imbalance for PC1.5:

- PC2 1Tx plateau at 3 dB

- PC2 2Tx plateau at 8 to 8.4 dB for 0.5 dB Rev-IMD

- PC1.5 plateau at ~ 10.8 to 11.3 dB for 0.5 dB Rev-IMD

- Compared to Rel-18, Option 2 would relax the PC1.5 MSD.

Figure 8: WF Option 1 (left), and Option 2 (right) to solve the Rel-18 PC1/5 and PC2 2Tx discrepancies

Observation 5:

- In Rel-18, for most PC1.5 MSD test points, PC1.5 is lower than the PC2 2Tx. This means  that PC2 2Tx MSD may be higher than the highest power class PC1.5.

- This may be explained by different assumptions used for MSD evaluations in Rel-17 for PC1.5, and in Rel-18 for PC2 2Tx.

- For Rel-19 new band combinations, we may consider two WF options to resolve this issue:

  - Option 1 (Fig.8-left): For Rel-19 new band combinations, calculate PC2 2Tx, assuming a 0 dB 1Tx Main/Div interference imbalance. Compared to Rel-18, Option 1 would tighten the PC2 2Tx MSD, and the PC1.5 MSD for CA_n40-n41.

- Option 2 (Fig.8-right): For Rel-19 new band combinations, calculate PC1.5, assuming a 9 dB 1Tx Main/Div interference imbalance. Compared to Rel-18, Option 2 would relax the PC1.5 MSD. Adopting the 9 dB Main/Div. interference imbalance may be justified because:

  - It represents a worst-case assumption, and hence it is more in line with the RAN4 “minimum requirements” objective, versus adopting the Option 1 “0 dB” imbalance assumption.

  - This assumption is used in most TP for TRs MSD analyses.

  - This assumption would not impact the agreed-upon Rel-18 test points. It would apply only to new Rel-19 combos.

- The Option 1 tightening of the specifications may be more problematic than the Option 2 relaxation.

- The Option 2 relaxation may not be an issue, since UEs may always report the lower MSD capability.

# Conclusion

This paper presents our analysis of parsing the agreed-upon 1UL HPUE MSD test points. From these results, we make the following observations. Based on these observations, we propose Rel-19 1UL HPUE MSD simplifications in [1].

Observation 1: PC2 2Tx increases with the 1Tx Main/Div imbalance assumption [5]:

- For a 0 dB imbalance, the PC2 2Tx plateau is at 3 dB.

- For a 6 dB imbalance, the PC2 2Tx plateau is at ~ 5.4 - 5.8 dB.

- For a 9 dB imbalance, the PC2 2Tx plateau is at ~ 8 - 8.4 dB.

Observation 2: Several PC2 2Tx MSD test points, that include 1UL HPUE harmonic interference, have already been agreed upon and specified in Rel-18, using the PC2 2Tx simplification approach, with a 9 dB 1Tx imbalance [10].

Observation 3: The PC2 1Tx and PC2 2Tx LUTs of Table 1 [5, 18] match, with reliable accuracy, the Rel 18.7.0 1UL HPUE MSD test points, due to UL harmonic and Rx harmonic mixing interference.

Observation 4:

- Rel-18 inherited several Rel-17 PC1.5 MSD test points, which appear to assume a 0 dB interference imbalance for 1Tx Main/Div.

- In Rel-17, the impact of PC2 2Tx architecture on MSD was not evaluated. There is only one set of PC2 1UL MSD requirements, and these agreements match the PC2 1Tx calculations.

- In Rel-18, there are several agreed-upon 1UL PC1.5 MSDs that match the PC1.5, calculated with a 9 dB imbalance. This creates inconsistencies within Rel-18 1UL the PC1.5 MSD requirements.

Observation 5:

- In Rel-18, for most PC1.5 MSD test points, PC1.5 is lower than the PC2 2Tx. This means that PC2 2Tx MSD may be greater than the highest power class PC1.5.

- This may be explained by different assumptions used for MSD evaluations in Rel-17 for PC1.5, and in Rel-18 for PC2 2Tx.

- For Rel-19 new band combinations, we may consider two WF options to resolve this issue:

  - Option 1 (Fig.8-left): For Rel-19 new band combinations, calculate PC2 2Tx, assuming a 0 dB 1Tx Main/Div interference imbalance. Compared to Rel-18, Option 1 would tighten the PC2 2Tx MSD, and the PC1.5 MSD for CA_n40-n41.

- Option 2 (Fig.8-right): For Rel-19 new band combinations, calculate PC1.5, assuming a 9 dB 1Tx Main/Div interference imbalance. Compared to Rel-18, Option 2 would relax the PC1.5 MSD. Adopting the 9 dB Main/Div. interference imbalance may be justified because:

  - It represents a worst-case assumption, and hence it is more in line with the RAN4 “minimum requirements” objective, versus adopting the Option 1 “0 dB” imbalance assumption.

  - This assumption is used in most TP for TRs MSD analyses.

  - This assumption would not impact the agreed-upon Rel-18 test points. It would apply only to new Rel-19 combos.

- The Option 1 tightening of the specifications may be more problematic than the Option 2 relaxation.

- The Option 2 relaxation may not be an issue, since UEs may always report the lower MSD capability.

# References

- R4-2419699 1UL HPUE MSD simplification, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions Inc.

- R4-2403628, WF on PC2 HPUE MSD, 3GPP TSG-RAN WG4 Meeting #110, Athens, Greece, Skyworks Solutions Inc, Huawei, Qualcomm Inc.

- R4-2405956, Guidelines for PC2 FDD Dual-TX MSD, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Skyworks Solutions Inc.

- R4-2406574, WF on PC2 FDD MSD Guidelines, 3GPP TSG-RAN WG4 Meeting #110bis, Changsha, China, Skyworks Solutions Inc, Qualcomm, Nokia.

- R4-2407156, Guidelines for FDD PC2 Harmonic MSD Analysis, 3GPP TSG-RAN WG4 Meeting #111, Fukuoaka, Japan, Skyworks Solutions, Inc.

- R4-2407160, PC2 CA_n25-n77 MSD, 3GPP TSG-RAN WG4 Meeting #111, Fukuoaka, Japan, Skyworks Solutions, Inc.

- R4-2215660, MSD analysis for CA_n3-n78 with PC2 n3 UL, 3GPP TSG-RAN WG4 Meeting #104bis-e, Electronic Meeting, Apple.

- R4-2215895, On CA band combination of DL CA_n3-n78 with UL PC2 n3, 3GPP TSG-RAN WG4 Meeting #104bis-e, Electronic Meeting, ZTE Corporation, China Unicom.

- R4- 2302731, MSD for CA_n3-n78 with PC2 n3 UL, 3GPP TSG-RAN WG4 Meeting #106, Athens, Greece, Huawei, HiSilicon.

- 3GPP TR 38.850 V18.1.0.

- R4-2414437, CR for EN-DC Harmonic Mixing clean-up PC3, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Qualcomm France, Skyworks Inc, Nokia.

- R4-2414438 CR for EN-DC Harmonic Mixing clean-up PC2, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Qualcomm France, Skyworks Inc, Nokia.

- R4-2414439 CR for NR CA Harmonic Mixing clean-up PC3 PC5, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Qualcomm France, Skyworks Inc, Nokia.

- R4-2414440 CR for NR CA Harmonic Mixing clean-up PC2 PC1.5, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Qualcomm France, Skyworks Inc, Nokia.

- R4-2414384 CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC3, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Skyworks Inc, Qualcomm France, Nokia.

- R4-2414385 CR to TS 38.101-1 Rel-18 NR CA Uplink Harmonic clean-up PC2, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Skyworks Inc, Qualcomm France, Nokia.

- R4-2414386 CR to TS 38.101-3 Rel-18 NR CA Uplink Harmonic clean-up, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Skyworks Inc, Qualcomm France, Nokia.

- R4-2413068 Update on Harmonic MSD simplification, 3GPP TSG-RAN WG4 Meeting #112, Maastricht, Netherlands, Skyworks Solutions. Inc.

- R4-2220471, TP for HPUE CA_n78-n79 with 1UL for TR 38.899, 3GPP TSG-RAN4 Meeting #105, Toulouse, France, NTT DOCOMO, Inc.

- R4-2405373, TP for TR 38.899 to introduce PC2 and PC1.5 CA_n40A-n41A, 3GPP TSG-RAN WG4 Meeting #110-bis, CMCC, Huawei, HiSilicon, ZTE Corporation, Murata Manufacturing Co Ltd.