3GPP TSG-RAN WG4 Meeting #113									                        R4-2419717

Orlando, US, November 18 – 22, 2024

Agenda item:			7.8.2.2.2

Source:	Ericsson

Title:	On UE RF Rx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issues on UE RF Rx requirements for NR-NTN HPUE from the approved WF in RAN4#112-bis [2].

# Discussion

In the approved WF [2], the following has been agreed:

Issue 3-1: RSD form in spec

Agreement:

- Use similar way in TN spec to derive RSD, if needed.

Issue 3-2: RSD requirements for PC2

Agreement:

- For n256: [0dB] RSD for PC2 NR-NTN for 1Tx and 2Tx

  - FFS on the duplexer isolation and PA interference

Band n256 has a slightly larger duplexing distance than band n1 so at least the same duplexer isolation should be achievable for the two bands. In TS38.101-1 [3], 0dB RSD is specified for n1 for PC2 for both UE supporting Tx Diversity or not. For band n254 the duplexing distance is very large so 0dB RSD should be achievable as well.

Proposal 1: For n256 and n254, specify 0dB RSD for PC2 NR-NTN UEs for both 1Tx and 2Tx.

NR TN band n24 has the same frequency range as band n255 in NR NTN, although DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz [3]. In TS38.101-1, no RSD requirement is specified for n24, so it should be checked whether 0dB RSD could be specified for n255 for PC2.

Observation 1: Since no RSD requirement is specified for band n24 which has the same frequency range as band 255 (although with limited UL operation), it should be checked whether 0dB RSD could be specified for n255 for PC2.

Issue 3-4: REFSENS/RSD testing

Agreement:

- For PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration with PC3 NR NTN UE as starting point.

In TS38.101-1 [3], the UL configuration for the REFSENs test does not depend on the applied Power Class. So, the same approach could be adopted for NTN.

Proposal 2: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE.

# Conclusion

Based on the discussion in the previous sections we have made the following observation:

Observation 1: Since no RSD requirement is specified for band n24 which has the same frequency range as band 255 (although with limited UL operation), it should be checked whether 0dB RSD could be specified for n255 for PC2.

Based on the discussion in the previous sections we have made the following proposals:

Proposal 1: For n256 and n254, specify 0dB RSD for PC2 NR-NTN UEs for both 1Tx and 2Tx.

Proposal 2: The uplink configuration for reference sensitivity normally doesn’t depend on the power class, so for PC2 NR NTN UE, the REFSENs test can adopt the same UL configuration as for PC3 NR NTN UE.

# References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#105.

- R4-2417122, “WF on co-existence study and HPUE for NR-NTN”, Samsung, RAN4#112-bis.

- 3GPP TS 38.101-1 V18.7.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”