# 3GPP TSG-RAN WG4 Meeting # 113                                                          R4-2419721

# Orlando, USA, 18 – 22 November 2024

# Title:	Impact of SUL HPUE on MSD

# Agenda Item:	6.8.5

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

# Introduction

This document re-opens the topic on the impact of high power user equipment (HPUE) support in the supplementary uplink (SUL) band on one uplink (1UL) HPUE MSD requirements. This may be interpreted as a follow-up to the ongoing discussions related to simplifying the 1UL HPUE MSD requirements [1 and 2] and discussions on valid HPUE UL configurations [3].

# Discussion

Table 1 below reproduces the Rel-18 supported power classes for SUL bands. Power Class 2 (PC2) is supported in Bands n80, n84, n97 and n98.

Table 1: Rel-18 UE Power Class for SUL.

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n80 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n98 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| NOTE 1: PPowerClass is the maximum UE power specified without compensating for the tolerance. NOTE 2: Power Class 3 (PC3) is the default power class, unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: Achieved via dual Tx. NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE Power Class 1 (PC1) requirements for Band n14 are applicable for public safety scenarios only. NOTE 8: PC1 in Band n100 and n101 is allowed only for FRMCS cab-radio’s, i.e. user equipments installed on trains with external antenna on top of the train roof at approximately 4m above ground. |  |  |  |  |  |  |  |  |

In clause 5, and contrary to the NR CA configuration tables (e.g. Tables 5.5A.3.1-1a ~ 5.5A.3.1-1n) where the valid 1UL HPUE configurations are captured with dedicated footnotes [3], the SUL configuration tables (e.g. Table 5.5C-1) do not specify what is the valid 1UL HPUE power class. This is illustrated in Figure 1, where we compare the supported configurations for SUL_n1A-n80A (top) with its NR CA counterpart CA_n1A-n3A (bottom).  We highlight (in yellow) that for CA_n1A-n3A, 1UL PC2 operation is a valid uplink configuration. For SUL_n1A-n80A, there is no clear indication if PC2 is also supported as the SUL counterpart to Band n80. Thus, we may assume that this is the case, by default.

| SUL configuration | NR Band |  | Channel bandwidth (MHz) (NOTE 1) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| SUL_n1A-n80A | n1 |  | 5, 10, 15, 20, 25, 30, 40, 50 | 0 |
|  | n80 |  | 5, 10, 15, 20, 25, 30 |  |
| NOTE 1: The SCS of each channel bandwidth for NR band refers to Table 5.3.5-1. |  |  |  |  |
| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| CA_n1A-n3A | n38 CA_n1A-n3A | n1 | 5, 10, 15, 20 | 0 |
|  |  | n3 | 5, 10, 15, 20, 25, 30 |  |
|  |  | n1 | 5, 10, 15, 20, 25, 30, 40, 50 | 1 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n1 | 5, 10, 15, 20 | 2 |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 35, 40 |  |
|  |  | n1 | n1 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n3 | n3 channel bandwidths in Table 5.3.5-1 |  |

Figure 1:  Rel-18 SUL supported channel bandwidth for SUL_n1A-n80A (top, from Table 5.5C-1) versus supported configurations for CA_n1A-n3A (bottom, from Table 5.5A.3.1-1a).

We recognize that the default power class for the SUL is PC3, and consequently 1UL PC3 MSD requirements are specified for SUL. However, if the SUL HPUE configurations were valid for SUL combinations, we may conclude that many 1UL HPUE MSD requirements are unavailable. Thus, we seek to clarify if the SUL band HPUE power classes shown in Table 1 are, de-facto, supported as valid 1UL HPUE configurations for any SUL band combination.

Observation 1: The HPUE is supported by several SUL bands (n80, n84, n97, n98). For SUL band combinations, it may be assumed that 1UL HPUE automatically becomes a valid SUL configuration, since the SUL configuration tables specified in clause 5 do not preclude such (S)UL configurations. Thus, many 1UL HPUE MSD requirements may be unavailable.

Proposal 1: Based on Observation 1, RAN4 clarifies if 1UL HPUE configurations are, de-facto, supported for SUL band combinations. For example, is it reasonable to conclude that PC2 1UL 2Tx is a valid SUL configuration for SUL_n1A-n80A.

If 1UL HPUE MSD requirements are, indeed, unavailable in Rel-18, then we must verify if the ongoing work on 1UL HPUE MSD simplifications should also apply to SUL. For background information, the HPUE MSD requirements may either be automatically specified based on lookup table (LUT) proposals [1], or they may no longer be required, based on Option 3, from agreed-upon WF [2].

Observation 2: The HPUE MSD may not require a significant amount of work if  the 1UL HPUE MSD requirement simplifications are agreed upon (for example [1]), or if  the HPUE MSD requirements are no longer specified (Option 3 [2]).

Proposal 2: Interested companies are invited to evaluate the impact of unavailable 1UL HPUE MSD requirements, and determine if HPUE MSD simplification rules apply to the SUL.

# Conclusion

This paper consolidates our views on valid 1UL HPUE configurations for SUL combinations.

Observation 1: The HPUE is supported by several SUL bands (n80, n84, n97, n98). For SUL band combinations, it may be assumed that 1UL HPUE automatically becomes a valid SUL configuration, since the SUL configuration tables specified in clause 5 do not preclude such (S)UL configurations. Thus, many 1UL HPUE MSD requirements may be unavailable.

Proposal 1: Based on Observation 1, RAN4 clarifies if 1UL HPUE configurations are, de-facto, supported for SUL band combinations. For example, is it reasonable to conclude that PC2 1UL 2Tx is a valid SUL configuration for SUL_n1A-n80A.

Observation 2: The HPUE MSD may not require a  significant amount of work if the 1UL HPUE MSD requirement simplifications are agreed upon (for example [1]), or if  the HPUE MSD requirements are no longer specified (Option 3 [2]).

Proposal 2: Interested companies are invited to evaluate the impact  of unavailable 1UL HPUE MSD requirements, and determine if HPUE MSD simplification rules apply to the SUL.

# References

- R4-2419699 1UL HPUE NRCA MSD Simplifications for Rel-19, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.

- R4-2310321 WF on handling new highest-lowest CBW for BCS4-5 MSD, 3GPP TSG-RAN WG4 Meeting #107, Incheon, South Korea, Skyworks Solutions, Inc.

- R4-2419714 Improvements to valid Uplink HPUE configurations, 3GPP TSG-RAN WG4 Meeting #113, Orlando, USA, Skyworks Solutions, Inc.