3GPP TSG-RAN WG4 Meeting #113                                           R4-2419727

# Orlando, USA, 18th – 22nd of November 2024

Title: 	Updates for the Discussion on the Choice of Numerology for NTN Ku-band

Agenda Item:	7.9.3 - System parameters

Release:	Rel-19

Source: 	THALES

Document for:	Discussion

# For information

As described in the approved Way Forward for [112][314] NR_NTN_Ku_Band_General (R4-2413519, Eutelsat Group), the following agreements have been made at the previous RAN4#112 meeting:

#### Issue 3-1: Frequency range definition

Following the scope of the WID, definition of a new frequency range e.g. FR3, is not in scope.

#### Issue 3-2: Numerology - Decision criteria

The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology

- UL timing synchronization

- Phase noise

- Efficient channel utilization

- Beam hopping granularity

- Link budget UL/DL – as a function of SCS and channel bandwidth

#### Issue 3-3: Numerology – timing aspects

The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

#### Issue 3-4: Numerology – phase noise

The impact of SCS on susceptibility to phase noise is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues.

#### Issue 3-5: Numerology – efficient channel utilization

The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied.

#### Issue 3-6: Numerology– beam hopping granularity

An analysis of the impact of different SCS on performance, although not critical, can influence the decision on FR1 vs FR2 numerology.

Some other views have been provided also in R4-2416491 (“Discussion on the Choice of Numerology for Ku-band”, THALES).

Also, some other decisions have been taken as part of the Way Forward for [112bis][313] NR_NTN_Ku_Band_General (R4-2416593, Eutelsat Group):

#### Sub-topic 3-4: Phase noise target SNR

Further analysis on the impact of phase noise on FR1/FR2 numerology selection should assume a maximum of 12 dB SNR and maximum 16QAM modulation. This is just for analysis and is not intended to limit support of higher MCS.

#### Sub-topic 3-5: Efficient use of spectrum for FR1 – native bandwidths

Assume 10 MHz and 25 MHz native bandwidths are included in the analysis of spectrum use efficiency.

#### Sub-topic 3-12: SCS channel raster

SCS-based channel raster is assumed.

#### Sub-topic 3-13: Flexible TX Rx spacing

Flexible TX Rx spacing is assumed.

#### Sub-topic 3-14: Sync raster

The existing sync raster design is reused for Ku band(s).

# Updated Proposals from R4-2416491

For the time being, the following NTN Ku-band has been agreed in WF R4-2413519: 10.7 - 12.75 GHz (DL); 14.0 - 14.5 GHz (UL).

Moreover, the following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology (WF R4-2413519):

- UL timing synchronization: Impact of SCS on timing is critical to reliable performance

- Phase noise: Impact of SCS on susceptibility to phase noise is critical to reliable performance.

- Efficient channel utilization;

- Beam hopping granularity;

- Link budget UL/DL – as a function of SCS and channel bandwidth.

Observation 1. Decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis.

Some other aspects have to be considered, including satellite complexity (e.g. 800-1000 cells/satellite) and several operational constraints:

Observation 2: Satellite access networks operating in above 10 GHz are most likely to operate simultaneously in several NTN bands (i.e. Ku and Ka-band).

The Ka-band implementation already presents an obvious amount of complexity, with 800-1000 cells/satellite (see e.g. RAN1 working assumptions and R1-2401988 3GPP TSG RAN WG1 Meeting #116bis, THALES):

Table 1: FR2-NTN Satellite configuration

| LEO600km Set1-1 FR2 (i.e., Ka-band) |  |
| --- | --- |
| Maximum Bandwidth per beam | 400 MHz |
| SCS | 120 kHz |
| Beam size | TBD in next meeting |
| Satellite EIRP density /beam (dBW/MHz) |  |
| Payload Total DL power level (dBW) |  |
| Aggregated EIRP (Total) (dBW) |  |
| Satellite Tx max Gain |  |
| EIRP per Satellite beam (dBW) |  |
| Total number of beam footprints | 800 (note 1) |
| Total number of simultaneously active beams | 12 |
| % simultaneously active beams | 1.5 % |
| Note 1: A typical deployment scenario in FR2 should consider 800 satellites beams per a single satellite coverage area with an absolute number of simultaneously active beams equal to 16 (due to limitation of RF) |  |

Moreover, the resource allocation for different FR1 configurations have been provided below:

Figure 1: Resource utilization for Set1-1 FR1 and Set1-3 FR1

Figure 2: Resource utilization for Set1-2 FR1

Figure 3: Resource utilization for Set1-1 FR2

It seems therefore clear that the resource utilization for FR2 configurations is much more efficient with respect to similar beam periodicity.

Proposal 1: The simulations should therefore take into account the efficient resource utilization, spectrum efficiency, and not only BLER results.

Observation 3: The support of both FR1 and FR2 numerologies on board of the satellites operating above 10 GHz will lead to extra additional complexity.

Moreover, is detrimental to fragment the market of VSAT-like terminals operating in above 10 GHz, with the risk of having different numerology especially when the trend/future is to develop terminals able to operate in both bands (i.e. Ku and Ka-band).

Observation 4: Is detrimental to fragment the market of VSAT-like terminals operating in above 10 GHz.

Observation 5: Most chipset vendors do not support 60 kHz SCS configuration.

Observation 6: Advantages and disadvantages related to different SCS configurations:

- SCS configuration 1: 120 kHz SCS

      - Ku-band: Introduce 120 kHz SCS for both synchronization and UL/DL data transmission

      - Ka-band: No action

- SCS configuration 2: 60 kHz SCS

      - Ku & Ka-band: Introduce 60 kHz SCS for both synchronization and UL/DL data transmission

- SCS configuration 3: 30 kHz SCS

      - Ku & Ka-band: Introduce 30 kHz SCS for both synchronization and UL/DL data transmission

Observation 7: Current implementations using FR1 are considering for FDD 15 kHz numerology only and for TDD 30 kHz numerology only. However, 15 kHz numerology does not seem well adapted for the Ku-band range (because of Doppler and Phase Noise).

Observation 8: FR2 numerology provides better beam-hopping granularity and potentially a better service periodicity with respect to targeted service types (i.e. the latency of 2 consecutive transmission in the same beam can be decreased).

Proposal 2: Two band definitions have to be considered for Ku-band by taking into account FR1 (L/S-band) and FR2 numerologies (Ka-band) respectively.

Observation 9: RAN4 should adopt at least 120 kHz SCS and 30 kHz SCS for Ku-band for both synchronization and UL/DL data transmission for Ku-band definition, but in separate bands (so that FR1-NTN and FR2-NTN numerologies shall not be mixed in a single band).

Some simulation results have been further provided with respect to following hypothesis:

Table 2: NR CP-OFDM waveform configuration

| Parameters | Configuration A | Configuration B |
| --- | --- | --- |
| NR Carrier Bandwidth | 50 MHz (Maximum transmission bandwidth for SCS of 15 kHz) | Maximum allowed transmission bandwidth of each SCS |
| Subcarrier Spacing | 15 kHz (FR1) 30 kHz (FR1) 120 kHz (FR2) | 15 kHz (FR1) 30 kHz (FR1) 120 kHz (FR2) |
| NRB | 270 for 15 kHz SCS 133 for 30 kHz SCS 32 for 120 kHz SCS | 270 for 15 kHz SCS 273 for 30 kHz SCS 264 for 120 kHz SCS |
| PRB Allocation | Variable allocation (Defined to match Code block size ~ Kcb/2 i.e. ~2000 inf. bits for LDPC BG 1 and ~4000 inf. bits for LDPC BG 2). | 270 for 15 kHz SCS 273 for 30 kHz SCS 264 for 120 kHz SCS |
| Symbol Allocation | 14 per slot (All the OFDM symbols within a slot are allocated to PDSCH) | 14 per slot (All the OFDM symbols within a slot are allocated to PDSCH) |
| Modulation | 16QAM, R = [0.32, 0.35, 0.41, 0.46, 0.52, and 0.58] | 16QAM, R = [0.32, 0.35, 0.41, 0.46, 0.52, and 0.58] |
| HARQ | Disabled (single transmission with RV=0, no retransmissions) | Disabled (single transmission with RV=0, no retransmissions) |
| DMRS | Minimal config for all MCS | Minimal config for all MCS |
| PTRS | Disabled or Enabled (Maximal config for all MCS) | Disabled or Enabled (Maximal config for all MCS) |

For the simulations, an LDPC Soft Decoder with Max iteration = 50 was used and the target BLER = 1e-3. Moreover, an aggregated phase noise mask was used as presented below:

Table 3: Aggregated (service+feederlink) phase noise mask

| Offset from Carrier Frequency | Mask [dBc/Hz] |
| --- | --- |
| 100 Hz | -25.0 |
| 1 kHz | -50.0 |
| 10 kHz | -73.0 |
| 100 kHz | -92.0 |
| 1 MHz | -102.0 |
| 10 MHz | -113.0 |
| 100 MHz | -116.0 |

Spectral efficiency per MCS expressed in bits per second per Hertz is evaluated by taking into account the considered waveform configuration as well as other additional considerations:

- Framing Efficiency:

  - Transport Block CRC loss and potential Code Block CRC loss in case of TB segmentation

  - DMRS and PTRS overhead: RS are essential to assist the receiver for synchronization and demodulation.

  - The cyclic prefix loss: CP is an essential feature of NR to enable user transmissions synchronization, to deal with multipath channel and enable one tap equalization.

- Spectral Occupancy:

  - Standardized guard bands have been defined in the standard to enable good co-existence conditions between two systems operating in adjacent channel carriers.

After intensive simulations, the results are hereby provided below:

Figure 4: Configuration A with PTRS enabled

Figure 5: Configuration B with PTRS enabled

Observation 10: For Ku-band, FR1 SCS 15 kHz feature up to 20% spectral efficiency degradation compared to FR1 SCS 30 kHz and FR2 SCS 120 kHz for higher SNR value (12dB as decided during RAN4#112-bis as targeted value, even if in practice it can be different).

Observation 11: At 12dB SNR, simulations indicate that for a BLER target of 10^-3, an FR2-NTN configuration with 120 kHz SCS for Ku-band might reach up to 1Gbps useful throughput with 400 MHz configuration bandwidth.

Observation 12: At 12dB SNR, simulations indicate that for a BLER target of 10^-3, an FR1-NTN configuration with 30 kHz for Ku-band might reach up to 0.25Gbps useful throughput with 100 MHz configuration bandwidth.

Observation 13: At 12dB SNR, simulations indicate that for a BLER target of 10^-3, an FR1-NTN configuration with 15 kHz for Ku-band might reach up to 0.1Gbps useful throughput with 50 MHz configuration bandwidth.

Proposal 3: The simulations should be realistic, considering transparent architecture (worst case) with combined service link and feederlink noise phase, (more realistic scenario) with residual Doppler of at least +/-0.1ppm, simulations with/without PTRS, taking into account spectrum efficiency evaluation with respect to different channel bandwidths.

Proposal 4: PTRS shall be used for Ku-band as mandatory functionality, independently of band definition or numerology.

Proposal 5: RAN4 to consider, as part of the Ku-band WI, the definition of two NTN bands:

- NTN band with FR2 SCS = 120 kHz configuration (specially adapted to wideband satellite transponder with more than 400 MHz)

- NTN band with FR1 SCS = 30 kHz configuration (specially adapted to narrowband satellite transponder with only 36 MHz)

Proposal 6: Try not to mix FR1-NTN and FR2-NTN numerologies on the same band definition, as per current Ku-band WID description.

Proposal 7: Since CA is not currently considered by FR1-NTN, neither CBW combinations > 20MHz, it would be useful to consider also FR2-NTN numerology for Ku-band, which offers better throughput.

Proposal 8: RAN4 to inform/discuss with RAN1 through an LS the choice of FR1/FR2 numerologies for Ku-band.

# Conclusion

Observation 1. Decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis.

Observation 2: Satellite access networks operating in above 10 GHz are most likely to operate simultaneously in several NTN bands (i.e. Ku and Ka-band).

Proposal 1: The simulations should therefore take into account the efficient resource utilization, spectrum efficiency, and not only BLER results.

Observation 3: The support of both FR1 and FR2 numerologies on board of the satellites operating above 10 GHz will lead to extra additional complexity.

Observation 4: Is detrimental to fragment the market of VSAT-like terminals operating in above 10 GHz.

Observation 5: Most chipset vendors do not support 60 kHz SCS configuration.

Observation 6: Advantages and disadvantages related to different SCS configurations:

- Configuration 1: 120 kHz SCS

      - Ku-band: Introduce 120 kHz SCS for both synchronization and UL/DL data transmission

      - Ka-band: No action

- Configuration 2: 60 kHz SCS

      - Ku & Ka-band: Introduce 60 kHz SCS for both synchronization and UL/DL data transmission

- Configuration 3: 30 kHz SCS

      - Ku & Ka-band: Introduce 30 kHz SCS for both synchronization and UL/DL data transmission

Observation 7: Current implementations using FR1 are considering for FDD 15 kHz numerology only and for TDD 30 kHz numerology only. However, 15 kHz numerology does not seem well adapted for the Ku-band range (because of Doppler and Phase Noise).

Observation 8: FR2 numerology provides better beam-hopping granularity and potentially a better service periodicity with respect to targeted service types (i.e. the latency of 2 consecutive transmission in the same beam can be decreased).

Proposal 2: Two band definitions have to be considered for Ku-band by taking into account FR1 (L/S-band) and FR2 numerologies (Ka-band) respectively.

Proposal 3: The simulations should be realistic, considering transparent architecture (worst case) with combined service link and feederlink noise phase, (more realistic scenario) with residual Doppler of at least +/-0.1ppm, simulations with/without PTRS, taking into account spectrum efficiency evaluation with respect to different channel bandwidths.

Observation 9: RAN4 should adopt at least 120 kHz SCS and 30 kHz SCS for Ku-band for both synchronization and UL/DL data transmission for Ku-band definition, but in separate bands (so that FR1-NTN and FR2-NTN numerologies shall not be mixed in a single band).

Observation 10: For Ku-band, FR1 SCS 15 kHz feature up to 20% spectral efficiency degradation compared to FR1 SCS 30 kHz and FR2 SCS 120 kHz for higher SNR value (12dB as decided during RAN4#112-bis as targeted value, even if in practice it can be different).

Observation 11: At 12dB SNR, simulations indicate that for a BLER target of 10^-3, an FR2-NTN configuration with 120 kHz SCS for Ku-band might reach up to 1Gbps useful throughput with 400 MHz configuration bandwidth.

Observation 12: At 12dB SNR, simulations indicate that for a BLER target of 10^-3, an FR1-NTN configuration with 30 kHz for Ku-band might reach up to 0.25Gbps useful throughput with 100 MHz configuration bandwidth.

Observation 13: At 12dB SNR, simulations indicate that for a BLER target of 10^-3, an FR1-NTN configuration with 15 kHz for Ku-band might reach up to 0.1Gbps useful throughput with 50 MHz configuration bandwidth.

Proposal 4: PTRS shall be used for Ku-band as mandatory functionality, independently of band definition or numerology.

Proposal 5: RAN4 to consider, as part of the Ku-band WI, the definition of two NTN bands:

- NTN band with FR2 SCS = 120 kHz configuration (specially adapted to wideband satellite transponder with more than 400 MHz)

- NTN band with FR1 SCS = 30 kHz configuration (specially adapted to narrowband satellite transponder with only 36 MHz)

Proposal 6: Try not to mix FR1-NTN and FR2-NTN numerologies on the same band definition, as per current Ku-band WID description.

Proposal 7: Since CA is not currently considered by FR1-NTN, neither CBW combinations > 20MHz, it would be useful to consider also FR2-NTN numerology for Ku-band, which offers better throughput.

Proposal 8: RAN4 to inform/discuss with RAN1 through an LS the choice of FR1/FR2 numerologies for Ku-band.

# Annex

Moreover, the following information with respect to current specifications should be taken into account (with the current definition FR1-NTN: 410 MHz – 7125 MHz, FR2-NTN: 17300 MHz – 30000 MHz).

Table 5.3.5-1: SAN channel bandwidths and SCS per operating band in FR1-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 5 | 10 | 15 | 20 | 30 (NOTE) |
| n256 | 15 | 5 | 10 | 15 | 20 |  |
|  | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |
| n255 | 15 | 5 | 10 | 15 | 20 |  |
|  | 30 |  | 10 | 15 | 20 |  |
|  | 60 |  | 10 | 15 | 20 |  |
| n254 | 15 | 5 | 10 | 15 |  |  |
|  | 30 |  | 10 | 15 |  |  |
|  | 60 |  | 10 | 15 |  |  |
| NOTE: Deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded by introduction of all applicable Tx RF, Rx RF, and demodulation requirements. |  |  |  |  |  |  |

Table 5.3.5-2: SAN channel bandwidths and SCS per operating band in FR2-NTN

| SAN Operating Band | SCS (kHz) | SAN channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 50 | 100 | 200 | 400 |
| n512 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n511 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |
| n510 | 60 | 50 | 100 | 200 |  |
|  | 120 | 50 | 100 | 200 | 400 |

Table 5.4.3.3-1: Applicable SS raster entries per operating band (FR1-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30 kHz | Case C | 6218 – <1> – 6241 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Table 5.4.3.3-2: Applicable SS raster entries per operating band (FR2-NTN)

| SAN operating band | SS Block SCS | SS Block pattern (NOTE) | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n512 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n511 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| n510 | 120 kHz | Case D | 17448 – <12> – 19428 |
|  | 240 kHz | Case E | 17472 – <24> – 19416 |
| NOTE: SS Block pattern is defined in section 4.1 in TS 38.213 [7]. |  |  |  |

And the NTN timing error requirements represented for both FR1-NTN and FR2-NTN:

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN | Applicability |
| --- | --- | --- | --- | --- |
| FR1-NTN | 15 | 15 | 29*64*Tc | Timing error limit applicable for FR1-like NTN configurations. |
|  |  | 30 | 24*64*Tc |  |
|  |  | 60 | N/A |  |
|  | 30 | 15 | 24*64*Tc |  |
|  |  | 30 | 22*64*Tc |  |
|  |  | 60 | N/A |  |
| FR2-NTN | 120 | 60 | 13*64*Tc | Timing error limit applicable for FR2-like NTN configurations with fixed VSAT served by GSO, and fixed VSAT served by NGSO. |
|  |  | 120 | 7.5*64*Tc |  |
|  | 240 | 60 | 13*64*Tc |  |
|  |  | 120 | 7.5*64*Tc |  |
| FR2-NTN | 120 | 60 | 13*64*Tc | Timing error limit applicable for FR2-like NTN configurations with mobile VSAT served by GSO. |
|  |  | 120 | 7.5*64*Tc |  |
|  | 240 | 60 | 13*64*Tc |  |
|  |  | 120 | 7.5*64*Tc |  |
| NOTE: Tc is the basic timing unit defined in TS 38.211 |  |  |  |  |