3GPP TSG-RAN WG4 Meeting #113	R4-2419743

Orlando, US, November 18 – 22, 2024

Agenda Item:	7.15.2.2

Source:	MediaTek Inc.

Title:	Discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF

Document for:	Discussion

# Introduction

The objective for R19 NR RRM Phase-5 work item was updated in the WID as captured below [1]. In this paper, we continue the discussion on the first objective, particularly on optimizing CSSF outside gap in CA/DC scenarios for L3 measurement delay reduction.

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

In this paper we discuss the following issues captured in the WF [2].

### Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Agreement in last meeting: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”.

For Issue 2-2, optimizing CSSF outside gap for L3 measurement delay reduction may impact different measurements including Intra-frequency/Inter-frequency/Inter-RAT measurement without MG, however it is preferred RAN4 to prioritize the aspect which is commonly deployed in real field. It is known that SSB based Intra-frequency measurement without MG is the most common scenario for measurement without MG which was defined since R15. Therefore, it proposed that RAN4 to prioritize Intra-frequency measurement without MG in this WI.

Proposal 1: the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized:

- Aspect 1: SSB based Intra-frequency measurement without MG, including:

  - TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15

Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF Agreement in online session: On top of the previous agreement on the baseline assumption of 2 searchers, for 3 searchers: - Discuss the applicable scenario(s) and/or solution(s) based on 3 searchers. Decide whether or not to define the requirement later.

For Issue 2-4, in legacy RRM requirements, RAN4 assumed 2 searchers for CSSF calculations (1 for PCC and 1 shared among SCCs) considering different operation modes and whether it is for FR1 or FR2 CCs measurements, which at the end resulted in very complicated table to define the requirements. The complexity of CSSF calculation may become even more complex when we introduce 3 searchers. Therefore, it is preferred to prioritize optimizing CSSF based on the current assumption of 2 searchers rather than 3 searchers.

Proposal 2: RAN4 to prioritize optimizing CSSF outside gap based on 2 searchers as a baseline assumption, and whether to optimize CSSF based on 3 searchers can be postpended after concluding 2 searchers.

Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC Candidate options for further discussion: - Option 1: UE can measure one serving CC per band if multiple serving CCs are in the same band. For the CCs to be measurement, further discuss option 1A and option 1B - Option 1A: Reuse the legacy principle for neighboring cell measurement. - Option 1B: Based on network indication - The other CCs (not measured based on network indication) need not to be measured and reported. Network assume the same measurement results on the measured CCs and unmeasured CCs in the same band. - Option 2: UE can change the searcher occupancy ratio of measurement among the measured CCs - Option 2A: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions For 3 searchers: In addition to option 1 and 2 - Option 3: consider to use 3 searchers for CSSF reduction. - Depending on issue 2-4.

For Issue 2-5, we support Option 1 as the condition “UE can measure one serving CC per band if multiple serving CCs are in the same band” will optimize the CSSF outside gap by using 1 searcher for the CC to be measured in that band, while the other searcher can be utilized for other CCs not on the same band. This optimization will simplify the requirements based on 2 searchers. Option 2 on the other hand, is also another approach for optimizing CSSF, however, it may result in more complicated requirements. Option 3 is not preferred at this stage of the WI.

Proposal 3: Specify the following solution to apply L3 measurement delay reduction by optimizing CSSF outside gap:

- UE can measure one serving CC per band if multiple serving CCs are in the same band.

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized:

- Aspect 1: SSB based Intra-frequency measurement without MG, including:

  - TPSS/SSS_sync_intra and TSSB_measurement_period_intra

  - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15

Proposal 2: RAN4 to prioritize optimizing CSSF outside gap based on 2 searchers as a baseline assumption, and whether to optimize CSSF based on 3 searchers can be postpended after concluding 2 searchers.

Proposal 3: Specify the following solution to apply L3 measurement delay reduction by optimizing CSSF outside gap:

- UE can measure one serving CC per band if multiple serving CCs are in the same band.

# References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, Apple, CATT

- R4-2416853, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.