3GPP TSG-RAN WG4 Meeting # 113	R4-2419746

Orlando, US, Nov 18 – Nov 22, 2024

Agenda Item: 7.19.3.2

Source: MediaTek Inc.

Title: Discussion on R19 MIMO for other RRM requirements

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR_MIMO_Ph5” [1]. According to WF [2], RAN4 had some agreements in the last meeting while some issues were discussed without conclusion yet. In this meeting, this WID [1] is divided into two items to be discussed: (1) UE-initiated/event-driven beam management. (2) Other RRM requirements.

The discussion in this paper focus on the “Other RRM requirements”.

# Discussion

In the following sections, we provided our view and discussed sequentially.

- CSI support for up to 128 CSI-RS ports.

- UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul.

- Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios.

## 2.1 CSI support for up to 128 CSI-RS ports

Issue 1-1: Whether RRM impacts exist by the added SRS port grouping for CSI enhancement? In last RAN plenary meeting RAN#105, another sub-bullet was approved for CSI enhancement (Objective#2): - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI. Way forward: - Option 1: Previous RAN4 agreement still apply “No RRM impact by CSI enhancement to support up to 128 ports”. - Option 2: Wait for RAN1.

In our understanding, there’s no RRM impact on the revised part of objective for the CSI enhancement. This is mainly focus on SRS port grouping for 6/8 RX low complexity receiver. Therefore, the following proposal is suggested:

Proposal 1: No RRM impact for CSI support for up to 128 CSI-RS ports.

## 2.2 UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul

Issue 2-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration? Agreement: - RAN4 to simulate the performance of offsets for phase, time delay and frequency, based on one-shot measurement and multiple-sample measurement, and discuss the impact on RRM core requirement later. - Further discuss the simulation assumptions offline during this meeting. Way forward: - Companies are encouraged to bring simulation assumption/results in the next meeting. Simulation assumption in Appendix A can be used as a starting point for information only. Issue 2-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration? Way forward: - Option 1: Define accuracy requirements. - Option 2: FFS. Issue 2-3: Whether report mapping tables should be specified by UE reporting enhancement for CJT calibration? Way forward: - Option 1: Define mapping tables for frequency/delay/phase offset reporting in performance part.

RAN1 has already specified some mechanism of UE reporting enhancement for CJT deployments including timing misalignment, frequency offset and phase offset reporting. In our understanding, these are new feature for reporting in terms of the compensation of delay, frequency, and phase offset. Based on RAN1 agreement for CJT calibration reporting, RAN4 to discuss whether there’s any RRM impact based on feasibility study. Whether to define RRM requirements for CJT calibration report depend on whether the measurement is based on single or multiple shots of TRS. However, in the last meeting RAN4 agreed to simulate the performance of the measured offsets (T/F/P), based on one-shot measurement and multiple-sample measurement to determine whether a single or multiple shots are required. Based on the conclusion of the simulation results RAN4 can then decide whether to define the requirements for CJT calibration report. In this meeting RAN4 needs at least to agree on the simulation assumption before collecting the results. Therefore, we provide our simulation assumption as shown below in Table 1.

Proposal 2: RAN4 to consider the simulation assumption for timing and frequency offset estimation on table 1.

Table 1. Simulation assumption for timing and frequency offset estimation

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDL-C |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 24RB (10 MHz) |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm. |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples for measurement | 1, 2, 3, 4, 5, 10 |
| CSI-RS periodicity | 10 slots |
| CSI-RS offset | slot 1, 2 |

## 2.3 Enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung The detailed objectives are as follows: RAN1: 5. Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Issue 4-2: RRM core impacts of Active uplink TCI state switching delay for unified TCI? Way forward: - Option 1: No RRM impacts on UL TCI state switching delay requirement for unified TCI states. - Option 2: RAN4 to specify UL TCI state switching requirements. - Option 3: FFS. [3] R1-2405346 Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP Moderator (OPPO) Agreement For indicating a PL offset for PDCCH-order PRACH transmission at least for FR1, further study and down-select one from the Alt1 and Alt3 by RAN1#118 meeting: - Alt1: RRC configures multiple PL offset values [in PRACH-Config] and PDCCH-order DCI indicates one of them through one DCI field - Alt3: The PL offset associated with one of the indicated joint/UL TCI state for UL TRP in unified TCI framework is applied on the PDCCH-order PRACH transmission - FFS: the details of DCI field design. Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

Based on RAN1 agreement, NW should indicate PL offset for UE to calibrate UL only TRP. In our understanding, the DL reference timing definition for PDCCH-order PRACH will be discussed in RAN1. We can wait for RAN1 progress on DL reference timing. The unified TCI state indication for the asymmetric DL sTRP/UL mTRP can totally reuse Rel-17 and Rel-18 unified TCI framework. Therefore, the following proposal is suggested.

Proposal 3: No RRM impact at least for unified TCI state switching since RAN1 agreed to reuse Rel-17/Rel-18 unified TCI framework to indicate TCI states of asymmetric DL sTRP/UL mTRP deployment scenarios.

It is noted that in the last meeting companies understand that UL TCI state can be associated to SRS. However, in the legacy MIMO requirements, there was no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS. Therefore, similar principle can be reused here in R19 MIMO.

Proposal 4: If UL TCI states can be associated to SRS, then reuse the same principle in Rel-17 which is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

# Summary

In this paper, the discussion of R19 MIMO is provided. We have the following proposals:

Proposal 1: No RRM impact for CSI support for up to 128 CSI-RS ports.

Proposal 2: RAN4 to consider the simulation assumption for timing and frequency offset estimation on table 1.

Table 1. Simulation assumption for timing and frequency offset estimation

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | TDD, OFDM |
| Carrier Frequency | 3.5 GHz |
| Channel Model | TDL-C |
| Delay Spread | 300ns |
| antenna configuration | 1X2 |
| TRP number | 2 |
| TRS Bandwidth | 24RB (10 MHz) |
| Numerology | 14 OFDM symbol per slot, 30kHz SCS |
| UE speed | 3km/h |
| SNR | -3dB, 0dB, 3dB |
| Frequency offset between TRP (only for freq. offset simulations) | 0.1ppm. |
| Delay difference between TRP (only for delay offset simulations) | 0.5CP, CP |
| Number of samples for measurement | 1, 2, 3, 4, 5, 10 |
| CSI-RS periodicity | 10 slots |
| CSI-RS offset | Slot 1, 2 |

Proposal 3: No RRM impact at least for unified TCI state switching since RAN1 agreed to reuse Rel-17/Rel-18 unified TCI framework to indicate TCI states of asymmetric DL sTRP/UL mTRP deployment scenarios.

Proposal 4: If UL TCI states can be associated to SRS, then reuse the same principle in Rel-17 which is no requirement when the UE is requested to switch to a TCI state with the higher layer parameter TCI-UL-State associated to SRS.

# Reference

[1] RP-242394, Revised Work Item: NR MIMO Phase 5, Samsung

[2] R4-2416857, WF on RRM requirements for NR_MIMO_Ph5 (Part2)

[3] R1-2405346 Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)